# XiangShan Frontend Architecture Documentation

Branch: KunMingHu v3  |  Last updated: 2026-02-27
Sources: Frontend.scala, Bpu.scala, NewFtq.scala, IFU.scala, IBuffer.scala, and sub-module files under bpu/

# 1. Frontend Block Diagram Overview

## 1.1 Block Summary

XiangShan Frontend is a speculative instruction-supply pipeline: BPU (Branch Predictor) → FTQ (Fetch Target Queue) → IFU (Instruction Fetch Unit) → IBuffer (Instruction Buffer) → Backend (Decode). It continuously supplies decoded instructions to the backend while tolerating ICache misses, branch mispredictions, and TLB faults.

Main inputs: Backend redirect (misprediction flush), sfence, tlbCsr, csrCtrl

Main outputs: io.backend.cfVec (DecodeWidth CtrlFlow entries, IBuffer→Decode), io.backend.stallReason (stall reason for TopDown analysis)

Performance bottlenecks: ICache miss (IFU stall), FTQ full (BPU back-pressure), IBuffer full (fetch stall), misprediction redirect flush

## 1.2 Key Parameters

| Parameter | Source | Default | Effect |
| --- | --- | --- | --- |
| FtqSize | FtqParameters.FtqSize | 64 | FTQ entry count (BPU-IFU buffer depth) |
| IBuffer.Size | IBufferParameters.Size | 48 | Total IBuffer entries |
| NumWriteBank | IBufferParameters.NumWriteBank | 4 | IBuffer write banks |
| NumReadBank | IBufferParameters.NumReadBank | 8 | IBuffer read banks (≥ DecodeWidth) |
| FetchBlockInstNum | FetchBlockSize(64B)/instBytes | 32 (HasCExtension=true), else 16 | Max instructions per fetch block |
| DecodeWidth | XSCoreParamsKey.DecodeWidth | 6 | IBuffer→Decode output width |
| PhrHistoryLength | FrontendParameters.getPhrHistoryLength | computed | Path History Register length |
| GhrHistoryLength | HasBpuParameters.GhrHistoryLength | SC max table | Global History Register for SC |
| ResolveEntryBranchNumber | FrontendParameters | 8 | Max branch slots per FTQ resolve entry |

## 1.3 Top-Level Block Diagram

XiangShan Frontend Pipeline — Block Diagram

============================================

CSR/sfence/tlbCsr/csrCtrl

|

v

+---------------------------------+

|       Predictor (BPU)           |

|  S0: PC mux, PHR/GHR gen        |

|    |                            |

|  S1: UBTB+ABTB+UTAGE+MicroRAS  |  <-- 1st prediction (1 cycle)

|    |  s1_fire --> prediction.v  |

|  S2: MBTB+TAGE+SC              |  <-- (internal only, no FTQ send)

|    |                            |

|  S3: MBTB(latched)+ITTAGE+RAS  |  <-- s3_override? --> FTQ update

+--------+-----------------------+

|  bpu_to_ftq (Decoupled)         ^ toBpu.redirect/update (Valid)

v                                 |

+---------------------------------+        |

|             Ftq                 |--------+

|  bpuPtr  <-- BPU enqueue        |

|  ifuPtr  --> IFU dequeue        |------> IFU (toIfu.req)

|  commPtr <-- ROB commit         |------> ICache (toICache.req)

|                                 |------> Prefetcher (toPrefetch)

+--------+------------------------+

|  toIfu.req (Decoupled)   flushFromBpu (BpuFlushInfo)

|

v

+---------------------------------+

|          NewIFU                 |       +---------------------+

|  F0: fetch req to ICache        |       |       ICache        |

|  F1: PC calc (CatPC adder)      |<------| MainPipe (S0/S1/S2) |

|  F2: ICache resp, Predecode     |       | MissHandler         |

|  F3: RVC expand, PredChecker    |       | Prefetcher          |

+--------+------------------------+       +---------+-----------+

|  toIbuffer (Decoupled)                   |

|  pdWb (Valid) --> FTQ                    v

v                               +------------------+

+---------------------------------+      |   iTLB / PMP     |

|           IBuffer               |      |  TLB PortNum+1   |

|  ReadBanks=8, WriteBanks=4      |      |  PMP+Checker     |

|  48 entries, interleaved        |      +------------------+

|  Output Reg (1 stage)           |

+--------+------------------------+

|  cfVec (DecoupledIO Vec, DecodeWidth=6)

v

+---------------------------------+

|     Backend / Decode            |<-- rob_commits

|                                 |--> toFtq.redirect (mispred)

+---------------------------------+

Key Parameters:

FtqSize=64, IBuffer.Size=48, IBuffer(read=8, write=4), FetchBlockInstNum=32

DecodeWidth=6, PhrHistoryLength=(computed), GhrHistoryLength=(SC max)

FetchBlockSize=64B, FetchBlockAlignSize=32B

## 1.4 BPU Internal S3 Override

When S3's result differs from S1's result for the same fetch PC, s3_override=true is asserted (Bpu.scala: s3_valid && !(s3_prediction === s3_s1Prediction)). The FTQ receives the corrected S3 prediction and updates the existing entry (io.toFtq.prediction.bits.s3Override := s3_override). There is no S2-direct override; when s3_flush fires, S2/S1 are also flushed. FTQ relays flushFromBpu to IFU, which consumes it in F0.

## 1.5 Exception / Error Paths

| Path | Detection | Handling |
| --- | --- | --- |
| iTLB Page Fault (PF) | ICache → iTLB → ExceptionType.pf | Propagated via fromICache.bits.exception → IFU F2 → IBuffer → Decode |
| iTLB Guest PF (GPF) | iTLB response | Same path as PF |
| PMP Access Fault (AF) | ExceptionType.fromPMPResp | Merged into ICache response → same path |
| ECC / TileLink corrupt (AF) | ExceptionType.fromECC / fromTilelink | icache.io.error → errorReg → io.error → L1BusErrorUnit |
| MMIO instruction | IFU F3: f3_pmp_mmio | InstrUncache path (64-bit per fetch); mmio_redirect → F2/F3 flush |
| Cross-page RVI | F2: last-in-line 32-bit spanning page boundary | CrossPF/GPF/AF exception set in IBuffer |

Exception priority (ExceptionType.merge): iTLB(PF/GPF/AF) > PMP(AF) > ECC(AF)

## 1.6 Timing Hints

| Module | Critical Path | Notes |
| --- | --- | --- |
| IFU F1 | PC adder | f1_pc_lower_result: 16 parallel adders (CatPC optimization) |
| IFU F2 | ICache resp addr match | fromICache.bits.vaddr[0] === f2_ftq_req.startAddr — timing critical (IFU.scala:366) |
| BPU S2/S3 | TAGE/SC multi-table lookup | Multiple folded history XOR + SRAM reads |
| FTQ | FtqPtr comparison | isAfter / isBefore on 64-entry circular queue |
| IBuffer | enqOffset PopCount | enqOffset = PopCount(io.in.bits.valid.take(i)), PredictWidth=16 wide |
| BPU→IFU | flushFromBpu.shouldFlushByStage2/3 | FtqPtr comparison, must be consumed without delay in IFU F0 |

# 2. Frontend I/O Sequence Diagrams

## 2.1 I/O Summary

### 2.1.1 Inputs (FrontendInlinedImp.io)

| Port | Protocol | Source | Description |
| --- | --- | --- | --- |
| io.backend.toFtq.redirect | Valid | Backend → FTQ | Misprediction/MemVio redirect |
| io.backend.toFtq.commit | Valid FtqPtr | Backend → FTQ | ROB commit signal (update FTQ commit ptr, MMIO lastCommit check) |
| io.backend.canAccept | Bool | Backend → IBuffer | Decode can accept |
| io.backend.wfi.wfiReq | Bool | Backend → ICache/Uncache | WFI request |
| io.reset_vector | UInt | SoC → BPU | Reset start PC |
| io.sfence | Bundle | CSR → iTLB | TLB flush |
| io.tlbCsr | Bundle | CSR → iTLB/BPU | SATP/STATUS etc. |
| io.csrCtrl | Bundle | CSR → modules | bp_ctrl, frontend_trigger, pf_ctrl etc. |
| io.ptw | TlbPtwIO | PTW → iTLB | Page table walk response |
| io.softPrefetch | Valid Vec | Backend → ICache | Software prefetch request |

### 2.1.2 Outputs

| Port | Protocol | Destination | Description |
| --- | --- | --- | --- |
| io.backend.cfVec | DecoupledIO Vec | Frontend → Decode | DecodeWidth CtrlFlow instructions |
| io.backend.fromFtq | Bundle | FTQ → Backend | PC mem write, newest entry etc. |
| io.backend.fromIfu | Bundle | IFU → Backend | gpaddr mem write |
| io.backend.wfi.wfiSafe | Bool | Frontend → Backend | WFI entry safe |
| io.error | L1BusErrorUnitInfo | ICache → Backend | ECC/bus error |
| io.frontendInfo.ibufFull | Bool | IBuffer → Backend | IBuffer full status |
| io.frontendInfo.bpuInfo | Bundle | FTQ → Backend | BPU hit/miss counters |

## 2.2 Group A: Normal Fetch (ICache Hit)

BPU prediction → FTQ enqueue → IFU fetch → ICache hit → IBuffer enqueue → Decode delivery

Sequence Diagram A: Normal Fetch (ICache Hit)

=============================================

BPU             FTQ             IFU             ICache          IBuffer         Backend

|               |               |               |               |               |

| [C0] S0: PC mux, history gen  |               |               |               |

|               |               |               |               |               |

| [C1] S1: UBTB+ABTB+UTAGE+MicroRAS lookup     |               |               |

|--prediction-->|               |               |               |               |

|  startPc,     |               |               |               |               |

|  ftqIdx,      |               |               |               |               |

|  s3Override=0 |               |               |               |               |

|               |--toIfu.req--->|               |               |               |

|               |  startAddr,   |               |               |               |

|               |  nextline,    |               |               |               |

|               |  ftqIdx       |               |               |               |

|               |--toICache.req------------>|               |               |

|               |               |               |               |               |

|               |         [C2] F1: PC calc (CatPC adder), f1_valid=1           |

|               |               |               |               |               |

|               |         [C3] F2: ICache resp received          |               |

|               |               |<--fetch.resp.valid------------|               |

|               |               |  data, exception, paddr       |               |

|               |         [C3] F2: predecode, exception, instr_range            |

|               |               |               |               |               |

|               |         [C4] F3: RVC expand, PredChecker, f3_valid=1         |

|               |               |--toIbuffer.valid------------->|               |

|               |               |  instrs, pd, pc, ftqPtr       |               |

|               |<--pdWb.valid--|               |               |               |

|               |               |               |         [C5] output          |

|               |               |               |               |--cfVec(0..5)->|

## 2.3 Group B: BPU S3 Override

BPU uses two independent paths to FTQ:
  (1) S1 new enqueue: s1_fire → FTQ entryQueue[bpuPtr] new write, bpuPtr++
  (2) S3 override: s3_override → entryQueue[s3FtqPtr] overwrite, bpuPtr := s3FtqPtr+1

S1 (uBTB/ABTB) prediction is always sent to FTQ first. S3 override occurs only when S3 result != S1 result, performing a post-correction. S2 only causes BPU-internal flush (not sent to FTQ). s3_override is independent of prediction.ready (bpuS3Redirect path).

### FTQ Index Tracking (s3FtqPtr)

| Stage | Code reference | Description |
| --- | --- | --- |
| S1 fire | s2_ftqPtr = RegEnable(io.fromFtq.bpuPtr, s1_fire) | Latch bpuPtr at S1 fire into s2 |
| S2 fire | s3_ftqPtr = RegEnable(s2_ftqPtr, s2_fire) | Forward s2_ftqPtr to S3 |
| S3 | io.toFtq.s3FtqPtr := s3_ftqPtr | Send override target index to FTQ |
| Override | predictionPtr = s3Override ? s3FtqPtr : bpuPtr(0) | Update entryQueue(s3FtqPtr) |
| bpuPtr rollback | when(s3Override) { bpuPtr := s3FtqPtr + 1 } | Move enqueue pointer past override entry |

### FTQ Override Behavior Summary

| Condition | Action | Code reference |
| --- | --- | --- |
| prediction.fire (S1, not s3Override) | entryQueue(bpuPtr) new write, bpuPtr+1 | prediction.fire |
| s3Override=1 | entryQueue(s3FtqPtr) overwrite, bpuPtr := s3FtqPtr+1 | bpuS3Redirect, predictionPtr |
| ifuPtr >= s3FtqPtr | ifuPtr := s3FtqPtr (rollback) | when(ifuPtr >= ftqIdx) |
| pfPtr >= s3FtqPtr | pfPtr := s3FtqPtr (rollback) | when(pfPtr >= ftqIdx) |
| IFU stage idx >= s3FtqPtr | shouldFlushByStage3=true → flush | !isAfter(s3FtqPtr, idxToFlush) |

### 2.3.1 B-1: Standard S3 Override (IFU already ahead)

Sequence Diagram B-1: S3 Override (ifuPtr > s3FtqPtr — IFU already ahead)

===========================================================================

Context: S1 enqueues idx=X → IFU processes idx=X,X+1 → S3 finds tgt_C != tgt_A

BPU             FTQ             IFU

|               |               |

| [C0] S1 fire: PC_A → idx=X enq, s3Override=0, tgt=tgt_A

|--prediction-->|               |

|               | entryQueue(X).startPc:=PC_A, bpuPtr:=X+1

|               |--toIfu.req(idx=X, startAddr=PC_A)-->|

|               |               |               ifuPtr=X

|               |               |

| [C1] S1 fire: PC_A+blk → idx=X+1, s3Override=0

|--prediction-->|               |

|               | entryQueue(X+1):=PC_A+blk, bpuPtr:=X+2

|               |--toIfu.req(idx=X+1)--------->|

|               |               |               ifuPtr=X+1

|               |               |

| [C2] S3 fire: s3_prediction(PC_A)=tgt_C != s1_prediction=tgt_A → s3_override=1

|               | s2_flush=1 (BPU internal S1+S2 in-flight kill), s0_startPc:=tgt_C

|--prediction(s3Override=1, s3FtqPtr=X, tgt=tgt_C)-->|

|               | bpuS3Redirect=1                |

|               | entryQueue(X).tgt := tgt_C     |

|               | bpuPtr := X+1 (discard X+1)    |

|               | ifuPtr=X+1 >= s3FtqPtr=X → ifuPtr:=X (rollback)

|               |--flushFromBpu.s3(valid=1, bits=X)-->|

|               |               | shouldFlushByStage3(idx>=X) → all in-flight invalidated

|               |               |

| [C3] FTQ re-issues IFU request with corrected target

|               | bpuPtr=X+1, ifuPtr=X → bypass-2 applied

|               |--toIfu.req(idx=X, nextStartVAddr=tgt_C)-->|

### 2.3.2 B-2: Early S3 Override (IFU still behind)

Sequence Diagram B-2: Early S3 Override (ifuPtr <= s3FtqPtr — IFU still behind)

=================================================================================

Context: FTQ back-pressure stalls IFU → ifuPtr < s3FtqPtr

BPU             FTQ             IFU

|               |               |

| [C0] S1 fire: PC_A → idx=X enq, IFU stall (ifuPtr=X-1)

|--prediction-->|               |

|               | entryQueue(X):=PC_A, bpuPtr:=X+1

|               |               IFU stall: ifuPtr=X-1

|               |               |

| [C1] S2: internal BPU processing

|               |               |

| [C2] S3 fire: s3_prediction(PC_A)=tgt_C != s1_prediction → s3_override=1

|--prediction(s3Override=1, s3FtqPtr=X, tgt=tgt_C)-->|

|               | bpuS3Redirect=1                |

|               | entryQueue(X).tgt := tgt_C     |

|               | bpuPtr := X+1                  |

|               | ifuPtr=X-1 < s3FtqPtr=X → NO rollback

|               |--flushFromBpu.s3(valid=1, bits=X)-->|

|               |               | shouldFlushByStage3(idx=X-1):

|               |               |   !isAfter(X, X-1) = false → no flush

|               |               |   in-flight fetch idx=X-1 preserved

|               |               |

| [C3] IFU requests idx=X with updated tgt_C

|               |--toIfu.req(idx=X, nextStartVAddr from SRAM=tgt_C)-->|

### 2.3.3 B-3: Back-pressure S3 Override (prediction.ready=0)

Sequence Diagram B-3: Back-pressure S3 Override (prediction.ready=0)

=====================================================================

Context: FTQ full → new S1 enqueue blocked, but s3_override executes independently

BPU             FTQ             IFU

|               |               |

| [C0] FTQ full: prediction.ready=0

|--prediction(s3Override=0)-->| (new S1 attempt)

|               |<--ready=0   | (FTQ full: bpuPtr - deqPtr >= FtqSize)

|               |               s1_fire=0 (stall)

|               |               Note: S3 has PC_A already latched

|               |               |

| [C1] S3 override fires (independent of prediction.ready)

| S3: s3_prediction(PC_A)=tgt_C != s1_prediction → s3_override=1

|--prediction(s3Override=1, s3FtqPtr=X, tgt=tgt_C)-->|

|               | bpuS3Redirect = prediction.valid && s3Override = 1 (ready-independent)

|               | entryQueue(X).tgt := tgt_C

|               | bpuPtr := X+1

|               | ifuPtr >= X → ifuPtr:=X (rollback)

|               |--flushFromBpu.s3(valid=1, bits=X)-->|

|               |               | IFU flush

|               |               |

| [C2] FTQ slot freed → prediction.ready=1

|               |<--ready=1   |

|--prediction(s3Override=0, startPc=tgt_C)-->|

|               | entryQueue(X): new S1 enq

|               |--toIfu.req(idx=X, startAddr=PC_A, nextStartVAddr=tgt_C)-->|

## 2.4 Group C: Backend Redirect (Misprediction Flush)

ROB detects misprediction → full Frontend flush → restart from correct PC.

Sequence Diagram C: Backend Redirect (Misprediction Flush)

===========================================================

Context: ROB detects misprediction → full Frontend flush → restart from correct PC

Backend(ROB)    FTQ             BPU             IFU             IBuffer

|               |               |               |               |

| [CN] Misprediction detected

|--toFtq.redirect.valid(BranchPredictionRedirect: target, ftqIdx, ftqOffset)-->|

|               |               |               |               |

| [CN+1] needFlush = RegNext(redirect.valid)

|               |--toIfu.redirect.valid------------------->|

|               |               |               | f0/f1/f2/f3_flush

|               |--icacheFlush---------------------------------------->(ICache)

|               |--toBpu.redirect.valid-------->|

|               |               | BPU: GHR/PHR rollback

|               |               | s3/s2/s1_valid := false

|               |               |               |--flush---------->|

|               |               |               |               | full IBuffer flush

|               |               |               |               |

| [CN+2] BPU restarts

|               |               | S0: redirect.target → new startPc

|               |<--prediction(new start)-------|

|               |               |               |

| [CN+3] IFU restarts

|               |--toIfu.req(new FetchRequestBundle)---------->|

|               |               |               |

|               |               |               | (waiting for new instructions)

## 2.5 Group D: ICache Miss (Stall)

ICache miss → IFU F2 stall → FTQ back-pressure.

Sequence Diagram D: ICache Miss (Stall)

========================================

Context: ICache miss → IFU F2 stalls → FTQ back-pressure

FTQ             IFU             ICache          MissHandler

|               |               |               |

|         [C2] F2: ICache miss  |               |

|               | f2_valid=1, f2_icache_all_resp_wire=0

|               |--icacheStop := !f3_ready------>|

|               |               |--miss req---->|

|               |               |               | → L2 fetch

|               |               |               |

| [C2~X] F2 stall               |               |

|               | f2_fire=0 (icacheRespAllValid=0)

|<--fromFtq.req.ready=0---------|

|  (FTQ stall: f1_ready=0)      |               |

|               |               |               |

| [CX] L2 responds, miss resolved               |

|               |               |<--refill-------|

|               |<--fetch.resp.valid=1-----------|

|               | icacheRespAllValid=1, f2_fire=1

|<--fromFtq.req.ready=1---------|

|  (resume)     |               |               |

|               | TopDown: icacheMissBubble → topdown_stages(1)

## 2.6 Group E: MMIO Fetch

MMIO-region instruction fetch via InstrUncache, serialized per ROB commit.

Sequence Diagram E: MMIO Fetch

================================

Context: MMIO-region instruction fetch via InstrUncache, serialized per ROB commit

IFU(F3)         FTQ             InstrUncache    Backend(ROB)

|               |               |               |

| F2: f2_pmp_mmio detected

| → F3: MMIO mode entered       |               |

|               |               |               |

| F3: MMIO fetch request        |               |

|--toUncache.valid(InsUncacheReq: addr)-------->|

|               |               |<--ready-------|

|               |               | (latency L)   |

|               |               |--fromUncache.valid(InsUncacheResp: data)-->|

|               |               |               |

| MMIO instr extracted (1 instruction at a time)

|--mmioCommitRead.valid-------->|               |

|               |               |               |

| ROB commits the MMIO instruction              |

|               |<--toFtq.commit.valid(ROB commit ptr)-------------------|

|               |--mmioLastCommit (commit ptr >= mmioPtr)--------------->|

|               |               |               |

| [if mmioLastCommit=1]         |               |

| mmio_redirect → next MMIO PC (flush F1/F2)

|--pdWb(misOffset, target)----->|               |

|               |               |               |

| [else: wait (stall)]          |               |

## 2.7 Edge Cases

| Case | Behavior |
| --- | --- |
| IBuffer full → fetch stall | allowEnq=false → io.in.ready=false → IFU stall → F3 stall → ICache stall → FTQ back-pressure |
| FTQ full → BPU stall | ftqFullStall → s3_ready=false → s2/s1 stall chain → s0 PC freeze |
| Cross-cacheline fetch | f0_doubleLine=1 → ICache fetches two cachelines; F2 combines with Cat(data,data)+cut() |
| Cross-page RVI exception | F2: last-in-line 32-bit instruction; CrossPF/GPF/AF set in IBuffer |
| Flush classification (TopDown) | ControlBTBMissBubble / TAGEMissBubble / SCMissBubble / ITTAGEMissBubble / RASMissBubble |

# 3. Predictor (BPU) Analysis

## 3.1 Module Summary

Role: 3-stage branch prediction pipeline. S1 (uBTB+ABTB+uTAGE), S2 (MBTB+TAGE+SC), S3 (MBTB-latched+ITTAGE+RAS). FTQ receives S1 prediction, and S3 sends override updates only when it differs from S1. Location: Frontend.scala → Module(new Predictor) inside FrontendInlinedImp. Pipeline: 3 register stages (S1/S2/S3); S0 is the combinational launch phase.

## 3.2 Key Parameters

| Parameter | Source | Default | Effect |
| --- | --- | --- | --- |
| numDup | HasBPUConst.numDup | 4 | PC/history register copies (timing closure) |
| HistoryLength | XSCoreParamsKey.HistoryLength | 256 | Global history register width |
| numBr | XSCoreParamsKey.numBr | 2 | Branch slots per FTB entry |
| totalSlot | numBr | 2 | Total slots (brSlots + tailSlot) |
| MaxMetaLength | HasBPUConst.MaxMetaLength | 512 | Prediction metadata storage bits |
| FtqSize | XSCoreParamsKey.FtqSize | 64 | FTQ size (BPU stall threshold) |
| numBpStages | constant | 3 | Pipeline stage count |

## 3.3 Interfaces

| Port | Dir | Protocol | Description |
| --- | --- | --- | --- |
| io.bpu_to_ftq.resp | out | Decoupled | Prediction result → FTQ (S1 new enqueue or S3 override update) |
| io.ftq_to_bpu.redirect | in | Valid | FTQ→BPU redirect (misprediction) |
| io.ftq_to_bpu.update | in | Valid | FTQ→BPU training data |
| io.ftq_to_bpu.enq_ptr | in | — | FTQ current enqueue pointer |
| io.ctrl | in | — | BPUCtrl: enable/disable per predictor |
| io.reset_vector | in | — | Reset PC |

## 3.4 BPU Pipeline Architecture

BPU (Predictor) Pipeline — Internal Architecture

=================================================

NOTE: All predictors receive the SAME s0_startPc simultaneously.

S1/S2/S3 results differ only in LOOKUP LATENCY, not in which PC is processed.

Cycle T:   S0 (combinational)

─────────────────────────────────────────────────────────────────

s0_startPc = MuxCase(s0_startPcReg, [

redirect.valid  → redirect.bits.target,        # highest priority

s3_override     → s3_prediction.target,

s1_valid        → s1_prediction.target,

else            → s0_startPcReg (hold)

])

All predictors (ubtb, abtb, utage, uras, mbtb, tage, sc, ittage, ras)

receive s0_startPc simultaneously.

Cycle T+1: S1 (1-cycle latency results)

─────────────────────────────────────────────────────────────────

Results available:

uBTB  → 1 candidate (index 0), always-taken block BTB (32-entry fully-assoc)

ABTB  → up to 8 candidates (index 1..8), block BTB (8-way set)

uTAGE → direction correction for conditional branches

uRAS  → fast return target (S1 early RAS)

Combined candidate pool: s1_btbPrediction[0..8]

[0] = uBTB output   [1..8] = ABTB outputs

(Not competing — merged into one Vec, position-sorted)

takenMask[i] = pred[i].valid &&

(direct || indirect || (conditional && Mux(utageHit, utageTaken, pred.taken)))

CompareMatrix(positions) → getLeastElementOH(takenMask)

→ s1_firstTakenBranchOH (1-hot, smallest cfiPosition among taken)

→ s1_prediction = Mux(s1_taken, firstTakenBranch, fallThrough)

if s1_prediction.isReturn && uras.isCanUse:

s1_prediction.target := uras.retTarget

s1_fire → FTQ: prediction.valid=1, s3Override=0  (new entry enqueue)

s1_prediction.target → next cycle s0_startPc

Cycle T+2: S2 (2-cycle latency results)

─────────────────────────────────────────────────────────────────

Results available:

MBTB  → s2_mbtbResult (main BTB, more accurate, block BTB 32B half-block)

TAGE  → direction prediction (8 tables, different history lengths)

SC    → Statistical Corrector override (threshold-gated)

s2_condTakenMask: apply tage/sc direction to mbtb entries

Mux(sc.scUsed → sc.scTaken, tage.useProvider → tage.pred, else → mbtb.taken)

s2_takenMask = s2_condTakenMask || jumpMask (direct/indirect always taken)

S2 → NOT sent to FTQ. Passes to S3 only.

s2_flush = s3_flush || s3_override  (BPU internal flush)

Cycle T+3: S3 (3-cycle latency results)

─────────────────────────────────────────────────────────────────

Results available:

ITTAGE  → indirect branch target

RAS     → return address (full speculative RAS)

s3_mbtbResult   = RegEnable(s2_mbtbResult, s2_fire)  (latched from S2)

s3_takenMask    = RegEnable(s2_takenMask, s2_fire)

s3_s1Prediction = RegEnable(RegEnable(s1_prediction, s1_fire), s2_fire)

target priority:

1. ras.topRetAddr   (if isReturn)

2. ittage.target    (if needIttage && ittage.hit)

3. mbtb.target      (if taken)

4. fallThrough

s3_override = s3_valid && !(s3_prediction === s3_s1Prediction)

if s3_override:

→ FTQ: prediction.valid=1, s3Override=1  (existing entry update)

→ s2_flush=1 (kill BPU in-flight S1+S2)

→ s0_startPc := s3_prediction.target (2nd priority MUX)

## 3.5 BTB / FTB Hierarchy

BTB/FTB Hierarchy (KunMingHu v3)

==================================

This version evolves from uFTB→FTB (2-tier) to uBTB→ABTB→MBTB (3-tier).

All BTBs are BLOCK BTBs: one entry covers one fetch block.

+──────────────────────────────────────────────────────────+

| Stage | BTB   | Direction  | Latency | Entry Coverage    |

+──────────────────────────────────────────────────────────+

| S1    | uBTB  | uTAGE      | 1 cycle | 64B fetch block   |

|       | ABTB  | uTAGE      | 1 cycle | 64B fetch block   |

+──────────────────────────────────────────────────────────+

| S2    | MBTB  | TAGE + SC  | 2 cycle | 32B half-block    |

+──────────────────────────────────────────────────────────+

| S3    | —     | ITTAGE+RAS | 3 cycle | (target refine)   |

+──────────────────────────────────────────────────────────+

uBTB: 32 entries, fully-associative, tag=PC[22:1], always-taken

ABTB: 8-way set, tag=PC[24:1], up to 8 candidates per lookup

MBTB: 2 AlignBanks × 4 ways, tag=PC[31:16], up to 8 entries per 32B half-block

## 3.6 uBTB (Micro BTB) — S1 Stage

| Property | Value |
| --- | --- |
| BTB type | Block BTB — tag = fetch block start PC[22:1], 1 entry per fetch block |
| Structure | Fully Associative Cache |
| Entries | 32 |
| Tag width | 22 bits |
| Target width | 22 bits (2B aligned) |
| Useful counter | 2-bit saturating (signed, SaturateNegative = invalid) |
| Replacement | PLRU (least-useful first) |
| History | None |
| Prediction latency | 1 cycle |
| Behavior | Always-taken predictor — if hit, taken=true |

## 3.7 Backpressure / Flush Control

| Condition | Behavior | Code reference |
| --- | --- | --- |
| FTQ full (prediction.ready=0) | s1_fire=0 → S1 stall, S0 stall cascade | s1_fire := s1_valid && s2_ready && prediction.ready |
| s3_override | s2_flush=1, s1_flush=1 → BPU internal S1+S2 kill | s2_flush := s3_flush ¦¦ s3_override |
| redirect.valid | s3_flush=1 → s3_valid cleared, cascade to s2/s1 | s3_flush := redirect.valid |
| s0_stall | s0_startPcReg held (PC freeze) | s0_stall := !(s1_valid ¦¦ s3_override ¦¦ redirect.valid) |
| s3_override + FTQ full | bpuS3Redirect executes regardless of prediction.ready | Ftq.scala: bpuS3Redirect = prediction.valid && s3Override |

# 4. NewIFU Analysis

## 4.1 Module Summary

Role: Fetches instructions from ICache using the fetch address supplied by FTQ, performs predecode, and delivers results to IBuffer. MMIO instructions are handled separately via InstrUncache. Location: Frontend.scala → Module(new NewIFU) inside FrontendInlinedImp. Pipeline: 3 register stages (F1/F2/F3) + F0 combinational launch phase.

## 4.2 Key Parameters

| Parameter | Source | Default | Effect |
| --- | --- | --- | --- |
| PredictWidth | HasXSParameter.PredictWidth | 16 | Max half-words per fetch block |
| HasCExtension | HasXSParameter.HasCExtension | true | RVC (16-bit) instruction support |
| CommitWidth | XSCoreParamsKey.CommitWidth | 6 | rob_commits port count |
| fetchQueueSize | HasIFUConst.fetchQueueSize | 2 | MMIO fetch internal queue size |
| blockOffBits | HasICacheParameters | 6 | Cache line offset bits (64B → 6 bits) |
| mmioBusWidth | HasInstrMMIOConst | 64 | MMIO fetch bus width (bits) |

## 4.3 Interfaces

| Port | Dir | Protocol | Description |
| --- | --- | --- | --- |
| io.ftqInter.fromFtq.req | in | Decoupled | FTQ→IFU fetch request (FetchRequestBundle) |
| io.ftqInter.fromFtq.redirect | in | Valid | Backend redirect |
| io.ftqInter.fromFtq.flushFromBpu | in | — | BPU S2/S3 flush info |
| io.ftqInter.toFtq.pdWb | out | Valid | Predecode writeback → FTQ |
| io.icacheInter.resp | in | ValidIO | ICache→IFU response |
| io.icacheInter.icacheReady | in | Bool | ICache ready (F0 req.ready condition) |
| io.icacheStop | out | Bool | ICache stop request (!f3_ready) |
| io.toIbuffer | out | Decoupled | IFU→IBuffer instruction delivery |
| io.uncacheInter.toUncache | out | Decoupled | MMIO fetch request |
| io.uncacheInter.fromUncache | in | Decoupled | MMIO fetch response |
| io.iTLBInter | in/out | — | MMIO re-translate iTLB (blockable) |
| io.pmp | in/out | — | PMP check (last IFU port) |

## 4.4 Internal Pipeline

NewIFU Internal Pipeline

=========================

F0 (combinational — launch phase)

──────────────────────────────────

fromFtq.req.valid → f0_valid

f0_doubleLine = startAddr[blockOffBits-1] === 1  (cross-cacheline fetch)

f0_vSetIdx = [get_idx(startAddr), get_idx(nextlineStart)]

f0_fire = fromFtq.req.fire = f0_valid && f1_ready && icacheReady

f0_flush_from_bpu = shouldFlushByStage2(ftqIdx) || shouldFlushByStage3(ftqIdx)

Note: FTQ sends req directly to ICache; IFU receives only the response.

F1 (1 register stage — PC calculation)

────────────────────────────────────────

f1_valid  = RegInit(false.B)

f1_ftq_req = RegEnable(f0_ftq_req, f0_fire)

PC adder optimization (PcCutPoint = VAddrBits/4 - 1):

f1_pc_lower_result(i) = Cat(0.U(1.W), startAddr[PcCutPoint-1:0]) + (i*2).U

f1_pc = CatPC(f1_pc_lower_result, f1_pc_high, f1_pc_high_plus1)

f1_cut_ptr(i) = startAddr[blockOffBits-1:1] + i  (ICache data slice pointer)

f1_fire = f1_valid && f2_ready

F2 (1 register stage — ICache response + Predecode)

─────────────────────────────────────────────────────

f2_valid = RegInit(false.B)

Wait for ICache response: f2_icache_all_resp_wire (timing-critical path)

= fromICache.valid

&& fromICache.bits.vaddr[0] === f2_ftq_req.startAddr

&& (!f2_doubleLine || fromICache.bits.vaddr[1] === f2_ftq_req.nextlineStart)

Data extraction:

f2_data_2_cacheline = Cat(fromICache.bits.data, fromICache.bits.data)

f2_cut_data = cut(f2_data_2_cacheline, f2_cut_ptr)  (PredictWidth+1 × 16-bit slices)

Exception generation:

f2_exception = ExceptionType.merge(iTLB_exception, mmio_mismatch_exception)

Priority: iTLB(PF/GPF/AF) > PMP(AF) > ECC(AF)

Predecode (combinational):

preDecoder.in = {f2_cut_data, f2_pc, frontendTrigger}

f2_pd = preDecoder.out.pd  (PreDecodeInfo per instruction)

f2_instr_range = jump_range & ftr_range

f2_fire = f2_valid && f3_ready && icacheRespAllValid

F3 (1 register stage — RVC expand + PredChecker + IBuffer send)

─────────────────────────────────────────────────────────────────

f3_valid = RegInit(false.B)

RVC expansion: expanders(i): RVCExpander → f3_expd_instr(i)

F3Predecoder: finalize brType/isCall/isRet

PredChecker: compare BPU prediction vs actual predecode result

Faults detected:

jalFault, jalrFault, retFault, targetFault, notCFIFault, invalidTakenFault

→ wb_redirect=true → IFU sends correction to FTQ

MMIO handling (if f3_pmp_mmio):

→ MMIO FSM → toUncache.valid=true

→ mmio_redirect → f1/f2 flush

→ wait mmioLastCommit → next MMIO fetch

Outputs:

io.toIbuffer.valid = f3_valid && !mmio_state

toFtq.pdWb.valid   = (predecode writeback)

Flush propagation:

f3_flush = backend_redirect || (wb_redirect && !f3_wb_not_flush)

f2_flush = backend_redirect || mmio_redirect || wb_redirect

f1_flush = f2_flush

f0_flush = f1_flush || f0_flush_from_bpu

## 4.5 Submodules

| Submodule | Role |
| --- | --- |
| PreDecode | Find instruction boundaries in 16-bit half-word stream; RVC detection; branch type first pass |
| F3Predecoder | Finalize brType/isCall/isRet in F3 |
| PredChecker | Compare BPU prediction vs actual predecode; generate IFU redirect if mismatch |
| FrontendTrigger | Hardware debug triggers |
| RVCExpander × PredictWidth | Expand 16-bit RVC to 32-bit |

## 4.6 Flush Propagation

| Condition | Behavior |
| --- | --- |
| IBuffer full (!toIbuffer.ready) | f3_fire=0, f3_valid held; icacheStop=!f3_ready → ICache stall |
| ICache not ready (!icacheReady) | fromFtq.req.ready=0 → FTQ stall |
| ICache miss (!icacheRespAllValid) | f2_fire=0, f2_valid held → f1_ready=0 → FTQ stall |
| Backend redirect | f0~f3 flush, f1/f2/f3_valid=0 |
| BPU S2/S3 flush | f0_flush_from_bpu → f0 flush only (F1+ preserved) |
| wb_redirect | f2/f3 flush; correction info sent to FTQ |
| MMIO redirect | f1/f2 flush; F3 occupied until MMIO complete |

## 4.7 Exceptions

| Exception | Detection | Handling |
| --- | --- | --- |
| Page Fault (PF) | F2: fromICache.bits.exception (iTLB result) | ExceptionType.pf → FetchToIBuffer.exceptionType |
| Guest Page Fault (GPF) | F2: iTLB response | ExceptionType.gpf → same path |
| Access Fault (AF) | F2: PMP/ECC/TileLink corrupt | ExceptionType.af → same path |
| MMIO mismatch | F2: double-line pmp_mmio/itlb_pbmt mismatch | AF on second line |
| Cross-page exception | F2: last-in-line RVI crossing page | CrossPF/GPF/AF in IBuffer |
| Illegal RVC | F3: expander.io.ill | IBufferExceptionType.rvcII |

# 5. FTQ (Fetch Target Queue) Analysis

## 5.1 Module Summary

Role: Central queue that buffers BPU predictions and supplies fetch requests to IFU/ICache/Prefetch in order. Receives IFU predecode writeback and Backend commit/redirect to update/restore BPU. Location: Frontend.scala → Module(new Ftq) inside FrontendInlinedImp. Structure: Pointer-based circular queue with SRAM storage — no register pipeline stages.

## 5.2 Key Parameters

| Parameter | Source | Default | Effect |
| --- | --- | --- | --- |
| FtqSize | XSCoreParamsKey.FtqSize | 64 | Max in-flight fetch blocks (circular) |
| PredictWidth | HasXSParameter.PredictWidth | 16 | commitStateQueue slots per entry |
| copyNum | Ftq.copyNum | 5 | Pointer copies (reduce fanout) |
| FtqRedirectAheadNum | HasXSParameter | ~4 | BjuCnt-based redirect ahead ports |

## 5.3 Interfaces

| Port | Dir | Protocol | Description |
| --- | --- | --- | --- |
| io.fromBpu.resp | in | Decoupled | BPU prediction result |
| io.toBpu.redirect | out | Valid | Misprediction → BPU redirect |
| io.toBpu.update | out | Valid | Commit info → BPU training |
| io.toBpu.enq_ptr | out | — | Current bpuPtr |
| io.fromIfu.pdWb | in | Valid | IFU predecode writeback |
| io.toIfu.req | out | Decoupled | Fetch request to IFU |
| io.toIfu.redirect | out | Valid | IFU flush |
| io.toIfu.flushFromBpu | out | — | BPU S2/S3 override flush info |
| io.toICache.req | out | Decoupled | ICache fetch request (5 copies) |
| io.fromBackend | in | CtrlToFtqIO | redirect, rob_commits, ftqIdxAhead etc. |
| io.toBackend | out | FtqToCtrlIO | pc_mem write, newest_entry info |
| io.icacheFlush | out | Bool | ICache pipeline flush |

## 5.4 Pointer Structure

FTQ Pointer Structure

======================

Invariant: commPtr <= ifuWbPtr <= ifuPtr <= bpuPtr

bpuPtr  ──► [X] BPU enqueue position

ifuPtr  ──► [Y] Next entry to send to IFU

ifuWbPtr──► [Z] IFU predecode writeback confirmed position

commPtr ──► [W] Last committed entry

entry_fetch_status:  f_to_send (0) / f_sent (1)

commitStateQueue:    c_empty / c_toCommit / c_committed / c_flushed  (2-bit per slot)

Circular queue size: FtqSize=64 entries

Each entry covers one fetch block (64B, up to 16 instructions)

## 5.5 Internal SRAM / Register Structures

| Name | Type | Size | Role |
| --- | --- | --- | --- |
| ftq_pc_mem | SyncDataModuleTemplate | FtqSize × Ftq_RF_Components | PC/nextLine storage, multiple read ports |
| ftq_redirect_mem | SyncDataModuleTemplate | FtqSize × Ftq_Redirect_SRAMEntry | Branch history/RAS state (redirect recovery) |
| ftq_meta_mem | SyncDataModuleTemplate | FtqSize × MetaEntry | BPU metadata + FTBEntry (training) |
| ftq_pd_mem | SyncDataModuleTemplate | FtqSize × Ftq_pd_Entry | IFU predecode result (brMask, jmpInfo) |
| ftb_entry_mem | SyncDataModuleTemplate | FtqSize × FTBEntry_FtqMem | FTB entry (redirect validation) |
| update_target | Reg Vec | FtqSize × VAddrBits | Predicted target addresses |
| cfiIndex_vec | Reg Vec | FtqSize × ValidUInt | CFI position index |
| commitStateQueueReg | RegInit Vec | FtqSize × Vec(PW, 2bit) | Per-slot commit state |
| entry_fetch_status | RegInit Vec | FtqSize × 1bit | f_to_send / f_sent |

## 5.6 Key Mechanisms

### BPU Enqueue 1-Cycle Delay

To break the critical path, entry_fetch_status / cfiIndex_vec / update_target are updated one cycle after BPU enqueue (last_cycle_bpu_in = RegNext(bpu_in_fire)). A bypass path (bpu_in_bypass_buf) supplies the just-enqueued PC directly to IFU when bpu_in_bypass_ptr === ifuPtr, avoiding SRAM read latency.

### FTQ Full Detection

validEntries = distanceBetween(bpuPtr, commPtr). When validEntries >= FtqSize → new_entry_ready=false → io.fromBpu.resp.ready=false → BPU back-pressure. Even when FTQ full, if canCommit fires, enqueue is allowed (deadlock prevention).

## 5.7 Backpressure / Flush Control

| Condition | Behavior |
| --- | --- |
| FTQ full | io.fromBpu.resp.ready=false → BPU stall |
| Backend redirect | allowBpuIn/ToIfu=false (2 cycles), bpuPtr/ifuPtr restored |
| BPU S2/S3 override | bpuPtr rollback, ifuPtr rollback (if needed), flushFromBpu sent |
| IFU not ready | toIfu.req.ready = f1_ready && icacheReady → ifuPtr stalls |
| Entry not f_to_send | entry_is_to_send=false → toIfu.req.valid=false |
| ifuFlush | allowToIfu=false → IFU request blocked |
| MMIO commit wait | mmioCommitRead.valid + mmioLastCommit → IFU control |

## 5.8 Timing Hints

| Critical Path | Description |
| --- | --- |
| entry_is_to_send | entry_fetch_status(ifuPtr.value) === f_to_send — indexed by ifuPtr |
| validEntries | distanceBetween(bpuPtr, commPtr) — directly gates io.fromBpu.resp.ready |
| toIfuPcBundle mux | 3-way mux: bypass / last_cycle_to_ifu_fire / SRAM read |
| commitStateQueueReg | FtqSize × PredictWidth 2-bit registers — copyNum split reduces fanout |
| FTBEntryGen | Combinational: br slot insertion/movement, pftAddr calculation |

# 6. IBuffer Analysis

## 6.1 Module Summary

Role: Buffers fetch packets (up to PredictWidth instructions) received from IFU and supplies up to DecodeWidth CtrlFlow entries to Decode each cycle. Applies back-pressure when full; flushes entirely on backend redirect. Location: Frontend.scala → Module(new IBuffer) inside FrontendInlinedImp. Pipeline: 1 output register stage + optional bypass path.

## 6.2 Key Parameters

| Parameter | Source | Default | Effect |
| --- | --- | --- | --- |
| IBufSize | IBufferParameters.Size | 48 | Total buffer entries |
| NumWriteBank | IBufferParameters.NumWriteBank | 4 | Write-bank count (enqueue datapath) |
| NumReadBank | IBufferParameters.NumReadBank | 8 | Read-bank count (must be ≥ DecodeWidth) |
| PredictWidth | HasXSParameter.PredictWidth | 16 | Max enqueue instructions per cycle |
| DecodeWidth | XSCoreParamsKey.DecodeWidth | 6 | Max dequeue instructions per cycle |
| ReadBankSize | IBufSize / NumReadBank | 6 | Entries per read bank |
| WriteBankSize | IBufSize / NumWriteBank | 12 | Entries per write bank |

Constraints: IBufSize % NumWriteBank == 0; IBufSize % NumReadBank == 0; NumReadBank >= DecodeWidth

## 6.3 Interfaces

| Port | Dir | Protocol | Description |
| --- | --- | --- | --- |
| io.in | in | Decoupled | IFU → IBuffer (PredictWidth instructions) |
| io.out(i) | out | Decoupled | IBuffer → Decode (DecodeWidth entries) |
| io.flush | in | Bool | Backend redirect → full flush |
| io.decodeCanAccept | in | Bool | Decode can accept |
| io.full | out | Bool | IBuffer full (!allowEnq) |
| io.ControlBTBMissBubble/TAGEMissBubble/... | in | Bool | Flush reason for TopDown classification |
| io.stallReason | out | StallReasonIO | TopDown stall reason vector |

## 6.4 Internal Structure

IBuffer Bank Layout (Interleaved)

===================================

IBufSize=48, NumReadBank=8, ReadBankSize=6, NumWriteBank=4, WriteBankSize=12

Linear index:  0  1  2  3  4  5  6  7  8  9  10  11  12 ...

Bank mapping:

Bank 0: ibuf[0, 8, 16, 24, 32, 40]

Bank 1: ibuf[1, 9, 17, 25, 33, 41]

Bank 2: ibuf[2,10, 18, 26, 34, 42]

Bank 3: ibuf[3,11, 19, 27, 35, 43]

Bank 4: ibuf[4,12, 20, 28, 36, 44]

Bank 5: ibuf[5,13, 21, 29, 37, 45]

Bank 6: ibuf[6,14, 22, 30, 38, 46]

Bank 7: ibuf[7,15, 23, 31, 39, 47]

Dequeue 2-stage read:

Stage 1: each bank selects 1 entry (bankSize:1 Mux per bank)

Stage 2: each output slot selects from NumReadBank candidates (NumReadBank:1 Mux)

→ DecodeWidth=6 reads, each from a different bank → no port conflict

Bypass path: enqPtr==deqPtr && decodeCanAccept

→ IFU data forwarded directly to OutputEntries (skip ibuf registers)

→ saves 1 cycle when buffer is empty

Full detection:

allowEnq := prevInstrCount < nextNumInvalid

(ready is registered and uses previous IFU enqueue demand)

## 6.5 Enqueue Logic

numFromFetch = PopCount(io.in.bits.enqEnable). enqOffset(i) = PopCount(io.in.bits.valid.take(i)) — computes each instruction's write slot. Bypass: when enqPtr==deqPtr && decodeCanAccept, the first DecodeWidth instructions are forwarded directly to OutputEntries; only the remainder is written to ibuf registers. Full guard: allowEnq := prevInstrCount < nextNumInvalid (registered ready logic).

## 6.6 Dequeue Logic (2-Stage Read)

Stage 1: Each bank selects 1 entry via bankSize:1 Mux (deqInBankPtr index). Stage 2: Each output slot selects from NumReadBank results via NumReadBank:1 Mux (deqBankPtr index). DecodeWidth=6 reads each come from a different bank → no structural hazard.

## 6.7 Exception Handling

| Exception info | Storage | Delivery |
| --- | --- | --- |
| Page Fault (PF) | IBufferExceptionType.NonCrossPF / CrossPF | cf.exceptionVec(instrPageFault) |
| Guest Page Fault (GPF) | NonCrossGPF / CrossGPF | cf.exceptionVec(instrGuestPageFault) |
| Access Fault (AF) | NonCrossAF / CrossAF | cf.exceptionVec(instrAccessFault) |
| Illegal RVC | rvcII | cf.exceptionVec(EX_II) |
| Cross-page IPF fix | CrossPF | cf.crossPageIPFFix |
| Backend exception | backendException: Bool | cf.backendException passed through |

IBufferExceptionType encoding (3-bit): 000=None, 001=NonCrossPF, 010=NonCrossGPF, 011=NonCrossAF, 100=rvcII, 101=CrossPF, 110=CrossGPF, 111=CrossAF. bit[2]: isCrossPage, bit[1:0]: exception type.

## 6.8 Backpressure Chain

Back-pressure propagation:

Backend not accept → decodeCanAccept=false

→ numOut=0 → outputEntries frozen

→ numValidNext increases → allowEnq=false

→ io.in.ready=false → IFU stall → F3 stall

→ icacheStop=true → ICache stall

→ FTQ toIfu.req.ready=false → further IFU requests blocked

## 6.9 Timing Hints

| Critical Path | Description |
| --- | --- |
| Enqueue write mux | IBufSize × PredictWidth Mux1H — select 1 source per ibuf entry |
| Dequeue 2-stage read | Stage1: ReadBankSize:1 Mux × NumReadBank; Stage2: NumReadBank:1 Mux × DecodeWidth |
| enqOffset PopCount | PopCount(valid.take(i)) × PredictWidth — parallel prefix sum |
| outputEntriesValidNum | PriorityMuxDefault over DecodeWidth outputs — priority encoder |
| nextNumInvalid → allowEnq | allowEnq := prevInstrCount < nextNumInvalid (registered ready path) |
| deqBankPtr update | deqBankPtrVec(i) + numDeq × DecodeWidth — parallel circular ptr |

# 7. Open Questions / TODO

MMIO fetch latency: How frequently does the ROB commit wait occur in practice?

numDup=4 register replication: Quantify the timing improvement in post-route results.

f2_mmio_mismatch_exception: Is the cacheable/non-cacheable boundary crossing case fully handled?

PTWFilter ifilterSize: How does it affect iTLB miss frequency?

IBuffer bypass path: How often is bypass actually used in waveform simulations?

uBTB aliasing: PC[VAddrBits-1:23] is ignored in tag comparison — what is the alias rate for large address spaces?

uBTB slot2: Always valid=false currently (TODO: 2-taken support).

FTQ TODO (Ftq.scala): 'wait for IFU/ICache to remove bpu s2 flush' — S2 flush path not yet fully implemented.