# XS Frontend

Generated from Markdown documents in the specified order

# frontend_block_diagram_overview.md

Block: Frontend

Module: N/A (Top-level overview)

Source: Frontend.scala, FrontendBundle.scala, bpu/Bpu.scala, ifu/Ifu.scala, ftq/Ftq.scala, ibuffer/IBuffer.scala

Protocols: Decoupled / Valid / Credit

Key Params: FtqSize, IBuffer.Size, NumWriteBank, NumReadBank, FetchBlockInstNum, DecodeWidth, PhrHistoryLength, GhrHistoryLength

Last updated: 2026-02-18

## 1. Block Summary

XiangShan Frontend is a non-sequential instruction supply pipeline that goes from BPU (Branch Prediction) → FTQ (Fetch Target Queue) → IFU (Instruction Fatch) → IBuffer (Instruction Buffer) → Backend (Decode).

Main input: redirect (misprediction flush), sfence, tlbCsr, csrCtrl from Backend

Main output: io.backend.cfVec (DecodeWidth CtrlFlow, IBuffer → Decode), io.backend.stallReason (stall cause information)

Performance/bottleneck: ICache miss (IFU stall), FTQ full (BPU back-pressure), IBuffer full (fetch stall), misprediction redirect flush

## 2. Key Parameters

| Parameter | Source | Default / Normal value | Impact |
| --- | --- | --- | --- |
| FtqSize | `FtqParameters.FtqSize` | 64 | Number of FTQ entries (BPU-IFU buffer depth) |
| IBuffer.Size | `IBufferParameters.Size` | 48 | IBuffer total number of entries |
| NumWriteBank | `IBufferParameters.NumWriteBank` | 4 | Number of IBuffer write banks (for IFU pre-align) |
| NumReadBank | `IBufferParameters.NumReadBank` | 8 | Number of IBuffer read banks (≥ DecodeWidth) |
| FetchBlockInstNum | `FetchBlockSize(64B) / instBytes` | 16 or 32 | Maximum number of instructions per fetch block |
| DecodeWidth | `p(XSCoreParamsKey).DecodeWidth` | 6 | IBuffer → Decode simultaneous output width |
| PhrHistoryLength | `FrontendParameters.getPhrHistoryLength` | calculated value | PHR (Path History Register) length |
| GhrHistoryLength | `HasBpuParameters.GhrHistoryLength` | SC max table | Global branch history (GHR) length for SC |
| ResolveEntryBranchNumber | `FrontendParameters.ResolveEntryBranchNumber` | 8 | Maximum number of branch slots per FTQ resolve entry |
| ipmpPortNum | `coreParams.ipmpPortNum` | ICache ports | Number of PMP checker ports |
| itlbPortNum | `coreParams.itlbPortNum` | 1 | Number of iTLB ports |

## 3. Top-Level Block Diagram

---

config:

layout: dagre

---

flowchart LR

subgraph BPU["Predictor (BPU)"]

BS3["S3\nMBTB+ITTAGE+RAS\ns3_override?"]

BS2["S2\nMBTB+TAGE+SC\ns2_prediction"]

BS1["S1\nUBTB+ABTB+UTAGE\n+MicroRAS\n1st prediction"]

BS0["S0\nPC mux\nPHR/GHR gen"]

end

subgraph FTQ["Ftq"]

FQ_COMM["commPtr\ncommit from ROB"]

FQ_IFU["ifuPtr\ndeq to IFU"]

FQ_BPU["bpuPtr\nenq from BPU"]

end

subgraph IFU["NewIFU"]

F3["F3\nRVC expand\nPredChecker\nf3_valid RegInit"]

F2["F2\nICache resp\nPredecode\nf2_valid RegInit"]

F1["F1\nPC calc\nf1_valid RegInit"]

F0["F0\nfetch req\nto ICache"]

end

subgraph ICache["ICache"]

IC_PREFETCH["Prefetcher"]

IC_MISS["MissHandler"]

IC_MAIN["MainPipe\n(S0/S1/S2)"]

end

subgraph TLB_PMP["iTLB / PMP"]

PMP["PMP\n+Checker"]

ITLB["TLB\nPortNum+1 ports"]

end

subgraph IBuf["IBuffer"]

IBOUT["Output Reg\nDecodeWidth"]

IBK["IBufNBank bank\n(Banked FIFO)"]

end

BS0 -- s0_fire RegEnable --> BS1

BS1 -- s1_fire RegEnable --> BS2

BS2 -- s2_fire RegEnable --> BS3

FQ_BPU --> FQ_IFU

FQ_IFU --> FQ_COMM

F0 -- f0_fire RegEnable --> F1

F1 -- f1_fire RegEnable --> F2

F2 -- f2_fire ICache resp --> F3

IC_MAIN --> IC_MISS

IBK --> IBOUT

CSR["CSR / sfence\n/ tlbCsr"] -. sfence/tlbCsr/csrCtrl .-> BPU

CSR -. sfence/tlbCsr .-> TLB_PMP

BS3 -- bpu_to_ftq Decoupled --> FTQ

FTQ -- toBpu redirect/update Valid --> BPU

FTQ -- toIfu Decoupled req --> IFU

FTQ -- toICache Decoupled req --> ICache

FTQ -- toPrefetch Decoupled --> IC_PREFETCH

ICache -- "fetch.resp Valid" --> IFU

ICache -- itlb --> ITLB

ITLB -- ptw --> Backend["Backend\n(ROB/LSU)"]

PMP -. pmp resp .-> ICache & IFU

F3 -- toIbuffer Decoupled --> IBuf

F3 -- pdWb Valid --> FTQ

IBuf -- "cfVec + stallReason" --> Backend

Backend -. "toFtq.redirect Valid" .-> FTQ

Backend -. rob_commits .-> IFU

FTQ -. icacheFlush .-> ICache

FTQ -. flushFromBpu BpuFlushInfo .-> IFU

IFU -. mmioCommitRead .-> FTQ

### 5.7 BPU internal s3_override

s3_override = true (Bpu.scala:380: s3_valid && !(s3_prediction === s3_s1Prediction)) when S3 results differ from S1 predictions

When override occurs, retransmit S3 prediction results to FTQ (io.toFtq.prediction.bits.s3Override := s3_override)

There is no separate override of the S2 stage, and when s3_flush occurs, S2/S1 are also flushed.

flushFromBpu delivered from FTQ to IFU → consumed at IFU F0 stage

## 6. Error / Exception Paths

### 6.1 iTLB Exceptions (PF/GPF/AF)

ICache → iTLB request → ExceptionType.fromTlbResp(resp) → fromICache.bits.exception → receive from IFU F2

Create f2_exception_vec → Save to IBuffer.exceptionType at F3 → Pass to Decode

### 6.2 PMP Access Exception (AF)

ExceptionType.fromPMPResp(resp) → merge into ICache response → forward the same path

### 6.3 ECC / TileLink corrupt (AF)

ExceptionType.fromECC(enable, corrupt) / fromTilelink(corrupt)

Output io.error Valid inside ICache → errorReg = RegNext(icache.io.error) → io.error L1BusErrorUnit delivered (Frontend.scala:262-263)

### 6.4 MMIO command processing

IFU F3: Detect f3_pmp_mmio → Via InstrUncache (64-bit unit fetch)

MMIO flush: mmio_redirect → F2/F3 flush, snpc redirect to FTQ

After confirming ROB commit, next MMIO fetch: mmioCommitRead ↔ FTQ

### 6.5 Exception priority (ExceptionType.merge)

iTLB(PF/GPF/AF) > PMP(AF) > ECC(AF) — ExceptionType.merge(...) function reference (FrontendBundle.scala:191)

## 7. Timing Hints

| module | Critical Path Candidate | Description |
| --- | --- | --- |
| IFU F1 | PC calculation adder | `f1_pc_lower_result` 16 adders in parallel (`CatPC` optimization applied) |
| IFU F2 | ICache resp addr match | `fromICache.bits.vaddr(0) === f2_ftq_req.startAddr` — timing critical annotation exists (IFU.scala:366) |
| BPU S2/S3 | TAGE/SC multi-table lookup | Multiple folded history XOR + SRAM read |
| FTQ | Compare FtqPtr | Using `isAfter` function, comparing 64-entry circular queue |
| IBuffer | enqOffsetPopCount | `enqOffset = PopCount(io.in.bits.valid.take(i))` — PredictWidth(16) width |
| BPU → IFU | `flushFromBpu.shouldFlushByStage2/3` | Determination of flush by comparing FtqPtr, must be consumed without delay at IFU F0 |

## 8. Open Questions / TODO

[ ] MMIO fetch latency: How often does ROB commit wait occur?

[ ] Verification of timing saving effect of numDup=4 replication structure

[ ] f2_mmio_mismatch_exception: Cacheable/non-cacheable boundary crossing case handling completeness

Effect of [ ] PTWFilter's ifilterSize parameter on iTLB miss frequency

[ ] How much is the IBuffer bypass path utilized in actual waveforms?

→ See [frontend_in_out_seq_diagram.md](./frontend_in_out_seq_diagram.md) → See [frontend_Predictor_analysis.md](./frontend_Predictor_analysis.md) → See [frontend_NewIFU_analysis.md](./frontend_NewIFU_analysis.md) → See [frontend_Ftq_analysis.md](./frontend_Ftq_analysis.md) → See [frontend_IBuffer_analysis.md](./frontend_IBuffer_analysis.md)

# frontend_in_out_seq_diagram.md

Block: Frontend

Module: N/A (total I/O flow)

Source: Frontend.scala, ifu/Ifu.scala, ftq/Ftq.scala, bpu/Bpu.scala, ibuffer/IBuffer.scala

Protocols: Decoupled / Valid

Key Params: FtqSize=64, FetchBlockInstNum=16, DecodeWidth=6, IBuffer.Size=48, NumWriteBank=4, NumReadBank=8

Last updated: 2026-02-18

→ See [frontend_block_diagram_overview.md](./frontend_block_diagram_overview.md)

## 1. I/O Summary

### 1.1 input (FrontendInlinedImp.io)

| Port | Protocol | direction | Description |
| --- | --- | --- | --- |
| `io.backend.toFtq.redirect` | Valid | Backend → FTQ | Misprediction/MemVio redirect |
| `io.backend.toFtq.commit` | Valid FtqPtr | Backend → FTQ | ROB commit signal (for FTQ commit ptr update, MMIO lastCommit check) |
| `io.backend.canAccept` | Bool | Backend → IBuffer | Decode is acceptable |
| `io.backend.wfi.wfiReq` | Bool | Backend → ICache/Uncache | WFI request |
| `io.reset_vector` | UInt | SoC → BPU | Start PC on reset |
| `io.sfence` | Bundle | CSR → iTLB | TLB flush |
| `io.tlbCsr` | Bundle | CSR → iTLB/BPU | SATP/STATUS etc. |
| `io.csrCtrl` | Bundle | CSR → Each module | bp_ctrl, frontend_trigger, pf_ctrl, etc. |
| `io.ptw` | TlbPtwIO | PTW → iTLB | Page table walk response |
| `io.softPrefetch` | Valid Vec | Backend → ICache | Software prefetch request |
| `io.debugTopDown.robHeadVaddr` | Valid | Backend → iTLB | For TopDown analysis |

### 1.2 output

| Port | Protocol | direction | Description |
| --- | --- | --- | --- |
| `io.backend.cfVec` | DecoupledIO Vec | Frontend → Decode | DecodeWidth CtrlFlow command |
| `io.backend.fromFtq` | Bundle | FTQ → Backend | PC mem write, newest entry, etc. |
| `io.backend.fromIfu` | Bundle | IFU → Backend | gpaddr mem write |
| `io.backend.wfi.wfiSafe` | Bool | Frontend → Backend | Is it safe to enter WFI |
| `io.error` | L1BusErrorUnitInfo | ICache → Backend | ECC/Bus Error |
| `io.frontendInfo.ibufFull` | Bool | IBuffer → Backend | IBuffer full status |
| `io.frontendInfo.bpuInfo` | Bundle | FTQ → Backend | BPU hit/miss counter |
| `io.resetInFrontend` | Bool | Frontend → SoC | reset signal propagation |

## 2. Sequence Diagram — Group A: Normal Fetch Flow (ICache Hit)

> BPU prediction → FTQ enqueue → IFU fetch → ICache hit → IBuffer enqueue → Decode delivery

sequenceDiagram

participant BPU as Predictor (BPU)

participant FTQ as Ftq

participant IFU as NewIFU

participant IC  as ICache

participant IBuf as IBuffer

participant BE  as Backend/Decode

Note over BPU,FTQ: Cycle 0 — BPU S0

BPU->>BPU: [C0] S0: PC mux, history gen

Note over BPU,FTQ: Cycle 1 — BPU S1

BPU->>BPU: [C1] S1: UBTB+ABTB+UTAGE+MicroRAS lookup → 1st prediction

BPU->>FTQ: [C1] io.toFtq.prediction.valid (s1 pred, startPc, ftqIdx)

FTQ-->>BPU: io.toFtq.prediction.ready

Note over FTQ,IFU: Cycle 1 — FTQ enqueue, IFU F0

FTQ->>IFU: [C1] toIfu.req.valid (FetchRequestBundle: startAddr, nextlineStart, ftqIdx)

FTQ->>IC:  [C1] toICache.req.valid (FtqToICacheRequestBundle)

IFU-->>FTQ: fromFtq.req.ready (f1_ready && icacheReady)

Note over IFU,IC: Cycle 2 — IFU F1, ICache S0

IFU->>IFU: [C2] F1: PC calculation (CatPC adder), f1_valid=1

Note over IFU,IC: Cycle 3 — IFU F2, ICache hit resp

IC->>IFU: [C3] icache.fetch.resp.valid (data, exception, paddr)

IFU->>IFU: [C3] F2: predecode, exception generation, instr range, f2_valid=1

Note over IFU,IBuf: Cycle 4 — IFU F3 → IBuffer

IFU->>IFU: [C4] F3: RVC expand, PredChecker, f3_valid=1

IFU->>IBuf: [C4] toIbuffer.valid (FetchToIBuffer: instrs, pd, pc, ftqPtr)

IBuf-->>IFU: toIbuffer.ready (allowEnq)

IFU->>FTQ:  [C4] pdWb.valid (PredecodeWritebackBundle)

Note over IBuf,BE: Cycle 5 — IBuffer → Decode

IBuf->>BE: [C5] cfVec(0..5).valid (CtrlFlow × DecodeWidth)

BE-->>IBuf: cfVec(i).ready (decodeCanAccept)

## 3. Sequence Diagram — Group B: BPU S3 Override (s3_override → FTQ entry update + IFU flush)

> Actual architecture (code basis: `bpu/Bpu.scala`, `ftq/Ftq.scala`): > > The BPU → FTQ path has two independent flows: > > | path | Conditions | FTQ operation | latency | > |------|------|-----------|---------| > | S1 new enq | s1_valid && s2_ready | entryQueue[bpuPtr] new record, bpuPtr++ | ~1 cycle | > | S3 override | s3_override (S3≠S1) | Overwrite entryQueue[s3FtqPtr], bpuPtr := s3FtqPtr+1 | ~3 cycle | > > - S1 (uBTB/ABTB) prediction always goes into FTQ first. BPU latency is not fixed to 3-cycle. > - S3 override only occurs when the S3 result is different from the S1 result (s3_override := s3_valid && !(s3_prediction === s3_s1Prediction)), and post-corrects the entry previously enqled by S1. > - S2 only processes the BPU internal flush (s2_flush := s3_flush || s3_override) and does not directly transmit it to the FTQ. > (Reference: Ftq.scala TODO — "wait for Ifu/ICache to remove bpu s2 flush") > - prediction.ready independent: bpuS3Redirect = prediction.valid && s3Override — Override is executed even if FTQ is full.

### FTQ index tracking (s3FtqPtr)

| steps | Code Basis | Description |
| --- | --- | --- |
| S1 fire city | `s2_ftqPtr = RegEnable(io.fromFtq.bpuPtr, s1_fire)` | Latch bpuPtr at S1 fire point to s2 |
| S2 fire city | `s3_ftqPtr = RegEnable(s2_ftqPtr, s2_fire)` | pass s2_ftqPtr to s3 |
| S3 viewpoint | `io.toFtq.s3FtqPtr := s3_ftqPtr` | Pass override target entry index to FTQ |
| run override | `predictionPtr = s3Override ? s3FtqPtr : bpuPtr(0)` | update entryQueue(s3FtqPtr) |
| rollback bpuPtr | `when(s3Override) { bpuPtr := s3FtqPtr + 1 }` | Move the FTQ enqueue pointer to the next override entry |

### FTQ Override Operation Summary

| Conditions | Action | Code Basis |
| --- | --- | --- |
| prediction.fire(s1, not s3Override) | entryQueue(bpuPtr) new record, bpuPtr+1 | `prediction.fire` |
| s3Override=1 | entryQueue(s3FtqPtr) update(overwrite), bpuPtr := s3FtqPtr+1 | `bpuS3Redirect`, `predictionPtr` |
| ifuPtr >= s3FtqPtr | ifuPtr := s3FtqPtr (rollback) | `when(ifuPtr >= ftqIdx)` |
| pfPtr >= s3FtqPtr | pfPtr := s3FtqPtr (rollback) | `when(pfPtr >= ftqIdx)` |
| IFU stage idx >= s3FtqPtr | shouldFlushByStage3 = true → flush | `!isAfter(s3FtqPtr, idxToFlush)` |

### nextStartVAddr Bypass case

| bpuPtr location | nextStartVAddr source | Code Basis |
| --- | --- | --- |
| bpuPtr(0) == ifuPtr(0) | prediction.bits.target (bypass-1) | `bpuPtr(0) === ifuPtr(0)` |
| bpuPtr(0) == ifuPtr(1) | prediction.bits.startPc (bypass-2, = tgt_C) | `bpuPtr(0) === ifuPtr(1)` |
| Other | entryQueue(ifuPtr(1)).startPc (SRAM) | default MuxCase |

### B-1: Standard S3 Override (ifuPtr > s3FtqPtr — IFU is already ahead)

> enq idx=X in S1 → IFU is processing more than idx=X → s3_override=1 in S3 (tgt_C ≠ tgt_A) > → update entryQueue(X) + rollback ifuPtr + IFU flush + restart based on tgt_C

sequenceDiagram

participant BPU as Predictor (BPU)

participant FTQ as Ftq

participant IFU as Ifu

Note over BPU,FTQ: Cycle 0 — S1 fire: PC_A → FTQ idx=X enq

BPU->>FTQ: [C0] prediction.valid=1, s3Override=0, startPc=PC_A, tgt=tgt_A

FTQ-->>BPU: [C0] prediction.ready=1

FTQ->>FTQ: [C0] entryQueue(X).startPc:=PC_A, bpuPtr:=X+1

FTQ->>IFU: [C0] toIfu.req (idx=X, startAddr=PC_A)

Note over BPU,IFU: Cycle 1 — S1 PC_A+blk → idx=X+1 enq, IFU idx=X processing

BPU->>FTQ: [C1] prediction.valid=1, s3Override=0, startPc=PC_A+blk, tgt=tgt_A2

FTQ->>FTQ: [C1] entryQueue(X+1).startPc:=PC_A+blk, bpuPtr:=X+2

FTQ->>IFU: [C1] toIfu.req (idx=X+1, startAddr=PC_A+blk)

Note over IFU: ifuPtr=X+1, bpuPtr=X+2

Note over BPU,IFU: Cycle 2 — S3 fire: s3_override=1, s3FtqPtr=X

BPU->>BPU: [C2] S3: s3_prediction(PC_A)=tgt_C ≠ s1_prediction(PC_A)=tgt_A → s3_override=1

BPU->>BPU: [C2] s2_flush:=1 (BPU internal S1+S2 in-flight kill), s0_startPc:=tgt_C

BPU->>FTQ: [C2] prediction.valid=1, s3Override=1, s3FtqPtr=X, startPc=PC_A, tgt=tgt_C

FTQ->>FTQ: [C2] bpuS3Redirect=1 → update entryQueue(X): tgt:=tgt_C

FTQ->>FTQ: [C2] bpuPtr := X+1 (discard entry X+1)

FTQ->>FTQ: [C2] ifuPtr=X+1 >= s3FtqPtr=X → ifuPtr := X (rollback)

FTQ->>FTQ: [C2] pfPtr >= X → pfPtr := X (rollback)

FTQ->>IFU: [C2] flushFromBpu.s3.valid=1, bits=X

IFU->>IFU: [C2] shouldFlushByStage3(idx>=X) → s0/s1_flush=1, invalidate all in-flight

Note over FTQ,IFU: Cycle 3 — IFU re-request (bypass-2 applied)

Note over FTQ: bpuPtr=X+1, ifuPtr=X → bpuPtr(0)==ifuPtr(1): bypass-2

FTQ->>IFU: [C3] toIfu.req (idx=X, startAddr=PC_A)

Note over FTQ: nextStartVAddr = prediction.bits.startPc = tgt_C (bypass-2, SRAM not used)

IFU-->>FTQ: [C3] toIfu.req.ready

### B-2: Early S3 Override (ifuPtr <= s3FtqPtr — IFU is still behind)

> IFU stall due to FTQ back-pressure, etc. → ifuPtr < s3FtqPtr > → no need for ifuPtr rollback, only update entryQueue(s3FtqPtr), no IFU flush range

sequenceDiagram

participant BPU as Predictor (BPU)

participant FTQ as Ftq

participant IFU as Ifu

Note over BPU,IFU: Cycle 0 — S1 fire: PC_A → idx=X enq, IFU stall (ifuPtr=X-1)

BPU->>FTQ: [C0] prediction.valid=1, s3Override=0, startPc=PC_A, tgt=tgt_A

FTQ-->>BPU: [C0] prediction.ready=1

FTQ->>FTQ: [C0] entryQueue(X).startPc:=PC_A, bpuPtr:=X+1

Note over IFU: IFU stall: ifuPtr=X-1 (FTQ→IFU request pending or not sent)

Note over BPU: Cycle 1 — S2 fire: BPU internal processing

BPU->>BPU: [C1] S2: Calculating prediction, preparing s3_override

Note over BPU,IFU: Cycle 2 — S3 fire: s3_override=1, ifuPtr < s3FtqPtr=X

BPU->>BPU: [C2] S3: s3_prediction(PC_A)=tgt_C ≠ s1_prediction=tgt_A → s3_override=1

BPU->>FTQ: [C2] prediction.valid=1, s3Override=1, s3FtqPtr=X, tgt=tgt_C

FTQ->>FTQ: [C2] bpuS3Redirect=1 → entryQueue(X): tgt:=tgt_C

FTQ->>FTQ: [C2] bpuPtr := X+1

Note over FTQ: ifuPtr=X-1 < s3FtqPtr=X → no ifuPtr rollback

FTQ->>IFU: [C2] flushFromBpu.s3.valid=1, bits=X

IFU->>IFU: [C2] shouldFlushByStage3(idx=X-1): !isAfter(X, X-1)=false → no flush

Note over IFU: Preserve in-flight fetch idx=X-1 (entry before s3FtqPtr=X)

Note over FTQ,IFU: Cycle 3 — IFU, idx=X normal request with updated tgt_C

FTQ->>IFU: [C3] toIfu.req (idx=X, startAddr=PC_A)

Note over FTQ: nextStartVAddr = entryQueue(X).startPc (SRAM, already updated with tgt_C)

IFU-->>FTQ: [C3] toIfu.req.ready

### B-3: Back-pressure S3 Override (prediction.ready=0 — override runs independently)

> FTQ full → prediction.ready=0 (new S1 enq not possible) > However, s3_override is executed regardless of prediction.ready with bpuS3Redirect path.

sequenceDiagram

participant BPU as Predictor (BPU)

participant FTQ as Ftq

participant IFU as Ifu

Note over BPU,FTQ: Cycle 0 — FTQ full: prediction.ready=0

BPU->>FTQ: [C0] prediction.valid=1, s3Override=0 (try new S1)

FTQ-->>BPU: [C0] prediction.ready=0 (FTQ full: bpuPtr - deqPtr >= FtqSize)

Note over BPU: s1_fire=0 (stall) — However, s3_override is ready for PC_A already latched to S3

Note over BPU,IFU: Cycle 1 — S3 override occurs (executes even when prediction.ready=0)

BPU->>BPU: [C1] S3: s3_prediction(PC_A)=tgt_C ≠ s1_prediction=tgt_A → s3_override=1

BPU->>FTQ: [C1] prediction.valid=1, s3Override=1, s3FtqPtr=X, tgt=tgt_C

Note over FTQ: bpuS3Redirect = prediction.valid && s3Override = 1 (ready independent)

FTQ->>FTQ: [C1] bpuS3Redirect=1 → entryQueue(X): tgt:=tgt_C (prediction.fire is irrelevant)

FTQ->>FTQ: [C1] bpuPtr := X+1

FTQ->>FTQ: [C1] ifuPtr >= X → ifuPtr := X (rollback)

FTQ->>IFU: [C1] flushFromBpu.s3.valid=1, bits=X → IFU flush

Note over BPU,FTQ: Cycle 2 — Recover prediction.ready=1 after FTQ deq

FTQ-->>BPU: [C2] prediction.ready=1 (Secure FTQ slot)

BPU->>FTQ: [C2] prediction.valid=1, s3Override=0, startPc=tgt_C (New S1)

FTQ->>IFU: [C2] toIfu.req (idx=X, startAddr=PC_A, nextStartVAddr=tgt_C)

## 4. Sequence Diagram — Group C: Backend Redirect (Misprediction Flush)

> Detect misprediction in ROB → Flush entire Frontend → Restart with correct PC

sequenceDiagram

participant BE  as Backend (ROB)

participant FTQ as Ftq

participant BPU as Predictor

participant IFU as NewIFU

participant IBuf as IBuffer

Note over BE,FTQ: Cycle N — Backend redirect

BE->>FTQ: [CN] toFtq.redirect.valid (BranchPredictionRedirect: target, ftqIdx, ftqOffset)

BE->>BPU: [CN] (via FTQ) redirect.valid → BPU in-flight invalidation

Note over FTQ,IBuf: Cycle N+1 — needFlush=RegNext(redirect.valid)

FTQ->>IFU: [CN+1] toIfu.redirect.valid → f0/f1/f2/f3_flush

FTQ->>IFU: [CN+1] icacheFlush → Invalidate ICache pipe

FTQ-->>BPU: [CN+1] toBpu.redirect.valid (BPU update + history restore)

IBuf->>IBuf: [CN+1] ibuffer.io.flush → entire IBuffer flush

Note over BPU: Cycle N+2 — BPU restart

BPU->>BPU: [CN+2] S0: Set redirect target to new PC

BPU->>FTQ: [CN+2] io.toFtq.prediction (new prediction)

Note over FTQ,IFU: Cycle N+3 — IFU restart

FTQ->>IFU: [CN+3] toIfu.req.valid (new FetchRequestBundle)

Note over BE: After IBuf flush, wait for new command to be supplied from IBuffer

## 5. Sequence Diagram — Group D: ICache Miss (Stall)

> ICache miss → IFU F2 stall → FTQ back-pressure

sequenceDiagram

participant FTQ as Ftq

participant IFU as NewIFU

participant IC  as ICache

participant MISS as ICache MissHandler

Note over IFU,IC: Cycle 2 — F2: ICache miss

IFU->>IFU: [C2] f2_valid=1, f2_icache_all_resp_wire=0

IFU->>IC:  icacheStop := !f3_ready (deassert when F3 stall)

IC->>MISS: [C2] miss req → L2 fetch

Note over IFU: Cycle 2~X — F2 stall

IFU->>IFU: [C2~X] f2_fire=0 (icacheRespAllValid=0)

IFU-->>FTQ: fromFtq.req.ready=0 (f1_ready=0 → FTQ stall)

Note over IC,IFU: Cycle X — L2 response, ICache miss resolution

MISS->>IC: [CX] refill cacheline

IC->>IFU: [CX] fetch.resp.valid=1 (f2_icache_all_resp_wire=1)

IFU->>IFU: [CX] icacheRespAllValid=1, f2_fire=1

IFU-->>FTQ: resume fromFtq.req.ready=1

Note over IFU: TopDown: icacheMissBubble → record topdown_stages(1)

## 6. Sequence Diagram — Group E: MMIO Fetch

> MMIO area command fetch → Via InstrUncache → Next MMIO fetch after ROB commit

sequenceDiagram

participant IFU as NewIFU (F3)

participant FTQ as Ftq

participant UC  as InstrUncache

participant BE  as Backend (ROB)

Note over IFU: F2: detect pmp_mmio

IFU->>IFU: [C] f2_pmp_mmio=1 → Enter MMIO mode at F3

Note over IFU,UC: F3: MMIO fetch request

IFU->>UC: [C] toUncache.valid (InsUncacheReq: addr)

UC-->>IFU: toUncache.ready

UC->>IFU: [C+L] fromUncache.valid (InsUncacheResp: data)

IFU-->>UC: fromUncache.ready

Note over IFU,FTQ: MMIO instr → IBuffer (1 each)

IFU->>FTQ: mmioCommitRead.valid=1 (MMIO FtqPtr lookup)

BE->>FTQ: toFtq.commit.valid (CtrlToFtqIO.commit — ROB commit ptr)

FTQ-->>IFU: mmioCommitRead.mmioLastCommit (whether commit ptr >= mmioPtr)

alt mmioLastCommit=1

IFU->>IFU: [C] mmio_redirect → flush to next MMIO PC

IFU->>FTQ: [C] pdWb (misOffset, target)

else

IFU->>IFU: [C] standby (stall)

end

## 7. Edge Cases

### 7.1 IBuffer full → fetch stall

allowEnq := io.in.bits.prevInstrCount < nextNumInvalid (nextNumInvalid = Size.U − nextNumValid) — Enqueue is allowed only when the number of next fetch instructions is less than the number of invalid entries in the next cycle.

io.full = !allowEnq → io.frontendInfo.ibufFull output → stall signal to backend

IFU toIbuffer.ready = false → F3 stall → F2 stall → FTQ back-pressure

### 7.2 FTQ full → BPU stall

ftqFullStall → BPU S3 s3_ready=false → s2/s1 also chain stall

s0_stall = !s3_ready → PC mux freeze in BPU S0

### 7.3 Cross-cacheline fetch (doubleLine)

f0_doubleLine = fromFtq.req.bits.crossCacheline — startAddr(blockOffBits-1) === 1

Added second cacheline request to ICache (FtqToICacheRequestBundle.readValid(1..4))

F2: f2_doubleLine && f2_icache_all_resp_wire = vaddr(0)===startAddr && vaddr(1)===nextlineStart

### 7.4 Cross-page RVI instruction exception

F2: isLastInLine(f2_pc(i)) && !f2_pd(i).isRVC && f2_doubleLine → f2_crossPage_exception_vec

If there is no exception on the first page, use the exception on the second page.

-IBufferExceptionType.isCrossPage() classification

### 7.5 Unnecessary flush (BTB Miss) classification

ControlBTBMissBubble = ControlRedirectBubble && !cfiUpdate.br_hit && !cfiUpdate.jr_hit

TAGEMissBubble = ControlRedirectBubble && cfiUpdate.br_hit && !cfiUpdate.sc_hit

SCMissBubble / ITTAGEMissBubble / RASMissBubble

IBuffer: Counter input for each bubble type (TopDown analysis)

→ See [frontend_block_diagram_overview.md](./frontend_block_diagram_overview.md) → See [frontend_Predictor_analysis.md](./frontend_Predictor_analysis.md) → See [frontend_NewIFU_analysis.md](./frontend_NewIFU_analysis.md) → See [frontend_Ftq_analysis.md](./frontend_Ftq_analysis.md) → See [frontend_IBuffer_analysis.md](./frontend_IBuffer_analysis.md)

# frontend_Predictor_analysis.md

Block: Frontend

Module: Predictor (BPU)

Source: BPU.scala, Composer.scala, FauFTB.scala, FTB.scala, Tage.scala, SC.scala, ITTAGE.scala, RAS.scala, Bim.scala

Protocols: Decoupled (bpu_to_ftq), Valid (redirect/update)

Key Params: numDup=4, HistoryLength, numBr, FtqSize, MaxMetaLength

Last updated: 2026-02-18

→ See [frontend_block_diagram_overview.md](./frontend_block_diagram_overview.md) → See [frontend_in_out_seq_diagram.md](./frontend_in_out_seq_diagram.md#group-b)

## 1. Module Summary

Role: 3-stage branch prediction pipeline. It consists of S1 (FauFTB), S2 (Main FTB+TAGE base), and S3 (TAGE+SC+ITTAGE+RAS). Each stage transmits predictions to FTQ, and when subsequent stages make more precise predictions, FTQ is overridden (redirected).

Position: Frontend.scala → Module(new Predictor) (inside FrontendInlinedImp)

Number of pipeline stages: 3 register stages (S1/S2/S3), S0 is combinational launch phase

## 2. Key Parameters

| Parameter | Source | Default | Impact |
| --- | --- | --- | --- |
| numDup | `HasBPUConst.numDup` | 4 | Number of PC/history replicas (for timing closure) |
| HistoryLength | `p(XSCoreParamsKey).HistoryLength` | 256 | Global branch history length (GHR size) |
| numBr | `p(XSCoreParamsKey).numBr` | 2 | Number of branch slots per FTB entry |
| numBrSlot | `numBr - 1` | 1 | Number of dedicated branch slots |
| totalSlot | `numBr` | 2 | total slots (brSlots + tailSlot) |
| MaxMetaLength | `HasBPUConst.MaxMetaLength` | 512 | Predictive metadata storage bit count |
| FtqSize | `p(XSCoreParamsKey).FtqSize` | 64 | FTQ size (based on BPU stall) |
| numBpStages | 3 | 3 | Number of pipeline stages |

## 3. Interfaces

| Port | Dir | Bitwidth | Protocol | Description |
| --- | --- | --- | --- | --- |
| `io.bpu_to_ftq.resp` | out | BpuToFtqBundle | Decoupled | Prediction result → FTQ (including s1/s2/s3) |
| `io.ftq_to_bpu.redirect` | in | BranchPredictionRedirect | Valid | FTQ→BPU redirect (misprediction) |
| `io.ftq_to_bpu.update` | in | BranchPredictionUpdate | Valid | FTQ→BPU training data |
| `io.ftq_to_bpu.enq_ptr` | in | FtqPtr | — | FTQ current enqueue pointer (stall judgment) |
| `io.ctrl` | in | BPUCtrl | — | Each predictor enable/disable |
| `io.reset_vector` | in | PAddrBits | — | Reset PC |

BPUCtrl field: ubtb_enable, btb_enable, bim_enable, tage_enable, sc_enable, ras_enable, loop_enable

## 4. Internal Pipeline / State

### 4.1 Stage composition

> Key: All predictors receive s0_startPc at the same time. (p.io.startPc := s0_startPc — Bpu.scala:192) > The reason the output comes out of S1/S2/S3 is because of the difference in lookup latency of each predictor, and each stage does not process different PCs.

S0 (combinational)

├─ s0_startPc: MuxCase

│    priority: redirect.target > s3_override→s3_prediction.target > s1_valid→s1_prediction.target > s0_startPcReg

├─ s0_startPcReg: RegEnable(s0_startPc, !s0_stall) — Maintain PC when stalling

├─ s0_stall: !(s1_valid || s3_override || redirect.valid)

├─ s0_fire: s1_ready && all_predictors_reset_done

└─ → Pass s0_startPc to all predictors (simultaneously, parallel)

S1 (REG: s1_valid, s1_startPc = RegEnable(s0_startPc, s0_fire))

├─ [1-cycle lookup result] ubtb → 1 candidate (always taken, index 0)

├─ [1-cycle lookup result] abtb → maximum 8 candidates (index 1..8)

│ └─ Concat the two outputs: s1_btbPrediction = VecInit(ubtb.io.prediction) ++ abtb.io.prediction (9 pools in total)

│ ※ Two predictors do not compete — 9 candidates are combined into one Vec and sorted by position

├─ [1-cycle lookup result] utage → conditional branch direction correction (s1_utageHitMask: override on the same cfiPosition candidate)

├─ [1-cycle lookup result] uras → ret target provided (s1_isRet && uras.specOut.isCanUse → s1_prediction.target)

├─ [always on]          fallThrough → sequential fallthrough prediction (fallback)

├─ s1_takenMask: Calculate whether taken for each of the 9 (conditional → utage or BTB direction, direct/indirect → always taken)

├─ s1_firstTakenBranchOH: Select at least 1 cfiPosition among candidates with takenMask=1 using CompareMatrix

│ └─ s1_firstTakenBranchOH(0)=1 → uBTB wins / (0)=0 → 1 of ABTB candidates wins

├─ s1_prediction: Selected single candidate (or fallThrough) → .target is next cycle S0 PC

├─ s1_fire = s1_valid && s2_ready && prediction.ready

└─ if s1_fire → prediction.valid=1, s3Override=0 → FTQ (transfer s1_startPc, s1_prediction)

S2 (REG: s2_valid, s2_startPc = RegEnable(s1_startPc, s1_fire))  — same PC as S1

├─ [2-cycle lookup result] mbtb → s2_mbtbResult (main BTB, more accurate branch information)

├─ [2-cycle lookup result] tage → tage.io.prediction (TAGE table direction prediction)

├─ [2-cycle lookup result] sc → sc.io.scTakenMask (Statistical Corrector correction)

├─ s2_takenMask: Apply tage/sc direction results to mbtb entries

├─ s2_fire = s2_valid && s3_ready

├─ s2_flush = s3_flush || s3_override ← BPU internal flush (not passed to FTQ)

└─ [FTQ not delivered] S2 results are only latched to S3, not sent directly to FTQ

S3 (REG: s3_valid, s3_startPc = RegEnable(s2_startPc, s2_fire))  — same PC as S1/S2

├─ s3_mbtbResult = RegEnable(s2_mbtbResult, s2_fire) — S2 result latch

├─ s3_takenMask  = RegEnable(s2_takenMask, s2_fire)

├─ [3-cycle lookup result] ittage → ittage.io.prediction.target (indirect branch target)

├─ [3-cycle lookup result] ras → ras.io.topRetAddr (full RAS ret address)

├─ s3_prediction.target: MuxCase(fallThrough, [(taken&&useRas)→ras, (taken&&useIttage)→ittage, taken→mbtb])

├─ s3_s1Prediction = RegEnable(RegEnable(s1_prediction, s1_fire), s2_fire) — S1 prediction propagation

├─ s3_override = s3_valid && !(s3_prediction === s3_s1Prediction)

├─ s3_fire = s3_valid (always fire — result always consumed)

└─ if s3_override → prediction.valid=1, s3Override=1 → FTQ (transfer s3_startPc, s3_prediction)

### 4.2 History Register Management

phr (Phr module): Path History Register — Folded versions such as s0_foldedPhr, s1_foldedPhr provided

commonHR (CommonHR module): Global History Register — provided by s0_commonHR

commonHR update during S3 fire: startPc, target, taken, firstTakenBranch, branch position

### 4.3 FTQ forwarding structure (BpuToFtqIO)

prediction: DecoupledIO[BpuPrediction]

├─ startPc: PrunedAddr — s1_startPc or s3_startPc (when s3Override)

├─ target: PrunedAddr — predicted next PC

├─ taken:   Bool

├─ cfiPosition: UInt

├─ attribute: CfiAttribute   — isDirect/isIndirect/isConditional/isReturn

├─ s3Override: Bool — If true, FTQ updates existing entry (override), if false, new enq

└─ (via fromStage())

s3FtqPtr: FtqPtr — override target entry index (RegEnable level 2)

meta: Valid[BpuMeta] — s3_valid when: redirectMeta / resolveMeta / commitMeta

## 5. Functionality

### 5.1 PC Creation (S0)

s0_startPc MuxCase priority (lowest to highest):

redirect.valid → redirect.bits.target (backend mispred recovery)

s3_override → s3_prediction.target (S3 override)

s1_valid → s1_prediction.target (fetch next block with S1 prediction target)

else → s0_startPcReg (stall: maintain previous PC)

s0_stall = !(s1_valid || s3_override || redirect.valid) → When there is no valid PC source

### 5.2 S1 prediction assembly (1-cycle latency predictors)

Candidate Pooling (Bpu.scala:266):

s1_btbPrediction = VecInit(ubtb.io.prediction) ++ abtb.io.prediction

// ↑ Up to 1 uBTB (index 0) ↑ Up to 8 ABTB (index 1..8)

// → Concat a total of up to 9 candidates into one Vec

// uBTB: tag = fetch block start PC[22:1] → fully-assoc comparison → maximum 1 hit

// (32 entries × 1 entry/fetch block → maximum 1 entry for the same fetch block → max 1 hit)

// ABTB: tag = fetch block start PC[24:1] → Multiple branches within the same fetch block are stored in each way

// → Multiple ways can hit simultaneously → Up to 8 Valid[Prediction] outputs

// Combine the outputs of two predictors into one candidate pool and determine a single prediction by comparing positions.

takenMask calculation (Bpu.scala:270-278):

s1_utageHitMask[i]: If cfiPosition of utage matches candidate i → use utage direction

s1_takenMask[i] = pred[i].valid && (isDirect || isIndirect || (isConditional && Mux(utageHit, utageTaken, pred.taken)))

Select single prediction (Bpu.scala:299-304):

s1_compareMatrix = CompareMatrix(s1_btbPrediction[*].cfiPosition) // Pairwise comparison of 9 positions

s1_firstTakenBranchOH = compareMatrix.getLeastElementOH(s1_takenMask) // Minimum position among taken → 1-hot

s1_firstTakenBranch = Mux1H(s1_firstTakenBranchOH, s1_btbPrediction) // 1 selected candidate

// Determine the winner (debug signal, Bpu.scala:311-314):

// s1_firstTakenBranchOH(0) = 1 → uBTB candidate wins

// s1_firstTakenBranchOH(0) = 0 → 1 of the ABTB candidates wins (smaller position)

// If the position is the same, index 0 (uBTB) takes precedence (OHToUInt characteristic)

s1_prediction = Mux(s1_taken, s1_firstTakenBranch.bits, fallThrough.io.prediction)

Upon ret branch: s1_prediction.target := uras.io.specOut.retTarget (fast RAS)

s1_prediction.target → next cycle s0_startPc (Bpu.scala:439: s1_valid → s1_prediction.target)

### 5.3 S2 prediction assembly (2-cycle latency predictors)

s2_mbtbResult = mbtb.io.result — main BTB result (lookup to S0 PC, result after 2 cycles)

s2_condTakenMask: Apply tage/sc direction to mbtb conditional branch

sc.io.scUsed → If SC is used, sc.io.scTakenMask takes precedence

tage.useProvider → use provider table prediction

else → default direction of mbtb

s2_takenMask = s2_condTakenMask || s2_jumpMask (direct/indirect is always taken)

Not delivered to FTQ — only delivered to S3

### 5.4 S3 prediction assembly and override (3-cycle latency predictors)

s3_mbtbResult = RegEnable(s2_mbtbResult, s2_fire) — S2 mbtb result latch

s3_useIttage = s3_firstTakenBranch.bits.attribute.needIttage && ittage.io.prediction.hit

s3_useRas = s3_firstTakenBranch.bits.attribute.isReturn

s3_prediction.target priority: ras.topRetAddr > ittage.prediction.target > mbtb.target > fallThrough

s3_s1Prediction = RegEnable(RegEnable(s1_prediction, s1_fire), s2_fire) — S1 prediction propagation from the same PC

s3_override = s3_valid && !(s3_prediction === s3_s1Prediction) — when the two predictions are different

### 5.5 FTQ delivery path

// Bpu.scala:415-421

io.toFtq.prediction.valid := s1_valid && s2_ready || s3_override

when(s3_override) {

io.toFtq.prediction.bits.fromStage(s3_startPc, s3_prediction) // override path

}.otherwise {

io.toFtq.prediction.bits.fromStage(s1_startPc, s1_prediction) // Normal path

}

io.toFtq.prediction.bits.s3Override := s3_override

Normal path: s1_fire → s1_prediction to new enq to FTQ (s3Override=0)

override path: s3_override → Update existing entry (s3FtqPtr) with s3_prediction (s3Override=1), prediction.ready independent

## 6. Flow / Backpressure Control

| Conditions | Action | Code Basis |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| FTQ full (`prediction.ready=0`) | `s1_fire=0` → S1 stall, S0 stall chain | `s1_fire := s1_valid && s2_ready && prediction.ready` |  |  |  |  |
| s3_override occurs | `s2_flush=1`, `s1_flush=1` → BPU internal S1+S2 in-flight kill (following PCs) | `s2_flush := s3_flush |  | s3_override` |  |  |
| redirect.valid | `s3_flush=1` → s3_valid clear, s2_flush, s1_flush chain | `s3_flush := redirect.valid` |  |  |  |  |
| s0_stall | Keep `s0_startPcReg` (freeze PC), freeze predictors input | `s0_stall := !(s1_valid \ | \ | s3_override \ | \ | redirect.valid)` |
| s3_override + FTQ full | s3_override updates FTQ regardless of prediction.ready with `bpuS3Redirect` | `Ftq.scala: bpuS3Redirect = prediction.valid && s3Override` |  |  |  |  |

## 7. Error / Exception Handling

BPU itself does not generate exceptions.

Misprediction detection: FTQ compares ROB commit results and sends toBpu.redirect.valid

BranchPredictionUpdate.mispred_mask[numBr+1]: Indicates which slot is incorrect

false_hit: It was an FTB hit, but it was actually a different entry (ghost entry, etc.)

rollback speculatively updated GHR when redirect is received: restore ghistPtrGen priority

## 8. Timing Hints

| Critical Path | Description |
| --- | --- |
| S0 PC mux | `s0_startPc` MuxCase: redirect/s3_override/s1_valid/hold Level 4 Priority — Timing Critical |
| S1 ubtb/abtb lookup | ubtb: fully-associative tag comparison; abtb: ahead fetch + tag comparison (parallel) |
| S1 s1_prediction assembly | CompareMatrix(position): Select minimum cfiPosition — wide comparator |
| S2 mbtb SRAM | main BTB SRAM read latency + tag comparison |
| S2 tage/sc | Simultaneous lookup of multiple TAGE tables (different history lengths), SC application |
| S3 ittage/ras | indirect target table lookup (ittage), RAS top read (ras) |
| phr folded history | folded path history Multi-stage XOR tree — compLen bits all |
| s3_override decision | `s3_prediction === s3_s1Prediction`: Compare multiple fields across — wide equality |

## 9. Pseudocode

> Caution: All three predictor groups receive the same s0_startPc input. > The result in S1/S2/S3 is due to the lookup latency difference of each predictor, > S2 is a more accurate result for the same s0_startPc** rather than the next PC in S1.

=== Cycle T: S0 ===

s0_startPc = MuxCase(s0_startPcReg, [

redirect.valid  → redirect.bits.target,

s3_override     → s3_prediction.target,

s1_valid        → s1_prediction.target

])

s0_stall = !(s1_valid || s3_override || redirect.valid)

if not s0_stall:

s0_startPcReg := s0_startPc

//Simultaneous delivery of s0_startPc to all predictors (start lookup)

for p in [ubtb, abtb, utage, uras, fallThrough, mbtb, tage, sc, ittage, ras]:

p.io.startPc := s0_startPc

s0_fire = s1_ready && all_predictors_reset_done

=== Cycle T+1: S1 === (Receive 1-cycle latency result for s0_startPc)

if s0_fire:

s1_startPc := RegEnable(s0_startPc) // = s0_startPc of S0 above

s1_valid   := true

// 1-cycle lookup result (based on s0_startPc):

// uBTB (1) + ABTB (maximum 8) → concat with a total of 9 candidate pools (not competition)

s1_btbPrediction = VecInit(ubtb.io.prediction) ++ abtb.io.prediction  // index 0=uBTB, 1..8=ABTB

s1_utageHitMask  = [utage.prediction.cfiPosition == pred.cfiPosition for pred in btbPred]

s1_takenMask = [pred.valid && (direct || indirect || (conditional && Mux(utageHit, utageTaken, pred.taken)))]

// CompareMatrix: Select minimum cfiPosition among 9 candidates with takenMask=1 → single s1_prediction

s1_firstTakenBranchOH = CompareMatrix(positions).getLeastElementOH(takenMask)

s1_prediction = Mux(s1_taken, firstTakenBranch.bits, fallThrough.io.prediction)

// → s1_prediction.target determines s0_startPc for the next cycle (s1_valid path)

if s1_prediction.attribute.isReturn && uras.specOut.isCanUse:

s1_prediction.target := uras.io.specOut.retTarget

s1_fire = s1_valid && s2_ready && prediction.ready

if s1_fire:

// Forward to FTQ (new enq, s3Override=0)

prediction.valid = 1

prediction.bits  = fromStage(s1_startPc, s1_prediction)

prediction.bits.s3Override = 0

// s0_startPc next cycle: use s1_prediction.target as next block PC

if s1_flush (= s2_flush):

s1_valid := false

=== Cycle T+2: S2 === (Receive 2-cycle latency results for s0_startPc)

if s1_fire:

s2_startPc := RegEnable(s1_startPc) // = still original s0_startPc

s2_valid   := true

// 2-cycle lookup result (based on s0_startPc):

s2_mbtbResult = mbtb.io.result // main BTB (more accurate than ubtb/abtb)

s2_condTakenMask = Apply tage/sc direction to mbtb entries:

Mux(sc.scUsed → sc.scTaken, tage.useProvider → tage.providerPred, else → mbtb.taken)

s2_takenMask = s2_condTakenMask || jumpMask

// No direct delivery to FTQ — only to S3

s2_fire = s2_valid && s3_ready

if s2_flush (= s3_flush || s3_override):

s2_valid := false

=== Cycle T+3: S3 === (Receive 3-cycle latency results for s0_startPc)

if s2_fire:

s3_startPc := RegEnable(s2_startPc) // = still original s0_startPc

s3_valid   := true

// S2 result latch (pipeline forward to S3):

s3_mbtbResult        = RegEnable(s2_mbtbResult, s2_fire)

s3_takenMask         = RegEnable(s2_takenMask, s2_fire)

s3_firstTakenBranch  = Mux1H(firstTakenBranchOH, s3_mbtbResult)

s3_s1Prediction = RegEnable(RegEnable(s1_prediction, s1_fire), s2_fire) // S1 prediction of the same PC

// 3-cycle lookup result (based on s0_startPc):

s3_useRas    = s3_firstTakenBranch.attribute.isReturn

s3_useIttage = s3_firstTakenBranch.attribute.needIttage && ittage.prediction.hit

s3_prediction.target = MuxCase(fallThrough.target, [

(taken && useRas)    → ras.io.topRetAddr,

(taken && useIttage) → ittage.io.prediction.target,

taken                → s3_firstTakenBranch.bits.target

])

// Compare with s1 prediction (for same s0_startPc)

s3_override = s3_valid && !(s3_prediction === s3_s1Prediction)

if s3_override:

// Forward to FTQ (override, s3Override=1) — prediction.ready is irrelevant

prediction.valid = 1

prediction.bits  = fromStage(s3_startPc, s3_prediction)

prediction.bits.s3Override = 1

io.toFtq.s3FtqPtr = s3_ftqPtr // override target entry index

// Inside BPU: Clear in-flight for the following PCs

s2_flush := 1 // s1_flush also = s2_flush

s0_startPc Next cycle: s3_prediction.target (second-rank MUX)

// s3_fire = s3_valid (always)

if s3_valid:

commonHR.update(s3_startPc, s3_prediction)

ras.specIn := {s3_startPc, s3_prediction}

s3_fire = s3_valid // always fire

=== on redirect (backend mispred) ===

s3_flush := redirect.valid → clear s3/s2/s1_valid

phr rollback using redirect.bits meta

ras rollback using redirect.bits meta

s0_startPc priority: redirect.bits.target

### 9.1 Conditional Branch Prediction (mBTB + TAGE + SC)

=== [Case 1: Conditional Branch] mBTB + TAGE + SC ===

// Core: only determines direction(taken/not-taken). The target is fixed to mBTB.

// --- S0: SRAM read requests (simultaneous sending) ---

mbtb.readReq(alignBankIdx=PC[5], internalBankIdx=PC[7:6], setIdx=PC[15:8])

for t in tage.tables[0..7]:

t.readReq(setIdx = fold(PC XOR pathHist XOR globalHist, t.histLen),

tag    = fold(PC XOR globalHist, TagWidth))

for t in sc.tables:  // path-based + global-based

t.readReq(setIdx = fold(pathHist XOR PC, t.histLen))

// --- S1: SRAM responses arrive ---

mbtb_s1_rawEntries = mbtb.readResp() // tag compare in S2

tage_s1_rawResps   = tage.tables.readResp()

sc_s1_rawResps     = sc.tables.readResp()

// --- S2: tag compare + direction decision ---

// [mBTB] tag compare

for each way i:

mbtb_hit[i]   = entry[i].valid && entry[i].tag == PC[31:16]

mbtb_taken[i] = mbtb.counterSram[i].isPositive  // 2-bit saturating counter

// [TAGE] Select provider (hit table with longest history)

for each way i: // = based on mBTB result slot

hitTableMask    = [tage.tables[j].tag == computedTag[j] for j in 0..7]

providerTableOH = getLongestHistTableOH(hitTableMask)

provider        = tage.tables[providerTableOH]

hasAlt          = any hit besides provider

alt             = tage.tables[altTableOH]        // second longest hit

useProvider  = hasProvider && !(useAltOnNa && provider.takenCtr.isWeak)

providerPred = provider.takenCtr.isPositive      // MSB of 2-bit ctr

altPred      = alt.takenCtr.isPositive

// [SC] adaptive-threshold confidence check

for each way i:

percsum[j] = sc.tables[j].ctr * 2 + 1           // getPercsum (signed partial sum)

scSum      = sum(percsum for all SC tables)

// Adjust threshold according to TAGE provider confidence

tageConfHigh = provider.takenCtr.isSaturate // strong confidence (both ends)

tageConfMid = provider.takenCtr.isMid // middle

// tageConfLow  = otherwise

adaptiveThres = scThreshold >> (1 if confHigh else 2 if confMid else 3)

scUsed[i]  = mbtb_hit[i] && hasProvider && aboveThreshold(|scSum|, adaptiveThres)

scTaken[i] = scSum >= 0 // direction determined by sign

// [Final direction] Priority: SC > TAGE provider > TAGE alt > mBTB counter

for each way i:

condTaken[i] = mbtb_hit[i] && isConditional[i] &&

MuxCase(mbtb_taken[i], // Default: mBTB 2-bit counter

scUsed[i] → scTaken[i], // SC active: scSum sign used

useProvider → providerPred, // TAGE provider hit

hasAlt      → altPred       // TAGE alt hit

)

// --- S3: first taken selection + target + override ---

takenMask[i]     = condTaken[i] || jumpTaken[i]   // jumpTaken = isDirect||isIndirect

firstTakenOH     = CompareMatrix(positions).getLeastElementOH(takenMask)

firstTakenBranch = Mux1H(firstTakenOH, s3_mbtbResult)

s3_prediction.taken       = takenMask.reduce(||)

s3_prediction.cfiPosition = firstTakenBranch.bits.cfiPosition

s3_prediction.target      = reconstruct(PC, firstTakenBranch.bits.targetLowerBits,

firstTakenBranch.bits.targetCarry)

// conditional branch target only in mBTB — TAGE/SC only corrects direction

s3_override = s3_valid && (s3_prediction != s3_s1Prediction)

### 9.2 Indirect Jump prediction (mBTB + ITTage, non-return)

=== [Case 2: Indirect Jump] mBTB + ITTage (isReturn=false) ===

// Key point: direction is always taken. Target accuracy is the only issue.

// --- S0: SRAM read requests ---

mbtb.readReq(alignBankIdx=PC[5], internalBankIdx=PC[7:6], setIdx=PC[15:8])

// ITTage is not read from S0 (power opt: wait for s1_isIndirect)

// ※ In theory, S0 speculative read → prediction can be completed in S2, but not currently adopted.

// --- S1: mBTB SRAM resp + ITTage read req ---

mbtb_s1_rawEntries = mbtb.readResp()

s1_isIndirect = firstTakenBranch.attribute.needIttage // abtb/ubtb s1 result based

if s1_isIndirect:

for t in ittage.tables:

t.readReq(setIdx = fold(hist XOR PC, t.histLen),

tag    = fold(hist XOR PC, TagWidth))

// --- S2: mBTB tag compare + ITTage SRAM resp + provider selection ---

// [mBTB] tag compare

for each way i:

mbtb_hit[i]    = entry[i].valid && entry[i].tag == PC[31:16]

mbtb_target[i] = reconstruct(PC, entry[i].targetLowerBits, entry[i].targetCarry)

jumpTaken[i] = mbtb_hit[i] && isIndirect // always taken (no direction prediction required)

// Select [ITTage] provider (hit table with longest history)

ittage_hitMask    = [ittage.tables[j].tag == computedTag[j] for j in tables]

ittage_providerOH = getLongestHistTableOH(ittage_hitMask)

ittage_provided   = any(ittage_hitMask)

ittage_target = ittage.tables[ittage_providerOH].target // Save entire target

// s2_ittageTarget latch → output from S3 to io.prediction.target

// --- S3: select target + override ---

firstTakenBranch = Mux1H(firstTakenOH, s3_mbtbResult)

// Priority: RAS(isReturn) > ITTage(needIttage && hit) > mBTB target

s3_useRas    = firstTakenBranch.bits.attribute.isReturn

s3_useIttage = firstTakenBranch.bits.attribute.needIttage && ittage.prediction.hit

s3_prediction.taken = true // indirect is always taken

s3_prediction.target = MuxCase(firstTakenBranch.bits.target,  // fallback: mBTB

(s3_taken && s3_useRas)    → ras.topRetAddr,               // return: RAS

(s3_taken && s3_useIttage) → ittage.prediction.target      // indirect: ITTage

)

// ※ mBTB entry exists even when isReturn (attribute=Return) → target is overridden by RAS

s3_override = s3_valid && (s3_prediction != s3_s1Prediction)

## 10. Notes / Assumptions

Same PC input: All predictors share s0_startPc — Bpu.scala:192 p.io.startPc := s0_startPc

S1 prediction = fast path: ubtb+abtb+utage+uras result. Since it is 1 cycle, FTQ enq and next block fetch start immediately.

S3 override condition: !(s3_prediction === s3_s1Prediction) — Occurs only when the S3 result is different from S1 for the same PC (target, taken, cfiPosition, attribute all compared)

S2 is not delivered directly to FTQ: S2 mbtb/tage/sc results are propagated to S3 stage and used to assemble s3_prediction. Different from previous architecture (FauFTB→FTB s2_redirect)

phr (Phr), commonHR (CommonHR): Replaces GHR/FoldedHistory in previous architecture. separate management of path history + global history

uras (MicroRas): Provides fast ret target in S1. specOut of full RAS (ras) is used in S3

fastTrain: Generate BpuFastTrain signal when s3_valid → Used for abtb fast training (Bpu.scala:183-188)

## 11. BTB/FTB hierarchy details

### 11.1 Structure Overview

This version (KunMingHu v3) has evolved from the existing uFTB → FTB two-tier structure to a uBTB → ABTB → MBTB three-tier BTB layer. Each BTB produces results at a different stage and is paired with a different direction predictor.

S1 (1-cycle): uBTB + ABTB ──→ uTAGE (direction correction)

S2 (2-cycle): MBTB ──────────→ TAGE(8-table) + SC (direction correction)

S3 (3-cycle): (no BTB) ───→ ITTAGE (indirect target) + RAS (return target)

BTB method classification:

Common BTB designs fall into two categories:

| method | tag criteria | Action |
| --- | --- | --- |
| **Block BTB** (fetch-block indexed) | **fetch block start PC** | 1 entry covers 1 fetch block → 1 lookup returns prediction of the first taken branch of the corresponding fetch block |
| **Individual BTB** (branch-indexed) | **Individual branch instruction PC** | 1 entry covers 1 branch → If there are N branches in one fetch block, N lookups are required |

uBTB / ABTB / MBTB in this implementation are all Block BTB:

| BTB | tag unit | Number of returns per lookup |
| --- | --- | --- |
| **uBTB** | 64B fetch block (PC[22:1]) | **Maximum 1** (32 entries fully-assoc, maximum 1 entry for the same fetch block) |
| **ABTB** | 64B fetch block (PC[24:1]) | **Up to 8** (8 ways sharing the same tag, storing multiple branches of one fetch block) |
| **MBTB** | 32B half-block (tag=PC[31:16]) | **Up to 8** (2 AlignBanks × 4 ways, storing multiple branches in each half-block) |

### 11.2 uBTB (Micro BTB) — S1 Stage

| Item | value |
| --- | --- |
| BTB method | **Block BTB** — tag = fetch block start PC[22:1], 1 entry per fetch block, 1 taken branch per entry |
| structure | **Fully Associative Cache** |
| Number of entries | **32** |
| Tag Width | **22 bits** |
| Target Width | 22 bits (2B aligned) |
| Useful Counter | 2 bits |
| Replacer | **PLRU** (least-useful first) |
| Use History | **None** |
| Predicted Latency | **1 cycle** |

AddrField bit structure (ubtb/Helpers.scala:25, ubtb/Parameters.scala):

PC bit : [ 0  ] [    22:1    ] [ VAddrBits-1:23 ]

field  :  inst       tag           unused

Off    (22 bits)

| field | PC bits | Remarks |
| --- | --- | --- |
| instOffset | PC[0:0] | 1 bit, always 0 (RVC 2B alignment) |
| tag | **PC[22:1]** | 22 bits, used for branch prediction |
| targetLower (extraField) | **PC[22:1]** | 22 bits (same location as tag, for storing destination address) |

Lookup method (MicroBtb.scala:72):

// Simultaneous parallel comparison of all 32 entries (fully associative)

s1_tag   = PC[22:1]

s1_hitOH = entries.map(e => e.valid && e.tag === s1_tag)

// When hit: always-taken predictor

prediction.taken       := s1_hit

prediction.cfiPosition := hitEntry.slot1.position

prediction.target      := reconstruct(startPc, hitEntry.slot1.target)

> Feature: No set index. Direct comparison of all tags. The upper PC bits (PC[VAddrBits-1:23]) are ignored, so aliasing is possible when the address space is wide.

Entry Contents (ubtb/Bundles.scala:32):

MicroBtbEntry {

tag: UInt(22) // PC[22:1], for tag comparison

usefulCnt: SatCtr(2-bit, signed) // useful: SaturateNegative = invalid

slot1 { // main predicted branch (taken candidate)

position: UInt(5) // Branch position in fetch block (instruction slot based on 32B-align)

attribute:      BranchAttribute  // branchType(2) + rasAction(2) = 4 bits

target: UInt(22) // partial target: branch target [22:1] on PC

isStaticTarget: Bool // Only the same target was observed (for aliasing detection)

}

slot2 { // TODO: 2-taken — Candidate for the second taken branch in the fetch block (currently always valid=false, unused)

position:  UInt(5)

attribute: BranchAttribute

target:    UInt(22)

valid: Bool // Whether slot2 is valid (currently always false)

taken: Bool // slot2 predicted direction (currently not referenced)

}

}

Common Type Definition (bpu/Bundles.scala):

BranchAttribute (4-bit) {

branchType: 2-bit { None=0, Conditional=1, Direct=2, Indirect=3 }

rasAction:  2-bit { None=0, Pop=1(ret), Push=2(call), PopAndPush=3 }

// needIttage = isIndirect && !hasPop

}

TargetCarry (2-bit) { // For partial target boundary correction

Fit=0 : target upper == startPc upper (no carry)

Overflow=1: Target moves to upper area → upper + 1

Underflow=2: target is lower area → upper - 1

// uBTB: EnableTargetFix=false → TargetCarry not saved, startPc upper used as is

// MBTB: always save

}

CfiPosition (5-bit): 32B-align standard instruction slot index (0..31 / 2B = maximum 16 insts per 32B)

Next PC Decision (MicroBtb.scala:79):

if (s1_hit):

nextPc.taken       = true           // always-taken

nextPc.cfiPosition = slot1.position

nextPc.attribute   = slot1.attribute

// partial target → full VAddr reconstruction

nextPc.target = Cat(

startPc[VAddrBits-1:23], // upper: EnableTargetFix=false, so startPc stays on top

slot1.target[21:0],          // 22-bit partial target (= branch target PC[22:1])

0.U(1) // LSB=0: 2B sort

)

else:

→ Use fallThrough prediction

2-taken feature status (TODO):

Design intent:

- Simultaneously store/predict slot1 + slot2 and two taken branches in one fetch block

- slot1 = main taken branch (always-taken), slot2 = second taken branch candidate (including direction bit)

- Currently, uBTB processes only 1 taken branch (slot1) — throughput is expected to improve when 2-taken is implemented

Current status:

- Bundles.scala:50-55: slot2 field defined (valid, taken, position, attribute, target)

- MicroBtb.scala:193: Fixed slot2.valid := false.B during training (always disabled)

- MicroBtb.scala:78-83: Prediction logic does not reference slot2 at all — only slot1 is used

- Refer to slot2 in the entire codebase: only 2 places: declaration + false.B initialization

Conclusion: Only data structure is reserved, prediction/learning logic is not implemented (MicroBtb.scala class declaration // TODO: 2-taken)

Pair direction predictor: uTAGE (S1 simultaneous result, direction override to s1_utageHitMask)

### 11.3 ABTB (Ahead BTB) — S1 Stage

#### 11.3.0 Why ABTB exists

Problem 1: Capacity limitations of uBTB

uBTB is a 32 entries fully-associative structure. In large programs, there are many cold misses and coverage is insufficient.

Problem 2: Fully-Associative is not scalable

As the number of entries increases, the comparator increases linearly → timing and area costs rapidly increase. A Set-Associative structure is required.

Problem 3: If you just use Set-Associative SRAM, S1 timing will not be correct

By simply introducing Set-Associative BTB, PC_B is committed mid-cycle after uBTB output reconfiguration, causing SRAM access to start late:

[If you use a regular Set-Assoc BTB without the ahead trick]

Cycle N:   BPU S0 = PC_A.

Cycle N+1: PC_B confirmed with uBTB output → middle of cycle

Normal BTB: setIndex(PC_B) → start in the middle of SRAM read cycle

Cycle N+2: BPU S1 = PC_B.

SRAM response arrives mid-cycle → tag comparison late → S1 timing violation!

→ Results can only be used in S2 (1 cycle loss)

[ABTB’s ahead trick]

Cycle N: BPU S0 = PC_A.  PC_A is already a stable value.

ABTB S0: Start reading SRAM with setIndex(PC_A) → Early cycle, secure full cycle

Cycle N+1: Arrives early in the SRAM response cycle. s1_entries stable.

s1_startPc = io.startPc = PC_B (already stable)

tag Comparison cycle starts early, results latch at end of cycle

Cycle N+2: BPU S1 = PC_B.  Combination output from s2_* registers → S1 timing met!

The key to the ahead trick: Advance SRAM access by 1 cycle, allowing even large Set-Associative SRAMs to produce results within S1 timing. Without the trick, ABTB is relegated to the S2 predictor.

Problem 4: uBTB does not support Multi-Branch

uBTB stores only one slot1. Since ABTB tag = fetch block PC, multiple branches of one fetch block are simultaneously stored in 8 ways → Richer branch coverage.

summation:

| Reason | Description |
| --- | --- |
| capacity shortage | uBTB 32 entries → insufficient coverage |
| Scalability | Fully-Assoc expansion not possible → Set-Assoc required |
| Timing | Set-Assoc SRAM does not allow S1 timing → solved with ahead trick |
| Multi-Branch | Simultaneous cache of multiple branches of one fetch block in 8 ways |

| Item | value |
| --- | --- |
| BTB method | **Block BTB (multiple branches)** — tag = fetch block start PC[24:1], multiple ways share the same tag, returns up to 8 branches simultaneously |
| structure | **Set-Associative, Banked** |
| Total Entries | **1,024** |
| Banks | **4** (read-write conflict resolution) |
| Ways | **8** per set |
| Sets per bank | **32** (= 1024 / 8 / 4) |
| Tag Width | **24 bits** |
| Target Lower Bits | 22 bits (2B aligned) |
| Taken Counter | 2-bit |
| Write Buffer | 4 entries |
| Replacer | PLRU |
| Use History | **None** |
| Predicted Latency | **Depends on standard**: 2 cycles based on BPU s0 input PC / 1 cycle based on BPU s1 current block |

> Standard summary: > - Based on BPU s0 input PC (PC_A): two-block-ahead character seen as PC_A -> PC_B -> PC_C > - Based on BPU s1 current block (PC_B): PC_B -> PC_C prediction (non-lookahead)

AddrField bit structure (abtb/Helpers.scala:24):

PC bit : [ 0  ] [ 2:1 ] [ 7:3  ] [ VAddrBits-1:8 ]

field  :  inst   bank    setIdx      unused

Off   (2 bit)  (5 bit)

extraField:

tag         = PC[24:1]   (start=instOffsetBits=1, width=24)

targetLower = PC[22:1]   (start=instOffsetBits=1, width=22)

| field | PC bits | Remarks |
| --- | --- | --- |
| instOffset | PC[0:0] | 1 bit |
| bankIdx | **PC[2:1]** | 2 bits → 4 banks selection |
| setIdx | **PC[7:3]** | 5 bits → 32 sets/bank selection |
| tag | **PC[24:1]** | 24 bits (including bankIdx+setIdx bits) |

[Core] ABTB's tag is fetch block PC (abtb/Bundles.scala:85):

AheadBtbEntry.tag = PC[24:1] ← fetch block start PC, not individual branch PC

> General set-associative BTB tag = individual branch instruction PC → maximum 1 way hit per lookup. > ABTB tag = fetch block start PC → multiple branches within the same fetch block are stored in each way. > Therefore, if you look up the same fetch block, multiple ways may hit at the same time (normal operation). > Ways are distinguished by different position (branch positions within the fetch block).

Lookup method (abtb/AheadBtb.scala:107-178):

> Note: ABTB uses different PCs for SRAM index and tag comparison (ahead mechanism).

[Key] Why are set-index(PC_A) and tag(PC_B) different PCs?

In general BTB, set-index and tag originate from the same PC. ABTB is different:

| role | Regular BTB | ABTB |
| --- | --- | --- |
| set-index | PC_B[7:3] → SRAM which row? | **PC_A[7:3]** → SRAM which row? (Read 1 cycle ahead) |
| tag | PC_B[24:1] → "Is this entry for PC_B?" | **PC_B[24:1]** → "Is this entry for PC_B?" |
| entry data | Branch information in PC_B | Branch information in PC_B |

set-index(PC_A): Pure physical address — "Which row of SRAM to read?" PC_A is used to consume SRAM 2-cycle latency 1 cycle ahead.

tag(PC_B): Logical identifier — "Which of the read entries is for the PC_B block?" For confirmation of affiliation.

What if tag is used as PC_A?

entry.tag === getTag(PC_A) → "Does this entry record a branch of the PC_A block?" search for

Such entries are never saved (because they are saved as tag = getTag(PC_B) during training)

The branch of the PC_A block has already been processed much earlier in the cycle — the information needed now is the branch of the PC_B block.

Meaning of ABTB entry: "When executing the PC_A → PC_B path, which branch (position) in the PC_B block jumps to (target)?"

setIndex(PC_A): SRAM physical location (index for ahead access, stored at the time of moving to the next block)

tag = getTag(PC_B): Logical identifier indicating that this entry belongs to the PC_B block

data(position, target): Actual branch information in PC_B → Next target PC = PC_C

[S0] SRAM read (based on previous block PC_prev):

BankIndex = PC_prev[2:1] → Select the relevant bank (s0_previousStartPc = io.startPc at cycle N)

SetIndex = PC_prev[7:3] → select set (32 sets)

s0_previousStartPc = io.startPc ← Include “previous” in the variable name: “Previous” block of the next cycle

[S1] Wait for SRAM response + capture current BPU S0 PC:

s1_startPc = io.startPc ← Not registered (RegEnable)! LIVE value (=current BPU S0 PC=PC_curr)

s1_entries = SRAM response (based on setIndex(PC_prev))

[S2] Tag comparison (based on current block PC_curr):

s2_startPc = RegEnable(s1_startPc) = PC_curr ← A different PC than the PC used to read SRAM!

s2_tag     = getTag(s2_startPc) = PC_curr[24:1]

// 8 ways simultaneous tag comparison → hitMask creation

s2_hitMask[i] = s2_entries[i].valid && s2_entries[i].tag === s2_tag

// entries: value read from setIndex(PC_prev)

//   tag:     PC_curr[24:1]

// → HIT if there is an entry previously saved as (setIndex(PC_prev), tag=PC_curr)

// hitMask can turn on multiple bits — multiple branches of the PC_curr block are stored in each way

// Example) If branch@pos3, branch@pos7, and branch@pos12 exist in the PC_curr block,

// way0(tag=PC_curr, pos=3), way1(tag=PC_curr, pos=7), way2(tag=PC_curr, pos=12) all hit

// Prediction output for each hit way (AheadBtb.scala:172-178)

io.prediction[i].valid       := s2_valid && s2_hitMask[i]

io.prediction[i].taken       := takenCounter[bankIdx][setIdx][i].isPositive

io.prediction[i].cfiPosition := s2_entries[i].position

io.prediction[i].target      := reconstruct(s2_startPc, s2_entries[i])

// → Simultaneous output of up to 8 Valid[Prediction] (valid=true as many ways as hit)

// → BPU receives this result from S1 in PC_curr (arrived in a timely manner thanks to ahead)

> Exception handling (s2_multiHit): Two ways with the same position hit simultaneously → This is anomaly. > In the normal case, all hit ways have different positions (predict_hit_entry_num perf counter).

Entry Contents (abtb/Bundles.scala:83):

AheadBtbEntry { // 1 entry = 1 branch (one branch in the fetch block)

valid:           Bool

tag: UInt(24) // fetch block start PC[24:1] (not individual branch PCs!)

position: UInt(5) // CfiPosition: Branch position within the fetch block (separator between ways)

attribute:       BranchAttribute(4)    // branchType(2) + rasAction(2)

targetLowerBits: UInt(22)              // partial target: branch target PC[22:1]

// targetCarry: Not included (EnableTargetFix=false)

}

// 8 ways × 32 sets × 4 banks = 1,024 entries total

// Multiple ways within the same set can have the same tag (= same fetch block PC)

// → Up to 8 branches can be cached for one fetch block

[Key] Meaning of 8 ways — Multiple Branches in Fetch Block, not Branch Tree

8 ways indicates “the maximum number of branch instructions that the next fetch block (PC_B) can have.” This is not a multi-level branch tree — multiple branches within a single fetch block.

SRAM row read with setIndex(PC_A):

way0: {tag=PC_B, position=2, target=PC_C1} ← PC_B block offset 2nd instruction branch

way1: {tag=PC_B, position=5, target=PC_C2} ← PC_B block offset 5th instruction branch

way2: {tag=PC_B, position=11, target=PC_C3} ← PC_B block offset 11th instruction branch

way3: {tag=PC_K, position=7, target=PC_C4} ← Other path PC_A → branch of PC_K (different tag)

...

// Same tag (=PC_B) → Multiple branches within the same fetch block → Simultaneous hits are normal

// Other tag (=PC_K) → Entry in another path (PC_A → PC_K) → Hit only when PC_K fetch

Two roles in 8 ways:

| role | Description |
| --- | --- |
| **Same tag, different position** | Simultaneous cache of multiple branch instructions in PC_B block (maximum 8) |
| **Other tag** | Coexistence of entries from other paths (PC_A→PC_K, etc.) that share setIndex(PC_A) |

To avoid misunderstanding: It does not cache the branch tree as in "PC_A → PC_B (2 branches) → 8 final destinations". ABTB covers the fact that there may be multiple branches within a single next fetch block (PC_B).

Next PC decision (processed by Bpu.scala):

// Receive up to 8 Valid[Prediction] from ABTB (one for each branch instruction in PC_B block)

// 1 output of uBTB and concat → s1_btbPrediction[0..8] (up to 9 in total)

// CompareMatrix: Select minimum position among those taken → single s1_prediction

// Example) way0(pos=2, taken), way1(pos=5, not-taken), way2(pos=11, taken)

// → What is taken: min position among way0 and way2 = way0(pos=2)

//     → next PC = PC_C1

//

// Reason: Within the fetch block, the branch with the smallest position is executed first.

// → If the branch is taken, subsequent instructions will not be executed.

// → The target of “first taken branch” is next fetch PC

Feature: Cache up to 8 branches within a fetch block simultaneously and return them all in one lookup. CompareMatrix selects the “first branch taken” to determine the next PC.

Pair direction predictor: uTAGE (shared with uBTB, S1 simultaneous results)

Fast Training: BpuFastTrain (s3_valid signal) → Fast training with S3 results

### [Core] ABTB’s “Ahead” pipeline mechanism

#### Compare the paths that two predictors take to reach the same BPU S1

Although uBTB and ABTB index different PCs in different cycles, they are output simultaneously on the same BPU S1 for PC_B.

Cycle N:   BPU S0 = PC_A

uBTB: Register comparison (combinational) at PC_A

ABTB: Start reading SRAM at setIndex(PC_A) ← Physical SRAM, takes 1 cycle

Cycle N+1: BPU S1 = PC_A (→ determine next s0_startPc = PC_B with uBTB output)

uBTB → PC_A Prediction output: target = PC_B ← 1-cycle delay from PC_A

uBTB: Start new register comparison with PC_B

ABTB: SRAM response arrives, tag comparison begins (s1_startPc = PC_B live)

Cycle N+2: BPU S1 = PC_B

uBTB → PC_B prediction output: target = PC_C ← 1-cycle delay from PC_B (N+1)

ABTB → PC_B prediction output: target = PC_C ← 2-cycle delay from PC_A (N)

↑ Both arrive simultaneously from BPU S1 for PC_B!

CompareMatrix → Select minimum cfiPosition → s1_prediction → next s0_startPc = PC_C

Key points:

uBTB: 1-cycle (register-based, starts from PC_B)

ABTB: 2-cycle (based on physical SRAM, starting from PC_A) — ahead trick pulls 1-cycle

In the same ABTB, if the starting point is set to PC_B (BPU s1 current block), the result appears to be 1-cycle.

Both paths reach the same destination (BPU S1 for PC_B)

#### Why is ABTB so much larger that it can be printed in the same cycle?

|  | uBTB | ABTB |
| --- | --- | --- |
| Storage method | **Register** (CAM/FF) | **Physical SRAM** |
| given pipeline cycle | **1-cycle** | **2-cycle** (ahead trick) |
| Cycle N: | PC_A immediate comparison (combinational) | setIndex(PC_A) Read SRAM |
| Cycle N+1: | PC_B immediate comparison (combinational) | SRAM response + tag comparison (s1_startPc=PC_B) |
| Cycle N+2: | PC_B result output | PC_B result output |

Since uBTB is register-based, 1-cycle comparison is sufficient. ABTB is a physical SRAM, so it originally requires 2-cycle, but the ahead trick advances SRAM access by 1-cycle and matches the 2-cycle pipeline to the same BPU S1. Even if the SRAM size is much larger, the timing can be met because a 2-cycle budget is given.

#### entry storage method

Save entry:

setIndex = setIndex(PC_A) ← SRAM address to previous block PC

tag = getTag(PC_B) ← Current (next) block PC

data = branch information taken in PC_B (position, target=PC_C, attribute)

#### Detailed pipeline timeline

Cycle N:   BPU S0=PC_A. ABTB S0 fires.

→ Start reading SRAM: setIndex(PC_A)

Cycle N+1: BPU S0=PC_B, S1=PC_A.

ABTB S1 fires (s1_fire = s1_valid && predictReqValid).

→ s1_startPc = io.startPc = PC_B ← LIVE value, not registered!

→ s1_entries = SRAM response from setIndex(PC_A)

Cycle N+2: BPU S0=PC_C, S1=PC_B.

ABTB S2 fires (s2_fire = s2_valid && predictionSent=BPU.s1_fire).

→ s2_startPc = PC_B, s2_tag = getTag(PC_B)

→ hitMask: entries[setIndex(PC_A)] vs tag=getTag(PC_B)

→ io.prediction output

BPU S1 for PC_B: uBTB + ABTB simultaneous output → CompareMatrix → s1_prediction

→ s2_abtbMeta capture: setIdx=setIndex(PC_A), valid=true

Cycle N+3: BPU S2=PC_B. s3_abtbMeta = s2_abtbMeta, s3_startPc = PC_B.

Cycle N+4: BPU S3=PC_B. fastTrain fires:

→ fastTrain.startPc         = s3_startPc = PC_B

→ fastTrain.abtbMeta.setIdx = setIndex(PC_A)

→ fastTrain.finalPrediction = s3_prediction (final prediction of PC_B)

#### [Core] ABTB Training mechanism

ABTB only uses fastTrain — no FTQ commit train path

AheadBtb.scala:204-206:

private val t0_train = io.fastTrain.get.bits

private val t0_fire  = io.enable && io.fastTrain.get.valid

&& t0_train.finalPrediction.taken

&& t0_train.abtbMeta.valid

ABTB does not have a io.train (FTQ commit path) consumption code. Consumes only io.fastTrain. thus:

Training point: BPU S3 (~3 cycles after PC_B enters BPU S1)

Not “train when PC_C commits” — Completed well before commit (~20-30 cycles later)

Training Timing:

Cycle N: BPU S0 = PC_A → Start reading ABTB SRAM (setIndex(PC_A))

Cycle N+1: BPU S1 = PC_B → abtbMeta capture: setIdx=setIndex(PC_A)

Cycle N+2 : BPU S2 = PC_B → s2_abtbMeta → s3_abtbMeta

Cycle N+3: BPU S3 = PC_B → fastTrain occurs ← Training HERE

Cycle N+4: t1_fire → takenCounter update + SRAM write (if necessary)

commit: Cycle N+20~30 ← ABTB does nothing at this time

Training data source (Bpu.scala:182-186, AheadBtb.scala:272-275):

| SRAM Field | value | Source |
| --- | --- | --- |
| setIdx | `setIndex(PC_A)` | `abtbMeta.setIdx` (captured from BPU S1) |
| tag | `getTag(PC_B)` | `fastTrain.startPc = s3_startPc` |
| position | Location within the fetch block of the taken branch | `s3_prediction.cfiPosition` |
| target | PC_C | `s3_prediction.target` |

→ Result: entry at setIndex(PC_A), tag=getTag(PC_B), data=taken branch information of PC_B (target=PC_C)

abtbMeta lifetime — 3 cycles, no FTQ storage:

// Bpu.scala:151: "abtb meta won't be sent to ftq, used for abtb fast train"

BPU S1 fire → s2_abtbMeta (register)

BPU S2 fire → s3_abtbMeta (register)

BPU S3 → Consume with fastTrain → Overwrite with the next value

The meta of other predictors (MBTB, TAGE, etc.) is stored in the FTQ entry and preserved until commit, but abtbMeta exists only in the BPU internal register for 3 cycles and then disappears. This is why there is no abtbMeta field in the FTQ entry.

Training content — two updates (AheadBtb.scala:234-266):

takenCounter (register, update immediately):

needDecrease = updateThisSet && isCond && (!t1_trainTaken || (t1_trainTaken && posBefore))

// conditional branch whose position is ahead of the taken branch → was actually not taken → decreases

needIncrease = updateThisSet && isCond && t1_trainTaken && posEqual

// conditional branch whose position matches → actual taken → increment

SRAM entry write (2 cases):

t1_needWriteNewEntry: The branch is not in ABTB → New allocation to victim way

t1_needCorrectTarget: Hit, but target lower bits of indirect branch mismatch → target modified

Behavior in case of Misprediction:

If you trained based on S3 prediction and it is actually wrong (redirect when commit):

→ BPU starts re-fetching from the correct PC

→ Relearn (overwrite) ABTB with correct info when the new path passes through BPU S3

ABTB learns quickly with the “best guess based on BPU S3”, and if it is wrong, it is corrected later.

Prediction hit condition (when revisiting the same path PC_A → PC_B):

ABTB S0: SRAM read at setIndex(PC_A)

ABTB S1: s1_startPc = io.startPc = PC_B  ← LIVE

ABTB S2: tag comparison: entry.tag(=getTag(PC_B)) === getTag(PC_B) → HIT!

→ Return branch information taken within PC_B → Pass to BPU S1 for PC_B

Difference from “two-block ahead” in the paper:

|  | thesis two-block ahead | XiangShan ABTB |
| --- | --- | --- |
| By index | PC_A (current block) | PC_A (previous block) |
| By tag | PC_C (two blocks ahead) | PC_B (one block ahead) |
| Prediction target | branch of PC_C | branch of PC_B |
| Purpose | double I-fetch or pipelined | SRAM latency hiding → maintaining S1 timing |

### 11.4 MBTB (Main BTB) — S2 Stage

| Item | value |
| --- | --- |
| BTB method | **Block BTB (multiple branches)** — tag = 32B-aligned half-block PC[31:16], multiple ways share the same tag, 2 AlignBanks × 4 ways = returns up to 8 branches |
| structure | **Set-Associative + 2-Level Banking** |
| Total Entries | **8,192** |
| Ways | **4** |
| Align Banks | **2** (64B fetch block → split into 2 32B half-blocks) |
| Internal Banks | **4** (SRAM power saving / read-write conflict distribution) |
| Sets per SRAM | **256** (= 8192 / 4 ways / 4 ibanks / 2 abanks) |
| Tag Width | **16 bits** |
| Target Width | 20 bits (2B aligned) |
| Taken Counter | **2-bit bimodal** (base predictor) |
| Write Buffer | 4 entries |
| Replacer | **LRU** |
| Use History | **None** |
| Predicted Latency | **2 cycle** |

AddrField bit structure (mbtb/Helpers.scala:30, mbtb/Parameters.scala):

PC bit : [ 4:0  ] [ 5 ] [ 7:6 ] [  15:8  ] [  31:16  ] [ VAddrBits-1:32 ]

field  :  align   aBank  iBank    setIdx       tag           unused

Offset  (1b)   (2b)    (8 bits)    (16 bits)

extraField:

replacerSetIdx = PC[13:6] (FetchBlockSizeWidth=6 from 8 bits)

targetLower = PC[20:1] (instOffsetBits=1 to 20 bits)

position = PC[5:1] (instOffsetBits=1 to 5 bits)

cfiPosition = PC[6:1] (6 bits from instOffsetBits=1)

| field | PC bits | Remarks |
| --- | --- | --- |
| alignOffset | PC[4:0] | 5 bits, 32B offset in alignment block |
| alignBankIdx | **PC[5:5]** | 1 bit → 2 align banks selection |
| internalBankIdx | **PC[7:6]** | 2 bits → 4 internal banks selection |
| setIdx | **PC[15:8]** | 8 bits → 256 sets/SRAM selection |
| tag | **PC[31:16]** | 16 bits → way selection (4-way comparison) |

AlignBank vs InternalBank — 2-Level Banking Tier:

MBTB uses banking for two independent overlapping purposes.

MBTB (total 8192 entries)

├── AlignBank[0] ← Responsible for the lower 32B section of the 64B fetch block

│   ├── InternalBank[0]   ← PC[7:6]=00 entries (physical SRAM)

│   │   ├── entrySRAM × 4 ways

│   │   └── counterSRAM

│   ├── InternalBank[1]   ← PC[7:6]=01

│   ├── InternalBank[2]   ← PC[7:6]=10

│   └── InternalBank[3]   ← PC[7:6]=11

└── AlignBank[1] ← Responsible for the upper 32B section of the 64B fetch block

└── InternalBank[0..3]

Total SRAM: 2 × 4 × (4 entrySRAMs + 1 counterSRAM) = 40

AlignBank (2) — Covers multiple branches of 64B fetch blocks (mbtb/Parameters.scala):

NumAlignBanks = FetchBlockSize / FetchBlockAlignSize = 64B / 32B = 2

Parameter comment: "Highest level banks, alignment restriction"

Purpose: To cover multiple branches spanning a 64B fetch block, parallel lookup by dividing the fetch block into two 32B half-blocks

MBTB tag unit = 32B half-block (divided by PC[31:16] + alignBankIdx)

Parallel lookup from the front 32B of one fetch block → AlignBank[A], and the back 32B → AlignBank[B]

4 ways in each AlignBank → returns a total of 8 branch predictions

VecRotate operation (MainBtb.scala:76-84):

s0_startPcVec(0) = s0_startPc // 32B half-block to which startPc currently belongs

s0_startPcVec(1) = aligned(s0_startPc + 32B) // then 32B half-block

→ rotate by getAlignBankIndex(s0_startPc)

// Result: AlignBank[i] always receives a PC matching its alignIdx

startPc offset filtering (MainBtbAlignBank.scala:166):

val hit = rawHit && e.position >= s2_alignedInstOffset && !s2_crossPage

// If startPc starts in the middle of a 32B half-block, branch entries before startPc are excluded.

// (AlignBank[1] = always aligned PC → alignedInstOffset=0 → no filter)

Coverage constraints: Parameter comment: *"can provide at most (banks-1)/banks × predict width"*

If startPc starts in the middle of a 32B half-block, the 64B fetch may spill out into the third 32B half-block.

2 AlignBanks do not cover the third part → worst-case 50% coverage is guaranteed

InternalBank (4) — SRAM Power Savings + Read-Write Conflict Distribution (mbtb/Parameters.scala):

NumInternalBanks = 4

Parameter comment: "Lowest level banks, read-write conflicts and reduce SRAM power"

Purpose: Divide physical SRAM into 4 → Activate only 1 InternalBank in each cycle → Save 75% of power

Selection method (MainBtbAlignBank.scala):

s0_internalBankMask = UIntToOH(getInternalBankIndex(s0_startPc))

// getInternalBankIndex = PC[7:6] (2 bits) → Only 1 of InternalBank 0~3 is active

Conflict distribution: If the PC[7:6] of the lookup PC and the update PC are different, they are different InternalBank → No conflict

Conflict processing: Forwarding to WriteBuffer (4 entries) when hitting the same InternalBank

Physical SRAM Structure (MainBtbInternalBank.scala):

entrySrams = Seq.tabulate(NumWay)(wayIdx => SRAMTemplate(...)) // Separate entry SRAM for each way

counterSram = SRAMTemplate(TakenCounter, set=NumSets, way=NumWay) // SRAM dedicated to counter

|  | AlignBank | InternalBank |
| --- | --- | --- |
| **Number** | 2 | 4 (per AlignBank) |
| **Purpose** | Cross-alignment processing | SRAM power saving + conflict distribution |
| **Activate** | All cycles are parallel active | Only 1 active per cycle (based on PC[7:6]) |
| **Holds SRAM** | None (routing layer) | Yes (entrySRAM × 4 + counterSRAM) |

Lookup method (mbtb/MainBtbAlignBank.scala):

// Actions per AlignBank (2 in parallel):

alignBankIdx = PC[5] → Select AlignBank (routing has already been completed in MainBtb top)

internalBankIdx = PC[7:6] → Select 1 of 4 internal banks (SRAM enabled)

SetIndex = PC[15:8] → Select from 256 sets

Tag = PC[31:16] → Compare tags simultaneously with all 4 ways

// Same as ABTB: multiple branches of the same 32B half-block are stored in each way

// → Multiple ways can match tags simultaneously (multi-hit normal operation)

rawHit[i] = entry[i].valid && entry[i].tag == s2_tag

hit[i]    = rawHit[i] && entry[i].position >= alignedInstOffset && !crossPage

// position condition:

// AlignBank[0] (including startPc): alignedInstOffset > 0 possible → exclude branch before startPc

// AlignBank[1] (next half-block): alignedInstOffset = 0 → fully inclusive

// Result: Up to 4 Valid[Prediction] outputs from each AlignBank

// → 2 AlignBanks flatten → total maximum of 8 (same structure as ABTB)

io.result = alignBanks.flatMap(_.io.read.resp.predictions) // Total 8

> PC[VAddrBits-1:32] is not included in the tag → Aliasing of the upper bits of the physical address is possible. > Practically guarantees sufficient distinction within the 32-bit PC range (PC[31:0]).

Entry Contents (mbtb/Bundles.scala:35):

// MBTB uses two separate SRAMs (entry SRAM + counter SRAM)

// [1] Entry SRAM (structure information):

MainBtbEntry {

valid:           Bool

tag: UInt(16) // PC[31:16], for comparing way selection

attribute:       BranchAttribute(4)     // branchType(2) + rasAction(2)

position: UInt(4) // CfiAlignedPosition: branch position in alignBank

// = CfiPosition(5) - alignBankIdx(1) → 4 bits

targetCarry: TargetCarry(2) // Fit/Overflow/Underflow (always save)

targetLowerBits: UInt(20)             // partial target: branch target PC[20:1]

}

// [2] Counter SRAM (direction information, base predictor):

counters: Vec[4, SatCtr(2-bit)] // 4 ways each taken/not-taken bimodal counter

// → Use as TAGE base predictor

// TAGE/SC overrides this to determine the final direction

Next PC Decision (mbtb/MainBtb.scala, Bpu.scala):

// Receive 8 predictions from S2 (2 AlignBanks × 4 ways, same structure as ABTB)

io.result = alignBanks.flatMap(_.io.read.resp.predictions)  // Vec[8, Valid[Prediction]]

// Each prediction entry:

pred[i].valid       = rawHit && position >= alignedInstOffset && !crossPage

pred[i].cfiPosition = Cat(posHigherBits, entry.position)    // {alignBankIdx, 4-bit} = 5-bit

pred[i].target = reconstruct fullTarget (correct high bits with targetCarry)

pred[i].taken       = counter.isPositive  // bimodal base predictor

// Same as ABTB's multi-hit: multiple branches of the same 32B half-block are stored in each way

// → Multiple ways can hit at the same time (same tag, different position)

// → Valid ones out of 8 are sent to s2_mbtbResult of BPU top

// Determine direction (Bpu.scala s2_condTakenMask, applied independently for each entry):

// 1st priority: SC exceeds threshold → SC direction

// 2nd priority: TAGE provider table hit → TAGE direction

// 3rd place: pred[i].taken (MBTB bimodal counter)

finalTaken[i] = Mux(sc.scUsed[i],       sc.scTakenMask[i],

Mux(tage.useProvider[i], tage.providerPred[i],

pred[i].taken))

// Final branch selection: CompareMatrix → taken entry of minimum cfiPosition → single s3_prediction

Pair direction predictor: TAGE (8-table) + SC (Statistical Corrector)

MBTB’s built-in 2-bit counter = TAGE base predictor (fallback)

TAGE full predictor overrides provider table

### 11.5 History Types — PHR vs GHR

> Key: In this implementation, TAGE / uTAGE / ITTAGE / SC(path) all use PHR(Path History Register). > It is different from the GHR (branch direction taken/not-taken record) of traditional TAGE.

#### 11.5.1 PHR (Path History Register) — bpu/history/phr/

Every quarter, (PC, target) pairs are summarized and accumulated as a 15-bit hash.

pathHash(pc, target) = Cat(PC[9:1], 4'b0000)  XOR  target[16:2]

←─── 9 bits of PC ───→      ←─ 15 bits ─→

Result: 15 bits (PathHashWidth = 15)

PHR update method: Shift Shamt=2 bits to the left every quarter and then XOR the new hash

PHR_new = (PHR_old << 2) XOR pathHash(pc, target)

PHR maximum cumulative length: 397 bits (based on histLen in TAGE Table 7)

FoldedHistory: XOR-fold the long bit string of PHR into N-bits and supply it to the predictor

XOR-Fold Algorithm (computeFoldedHist, bpu/Helpers.scala:172):

FoldPHR(N) = Divide PHR into N-bit chunks and XOR them all

Example) PHR[397:0] → fold to 9 bits:

chunk0 = PHR[8:0]

chunk1 = PHR[17:9]

...

chunk44 = PHR[396:387]  (9 bits, zero-padded)

FoldPHR(9) = chunk0 XOR chunk1 XOR ... XOR chunk44

#### 11.5.2 GHR (Global History Register) — bpu/history/commonhr/

Record taken/not-taken results of conditional branch as bit stream. Updated upon S3 fire.

To be used only in SC global table (GlobalEnable), but currently `GlobalEnable = false` (inactive)

SC reverse table (BWEnable) is also `BWEnable = false` (inactive)

#### 11.5.3 PC bit extraction principle (AddrField)

AddrField allocates fields sequentially starting from bit 0 (utils/AddrField.scala):

extract(fieldName, pc) = pc[end:start] (start/end are sequential accumulation)

Common Prerequisites (XiangShan KMH v3, RVC enabled):

instOffsetBits = 1 (2B alignment, bit 0 = always 0)

FetchBlockAlignWidth = 5 (32B = FetchBlockSize/2)

-VAddrBits = 50 (PrunedAddr length)

### 11.6 uTAGE (Micro TAGE) — Index/Tag Hashing details

Pair with uBTB/ABTB. S1 1-cycle results. `MicroTageTable.scala:83`

unhashedIdx = PC[VAddrBits-1 : 1]   (PC >> instOffsetBits)

unhashedTag = PC[VAddrBits-1 : 7]   (PC >> PCHighTagStart=7)

#### Table 0 (512 sets, histLen=9, histBitsInTag=9, tagLen=15)

| Item | FoldedLength | formula |
| --- | --- | --- |
| idxFhInfo | min(9, 9) = **9 bits** | PHR[9-1:0] fold → 9 bits |
| tagFhInfo | min(9, 9) = **9 bits** | PHR fold → 9 bits |
| altTagFhInfo | min(9, 8) = **8 bits** | PHR fold → 8 bits |

SetIndex = (PC[9:1] XOR FoldPHR_9)[8:0]

lowTag   = (PC[VAddrBits-1:7] XOR FoldPHR_9 XOR (FoldPHR_8 << 1))[8:0]

highTag  = Cat(PC[16], PC[14], PC[12], PC[10], PC[8], PC[7], PC[6],  ← 11 bits

PC[5], PC[4], PC[3], PC[2]) (non-consecutive selection)

Tag      = Cat(highTag, lowTag)[14:0]                               ← 15 bits 截断

= {highTag[5:0], lowTag[8:0]}

> highTag applies the PCTagHashBitsForShortHistory = [15,13,11,9,7,6,5,4,3,2,1] index to unhashedIdx. > Since unhashedIdx[i] = PC[i+1], the actual PC bit is each index + 1.

#### Table 1 (512 sets, histLen=16, histBitsInTag=12, tagLen=16)

| Item | FoldedLength | formula |
| --- | --- | --- |
| idxFhInfo | min(16, 9) = **9 bits** | PHR fold → 9 bits |
| tagFhInfo | min(16, 12) = **12 bits** | PHR fold → 12 bits |
| altTagFhInfo | min(16, 11) = **11 bits** | PHR fold → 11 bits |

SetIndex = (PC[9:1] XOR FoldPHR_9)[8:0]

lowTag   = (PC[VAddrBits-1:7] XOR FoldPHR_12 XOR (FoldPHR_11 << 1))[11:0]

highTag  = Cat(PC[19], PC[17], PC[15], PC[13], PC[11],  ← 10 bits

PC[7],  PC[6],  PC[5],  PC[3],  PC[2])   (PCTagHashBitsForMediumHistory)

Tag      = Cat(highTag, lowTag)[15:0]                   ← 16 bits 截断

= {highTag[3:0], lowTag[11:0]}

Counter: 3-bit signed, Useful: 2 bits

Fast Training: Learning with S3 results (BpuFastTrain). No commit-based learning (no consecutive prediction blocks)

### 11.7 TAGE (Main TAGE) — Index/Tag Hashing details

Pair with MBTB. S2 Results. `tage/Helpers.scala:51`, `tage/TageTable.scala`

AddrField structure (same structure for 8 tables, 4096 total / 4 banks / 2 ways = 512 sets):

PC bit  : [ 0 ] [ 2:1 ] [ 11:3 ] [ 24:12 ] [ 49:25 ]

field   :  inst   bank   setIdx     tag      unused

Offset  Idx    (9 bit)  (13 bit)

| field | PC bits | Remarks |
| --- | --- | --- |
| instOffset | PC[0:0] | 1 bit, always 0 (RVC 2B alignment) |
| bankIdx | PC[2:1] | 2 bits, 4 banks |
| setIdx | PC[11:3] | 9 bits, 512 sets/bank |
| tag | PC[24:12] | 13 bits |

// Create 3 FoldedHistory for each of 8 tables

idxFhInfo    : FoldedLength = min(histLen, 9)   → FoldPHR_idx

tagFhInfo    : FoldedLength = min(histLen, 13)  → FoldPHR_tag

altTagFhInfo : FoldedLength = min(histLen, 12)  → FoldPHR_altTag

// tage/Helpers.scala:34

forIdx = FoldPHR_idx

forTag = FoldPHR_tag XOR Cat(FoldPHR_altTag, 0.U(1.W))

= FoldPHR_tag XOR (FoldPHR_altTag << 1)   ← 13 bits

// tage/Helpers.scala:64-65

BankIndex = PC[2:1] ← SRAM bank selection

SetIndex  = PC[11:3] XOR forIdx                   ← 9 bits

// tage/Helpers.scala:67-68

RawTag    = PC[24:12] XOR forTag                  ← 13 bits

// Tage.scala (CfiPosition XOR when tag matching)

FinalTag = RawTag

Example of FoldPHR bit range by histLen:

| Table | histLen | FoldPHR_idx (9 bit) | FoldPHR_tag (13 bit) |
| --- | --- | --- | --- |
| 0 | 4 | PHR[3:0] → fold4→pad9 | PHR[3:0] → fold4→pad13 |
| 3 | 29 | PHR[28:0] → fold9 | PHR[28:0] → fold13 |
| 7 | 397 | PHR[396:0] → fold9 | PHR[396:0] → fold13 |

> fold4→pad9: FoldedLength=4 < When SetIdxWidth=9, unlike uTAGE, TAGE takes 9-bit after simple XOR. > If histLen is short, only the lower histLen bits are valid in the folded result, and the remaining padding = 0.

### 11.8 TAGE Base vs TAGE Full

| Category | TAGE Base | TAGE Full |
| --- | --- | --- |
| Implementation | Built-in MBTB **2-bit bimodal counter** | 8 tagged history table |
| role | **default prediction** when no table is hit | Sophisticated predictions with Provider/Alternate logic |
| Use History | **None** | PHR up to **397 bits** route history |
| selection logic | simple taken/not-taken | Provider (longest matching history) → Alternate if weak |
| Override Relationship | Base = fallback | Full overrides base |

Provider selection logic:

if (provider table hit && !(useAltOnNA && counter is weak)):

prediction = provider.isTaken

else if (alternate table hit):

prediction = alternate.isTaken

else:

prediction = MBTB bimodal (base)

### 11.9 SC (Statistical Corrector) — Implementation Details

A predictor that statistically detects and corrects when TAGE is biased in a specific direction. `sc/Sc.scala`, `sc/Helpers.scala`

> Loop predictor ("L" in TAGE-SC-L): Not implemented in this codebase. There is only loop_enable field in BPUCtrl and no actual module.

#### 11.9.1 Table structure

| Table | active | Sets | histLen | By WayIdx | SetIndex formula |
| --- | --- | --- | --- | --- | --- |
| PathTable[0] | **True** | 128 | 8 | cfiPos[2:0] | `(PC[11:5] XOR FoldPHR_7)[6:0]` |
| PathTable[1] | **True** | 128 | 16 | cfiPos[2:0] | `(PC[11:5] XOR FoldPHR_7)[6:0]` |
| GlobalTable[0] | False | 128 | 8 | — | `(PC[11:5] XOR FoldGHR_7)[6:0]` ← GlobalEnable=false |
| GlobalTable[1] | False | 128 | 16 | — | `(PC[11:5] XOR FoldGHR_7)[6:0]` ← Inactive |
| BWTable[0] | False | 128 | 4 | — | `(PC[11:5] XOR FoldBW_7)[6:0]` ← BWEnable=false |
| BWTable[1] | False | 128 | 8 | — | `(PC[11:5] XOR FoldBW_7)[6:0]` ← Inactive |
| BiasTable | **True** | 128 | — | `Cat(cfiPos[2:0], {tageweak,tagetaken})` | `PC[11:5][6:0]` |

Common Structure:

NumWays = NumBtbResultEntries = 8 ← Number of MBTB result entries

WayIdx = cfiPosition[2:0] ← Branch position lower 3 bits within fetch block

BankIndex = PC[4:4] ← PC >> (1+3) lower 1 bit

Counter   = 6-bit signed saturating    ← ScEntry.ctr

BiasTable WayIdx = Cat(cfiPos[2:0], tageweak(1), tagetaken(1))  ← 5 bits → 32 ways

tageweak  = provider.valid && provider.ctr.isWeak

tagetaken = provider.valid && provider.ctr.isPositive

→ Save “Bias when TAGE is weakly taken” in a separate cell

#### 11.9.2 Core formula — percsum

// Helpers.scala:61

def getPercsum(ctr: SInt): SInt = Cat(ctr, 1.U(1.W)).asSInt

// Meaning: percsum = 2 * ctr + 1 (7-bit signed)

| ctr (6-bit signed) | percsum |
| --- | --- |
| +31 (maximum saturation) | **+63** (strong taken) |
| +1 (weak taken) | **+3** |
| 0 (neutral) | **+1** (≠0: dead-zone removal) |
| -1 (weak not-taken) | **-1** |
| -32 (maximum saturation) | **-63** (strong not-taken) |

> Reason for adding LSB=1: In the neutral state of ctr=0, percsum=+1 eliminates the dead-zone that becomes 0 when summed.

#### 11.9.3 Prediction Pipeline (S1→S2)

S1 (SRAM read complete):

s1_sumPercsum[wayIdx] = Σ percsum(pathTable[k][wayIdx].ctr) (k=0,1 active only)

S2 (MBTB results arrive):

biasWayIdx[i] = Cat(wayIdx, {tageweak, tagetaken})

totalPercsum[i] = s1_sumPercsum[wayIdx] + biasPercsum[biasWayIdx]

// Determine effective threshold according to TAGE provider confidence

threshold T = scThreshold[wayIdx].value >> 3 (initial 720 >> 3 = 90)

TAGE Saturate(isSaturate): effectiveThres = T >> 1 = 45

TAGE middle(isMid): effectiveThres = T >> 2 = 22

TAGE weak (isWeak): effectiveThres = T >> 3 = 11

scUsed[i] = hit && tagePredValid && aboveThreshold(totalPercsum[i], effectiveThres)

scPred[i] = (totalPercsum[i] >= 0) ← SC prediction direction

Intuition: The more confident TAGE is, the stronger the signal is needed for SC to overturn.

// Helpers.scala:63

def aboveThreshold(scSum: SInt, threshold: UInt): Bool =

(scSum > threshold.zext) && pos(scSum) ||

(scSum < -threshold.zext) && neg(scSum)

#### 11.9.4 Training

Table ctr update condition:

needUpdate = writeValid

&& tagePredValid

&& (scPred ≠ actual  ||  !sumAboveThres)

→ If the SC is wrong or the sum is less than the threshold, learn (if it exceeds the threshold and the correct answer is already enough → skip the learning)

Update direction: ctr.getUpdate(actual_taken)

Adaptive Threshold Updates:

shouldUpdate = (tagePred ≠ scPred) && (scWrong || !sumAboveThres)

threshold.getUpdate(scWrong)

→ If SC is wrong, threshold++ (use only when more certain)

→ If SC is correct, threshold-- (use more actively)

← Independent 12-bit saturating counter for each way

#### 11.9.5 Operation example

Scenario: Loop branch 0x1080 is taken 3 out of 4 times, not taken once. TAGE always predicts TAKEN.

branch: cfiPosition=5,  wayIdx = 5[2:0] = 5

[S2 prediction time]

TAGE provider: ctr=+1 (positive number, weak) → tageweak=1, tagetaken=1

biasWayIdx = Cat(5, 0b11) = 0b10_1011 = 43

PathTable[histLen=8 ]: ctr = -10 → percsum = -19

PathTable[histLen=16]: ctr = -15 → percsum = -29

sumPercsum[5] = -48

BiasTable[43]:  ctr = -8  → biasPercsum = -15

totalPercsum = -48 + (-15) = -63

TAGE weak → effectiveThreshold = 90 >> 3 = 11

|−63| = 63 > 11 → scUsed = true

scPred = (−63 >= 0) = NOT_TAKEN   ← TAGE(TAKEN) override!

→ Final prediction: NOT_TAKEN ← Correct answer

[Learning point: actual = NOT_TAKEN]

needUpdate = true && true && (NOT_TAKEN ≠ NOT_TAKEN || !sumAboveThres)

= true && (false || !(63 > 90)) = true && true = true

→ ctr update: increases in NOT_TAKEN direction

shouldUpdateThres = (tagePred=TAKEN ≠ scPred=NOT_TAKEN) && (scWrong=false || ...)

= true && (false || !(63>90)) = true && true = true

threshold.getUpdate(scWrong=false) → threshold--

→ Use SC more easily next time

#### 11.9.6 Current implementation status

| component | status |
| --- | --- |
| PathTable (histLen=8,16) | **Active** (PHR based) |
| GlobalTable (histLen=8,16) | **Disabled** (GlobalEnable=false, GHR inactive) |
| BWTable (histLen=4,8) | **Disabled** (BWEnable=false) |
| BiasTable | **Active** (PC + TAGE Result Index) |
| Adaptive Threshold | **Active** (independent 12-bit counter for each way, initial value 720) |
| Loop predictor ("L") | **Not implemented** (Only BPUCtrl.loop_enable field exists) |

### 11.10 ITTAGE (Indirect Target TAGE) — Index/Tag Hashing details

Indirect branch target prediction. `ittage/IttageTable.scala:94`

unhashedIdx = PC >> instOffsetBits = PC[VAddrBits-1 : 1]

NumBanks = 2, bankIdxWidth = 1

| Table | nRows | histLen | setsPerBank | setIdxWidth | bankIdx | setIdxBase | setIdx | tagBase |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 256 | 4 | 128 | 7 | PC[1] | PC[8:2] | `(PC[8:2] XOR FoldPHR_4)[6:0]` | PC[VAddrBits-1:9] |
| 1 | 256 | 8 | 128 | 7 | PC[1] | PC[8:2] | `(PC[8:2] XOR FoldPHR_7)[6:0]` | PC[VAddrBits-1:9] |
| 2 | 512 | 13 | 256 | 8 | PC[1] | PC[9:2] | `(PC[9:2] XOR FoldPHR_8)[7:0]` | PC[VAddrBits-1:10] |
| 3 | 512 | 16 | 256 | 8 | PC[1] | PC[9:2] | `(PC[9:2] XOR FoldPHR_8)[7:0]` | PC[VAddrBits-1:10] |
| 4 | 512 | 32 | 256 | 8 | PC[1] | PC[9:2] | `(PC[9:2] XOR FoldPHR_8)[7:0]` | PC[VAddrBits-1:10] |

Tag formula (same structure for all tables):

tagFhInfo    : FoldedLength = min(histLen, 9)   → FoldPHR_tag

altTagFhInfo : FoldedLength = min(histLen, 8)   → FoldPHR_altTag

Tag = (tagBase XOR FoldPHR_tag XOR (FoldPHR_altTag << 1))[8:0]   ← 9 bits

Example) Table 4 (histLen=32):

Tag = (PC[VAddrBits-1:10] XOR FoldPHR_9 XOR (FoldPHR_8 << 1))[8:0]

Region-based target compression: 16 regions, 2 ports, PLRU replacer

TargetOffset: 20 bits (offset within region)

### 11.11 PC Bit usage comparison table (full summary)

| Predictor | history type | PC bits for index | PC bits for tag |
| --- | --- | --- | --- |
| **uBTB** | None | — (fully assoc) | Direct comparison of entire PC tags (22 bits) |
| **ABTB** | None | PC[7:3] (5 bits) | PC[24:1] (24 bits) |
| **MBTB** | None | PC[15:8] (8 bits) | PC[31:16] (16 bits) |
| **uTAGE T0** | PHR (path) | PC[9:1] XOR FoldPHR_9 | lowTag: PC[VAddrBits-1:7] XOR PHR; highTag: PC select bit concat |
| **uTAGE T1** | PHR (path) | PC[9:1] XOR FoldPHR_9 | lowTag: PC[VAddrBits-1:7] XOR PHR; highTag: PC select bit concat |
| **TAGE (all)** | PHR (path) | PC[11:3] XOR FoldPHR_idx | PC[24:12] XOR forTag XOR CfiPos |
| **SC path** | PHR (path) | PC[11:5] XOR FoldPHR_7 | None (tagless) |
| **ITTAGE T0~1** | PHR (path) | PC[8:2] XOR FoldPHR | PC[VAddrBits-1:9] XOR FoldPHR |
| **ITTAGE T2~4** | PHR (path) | PC[9:2] XOR FoldPHR | PC[VAddrBits-1:10] XOR FoldPHR |

### 11.12 RAS (Return Address Stack) — S3 Stage

| Category | uRAS (Micro RAS) | RAS (Full) |
| --- | --- | --- |
| Stage | **S1** (fast speculative) | **S3** (commit-based) |
| Stack Size | — | Commit stack **16** + Speculative queue **32** |
| Stack Counter | — | 3-bit (merged call expression, maximum 7) |
| Use | ret target fast delivery | Restore correct ret address |

### 11.13 Maximum length by history

| History Type | maximum length | Usage Predictor |
| --- | --- | --- |
| PHR (Path: PC+target hash) | **397 bits** (TAGE Table 7) | TAGE, uTAGE, ITTAGE, SC(path) |
| GHR (Global: taken/not-taken) | 16 bits | SC(global) ← Currently inactive |
| Backward History | 8 bits | SC(backward) ← Currently inactive |
| PHR per-branch hash window | **15 bits** | pathHash = Cat(PC[9:1],0000) XOR target[16:2] |

### 11.14 FastTrain mechanism

#### 11.14.1 Why do you need FastTrain separately?

The general train signal arrives after a delay of 20 to 30 cycles through the path Backend → ROB commit → FTQ → BPU. Since the S1 predictors (uBTB, ABTB, uTAGE) are responsible for the fastest prediction path of the BPU, FastTrain is a BPU internal S3 result that is immediately converted into a learning signal to update them without delay.

General Train: BPU → FTQ → ROB execution → FTQ → BPU (~20~30 cycles)

FastTrain:   BPU S3 → BPU S1 predictor              (~2~3 cycles)

For uTAGE, FastTrain is set to Required because “there are no consecutive prediction blocks at the time of resolution” (EnableFastTrain = true, utage/Parameters.scala).

#### 11.14.2 BpuFastTrain bundle structure (bpu/Bundles.scala)

Comment: // use s3 prediction to train s1 predictors

BpuFastTrain {

startPc: PrunedAddr ← S3 block PC (by ABTB analysis: PC_B)

finalPrediction: Prediction { ← S3 final prediction

taken:       Bool

cfiPosition: UInt(5)

target:      PrunedAddr

attribute:   BranchAttribute

}

hasOverride: Bool ← Has S3 overridden S1 prediction?

abtbMeta: AheadBtbMeta { ← ABTB S2 output meta (FTQ non-delivery, fastTrain only)

valid:    Bool

setIdx: UInt ← setIndex(PC_A): SRAM read to previous block (ahead)

bankMask: UInt

entries:  Vec[8, {

hit: Bool ← Whether the corresponding way hit in S2

attribute:       BranchAttribute

position:        UInt(5)

targetLowerBits: UInt(22)

}]

}

utageMeta: MicroTageMeta { ← uTAGE S1 output meta

histTableHitMap: Vec[2, Bool] ← Whether each uTAGE table hit or not

histTableTakenMap: Vec[2, Bool] ← Taken prediction of each table

histTableUsefulVec: Vec[2, UInt] ← useful counter

histTableCfiPositionVec: Vec[2, UInt] ← CFI position of each table

baseTaken: Bool ← ABTB based prediction direction

baseCfiPosition:         UInt

}

}

#### 11.14.3 Create and Deliver (Bpu.scala:182-195)

fastTrain.valid                := s3_valid

fastTrain.bits.startPc         := s3_startPc          // PC_B

fastTrain.bits.finalPrediction := s3_prediction // S3 final prediction

fastTrain.bits.abtbMeta := s3_abtbMeta // ABTB meta captured during BPU S1(PC_B) fire

fastTrain.bits.utageMeta       := s3_utageMeta

fastTrain.bits.hasOverride     := s3_override

predictors.foreach { p =>

p.io.fastTrain.foreach(_ := fastTrain) // Passed only to predictors inherited from HasFastTrainIO

}

abtbMeta capture path (Bpu.scala:151-153):

// abtb meta won't be sent to ftq, used for abtb fast train

private val s2_abtbMeta = RegEnable(abtb.io.meta, s1_fire) // Capture when BPU S1(PC_B) fires

private val s3_abtbMeta = RegEnable(s2_abtbMeta, s2_fire)

// → s3_abtbMeta.setIdx = setIndex(PC_A): ahead indexing preservation

> Key: abtbMeta is not sent as FTQ. Preserved only within BPU → Feedback to ABTB with fastTrain.

#### 11.14.4 Consumer predictor and t0_fire conditions

| Predictor | Activate | t0_fire condition | Remarks |
| --- | --- | --- | --- |
| **ABTB** (`abtb/AheadBtb.scala:206`) | Always | `enable && valid && taken && abtbMeta.valid` | When taken, only when meta is valid |
| **uBTB** (`ubtb/MicroBtb.scala:103`) | `UseFastTrain=true` (default) | `enable && valid` | no conditions taken; If false, replaced by mispred train |
| **uTAGE** (`utage/MicroTage.scala:127`) | `EnableFastTrain=true` (required) | `enable && valid` | conditional branch only internal filter |

#### 11.14.5 ABTB fastTrain training logic

t0_fire condition interpretation:

enable — Enable ABTB

&& fastTrain.valid — BPU S3 stage valid

&& finalPrediction.taken — S3 final prediction taken

(fallthrough = not-taken is the default → no learning required)

&& abtbMeta.valid — Did ABTB actually output the prediction in S1?

(ABTB miss → meta.valid=false → skip)

T1 learning operation (abtb/AheadBtb.scala:217-304):

t1_setIdx = abtbMeta.setIdx = setIndex(PC_A) ← Use ahead index as is

t1_bankMask = abtbMeta.bankMask

[Update taken counter] (by conditional branch entry)

position < trainPos → counter decrease (that branch is not actually taken)

position == trainPos → increment counter (matches actual taken branch)

[write entry]

!t1_hit: → Assign new entry

tag = getTag(fastTrain.startPc) = getTag(PC_B) ← ahead tag

stored at setIndex(PC_A)

t1_hit && isIndirect && targetDiff → target modification (maintain tag/position)

#### 11.14.6 uBTB fastTrain training logic

uBTB is an always-taken predictor, so it is trained regardless of whether it is taken or not:

t0_fire = fastTrain.valid && enable (no taken condition)

By T1 case:

!hit: → Replace victim entry (select useful=0)

hit, mismatch or !actualTaken: → Decrease useful counter or reinitialize entry

hit, all matches, actualTaken: → increase useful counter (strengthen entry)

Operates when FastTrain=false (ubtb/Parameters.scala: can be changed to UseFastTrain=false):

// Replaced with regular train (only in case of misprediction)

t0_fire = io.stageCtrl.t0_fire && io.train.mispredictBranch.valid && io.enable

#### 11.14.7 uTAGE fastTrain learning logic

uTAGE only handles conditional branch directions and learns after detecting misprediction:

t0_fire = fastTrain.valid && enable

misprediction verdict:

t0_histHitMisPred = uTAGE hit, but (direction or cfiPosition) prediction was wrong

t0_histMissHitMisPred = uTAGE miss + prediction taken by S3 override

Learning decisions:

needAlloc = mispred → Allocate new entry (useful=0 overwrite victim)

needUpdate = hit → counter update

useful counter:

Increase: Accurate prediction && ABTB (base predictor) is incorrect ← uTAGE contributes to correction

reduction: misprediction

#### 11.14.8 Regular Train vs FastTrain comparison

| Item | General Train (`BpuTrain`) | FastTrain (`BpuFastTrain`) |
| --- | --- | --- |
| **Source** | Backend → FTQ → BPU | BPU internal S3 |
| **Delay** | ~20~30 cycles | ~2~3 cycles |
| **Accuracy** | Actual execution result (ground truth) | S3 prediction (estimate) |
| **Target** | All predictors | S1 predictor only (uBTB, ABTB, uTAGE) |
| **Frequency** | When mispred + commit | Every S3 valid cycle |
| **Meta** | BpuResolveMeta (store and return FTQ) | abtbMeta + utageMeta (BPU internal retention) |

# frontend_Ftq_analysis.md

Block: Frontend

Module: Ftq

Source: NewFtq.scala, FrontendBundle.scala

Protocols: Decoupled (fromBpu.resp, toIfu.req, toICache.req, toPrefetch.req), Valid (toBpu.redirect/update, fromIfu.pdWb, fromBackend.redirect)

Key Params: FtqSize=64, PredictWidth=16, copyNum=5, FtqRedirectAheadNum

Last updated: 2026-02-18

→ See [frontend_block_diagram_overview.md](./frontend_block_diagram_overview.md) → See [frontend_in_out_seq_diagram.md](./frontend_in_out_seq_diagram.md#group-c)

## 1. Module Summary

Role: A central queue that buffers BPU prediction results, supplies fetch requests to IFU / ICache / Prefetch in order, and updates/restores BPU by receiving IFU predecode results and Backend commit/redirect.

Position: Frontend.scala → Module(new Ftq) (inside FrontendInlinedImp)

Number of pipeline stages: No register stages (pointer-based circular queue) — However, 1 to 2 cycle SRAM latency exists between BPU enqueue and IFU transmission.

## 2. Key Parameters

| Parameter | Source | Default | Impact |
| --- | --- | --- | --- |
| FtqSize | `p(XSCoreParamsKey).FtqSize` | 64 | Maximum number of in-flight fetch blocks (circular) |
| PredictWidth | `HasXSParameter.PredictWidth` | 16 | number of slots per commitStateQueue entry |
| copyNum | `Ftq.copyNum` | 5 | Number of pointer replicas (reduces fanout) |
| FtqRedirectAheadNum | `HasXSParameter` | ~4 | BjuCnt based redirect ahead port number |
| IfuRedirectNum | `HasXSParameter` | 1 | IFU redirect SRAM read port number |

## 3. Interfaces

| Port | Dir | Bitwidth | Protocol | Description |
| --- | --- | --- | --- | --- |
| `io.fromBpu.resp` | in | BpuToFtqBundle | Decoupled | BPU prediction results (including s1/s2/s3) |
| `io.toBpu.redirect` | out | BranchPredictionRedirect | Valid | misprediction → BPU redirect |
| `io.toBpu.update` | out | BranchPredictionUpdate | Valid | commit information → BPU learning |
| `io.toBpu.enq_ptr` | out | FtqPtr | — | Pass current bpuPtr |
| `io.toBpu.redirctFromIFU` | out | Bool | — | IFU redirect or not |
| `io.fromIfu.pdWb` | in | PredecodeWritebackBundle | Valid | IFU predecode writeback |
| `io.toIfu.req` | out | FetchRequestBundle | Decoupled | fetch request with IFU |
| `io.toIfu.redirect` | out | BranchPredictionRedirect | Valid | IFU flush |
| `io.toIfu.flushFromBpu` | out | BpuFlushInfo | — | BPU S2/S3 override flush information |
| `io.toICache.req` | out | FtqToICacheRequestBundle | Decoupled | ICache fetch request (5 copies) |
| `io.toPrefetch.req` | out | FtqICacheInfo | Decoupled | Prefetch request |
| `io.fromBackend` | in | CtrlToFtqIO | — | redirect, rob_commits, ftqIdxAhead, etc. |
| `io.toBackend` | out | FtqToCtrlIO | — | pc_mem write, newest_entry info |
| `io.mmioCommitRead` | in/out | mmioCommitRead | — | IFU MMIO commit confirmation |
| `io.icacheFlush` | out | Bool | — | ICache pipe flush |
| `io.bpuInfo` | out | Bundle | — | BPU hit/misprediction counter |

## 4. Internal Pipeline / State

### 4.1 Pointer structure (all RegInit(FtqPtr(false.B, 0.U)))

| pointer | role | Increase conditions |
| --- | --- | --- |
| `bpuPtr` | BPU new entry enqueue location | `enq_fire` (BPU s1 response fire) |
| `ifuPtr` | Next entry to transfer to IFU | `toIfu.req.fire && allowToIfu` |
| `ifuPtrPlus1/2` | precompute ifuPtr+1/+2 | Same |
| `pfPtr` | Prefetch transfer pointer | `toPrefetch.req.fire` |
| `ifuWbPtr` | IFU predecode writeback confirmation pointer | `ifu_wb_valid` |
| `commPtr` | commit completion pointer | `canCommit` |
| `commPtrPlus1` | commPtr+1 precompute | Same |
| `robCommPtr` | ROB commit synchronous pointer | ROB commit |

Pointer invariant: commPtr ≤ ifuWbPtr ≤ ifuPtr ≤ bpuPtr

### 4.2 Internal SRAM/Register Structure

| name | Type | size | role |
| --- | --- | --- | --- |
| `ftq_pc_mem` (FtqPcMemWrapper) | SyncDataModuleTemplate | FtqSize × Ftq_RF_Components | PC/nextLine storage, multiple read ports |
| `ftq_redirect_mem` | SyncDataModuleTemplate | FtqSize × Ftq_Redirect_SRAMEntry | Branch History/RAS Status (for redirect restore) |
| `ftq_meta_mem` | SyncDataModuleTemplate | FtqSize × MetaEntry | BPU metadata + FTBEntry (for training) |
| `ftq_pd_mem` | SyncDataModuleTemplate | FtqSize × Ftq_pd_Entry | IFU predecode result (brMask, jmpInfo) |
| `ftb_entry_mem` | SyncDataModuleTemplate | FtqSize × FTBEntry_FtqMem | FTB entry (for redirect verification) |
| `update_target` | Reg Vec | FtqSize × VAddrBits | Predicted target address |
| `cfiIndex_vec` | Reg Vec | FtqSize × ValidUInt | CFI Position Index |
| `mispredict_vec` | Reg Vec | FtqSize × Vec(PW, Bool) | Whether each slot has misprediction |
| `pred_stage` | Reg Vec | FtqSize × 2 | prediction stage (S1/S2/S3) |
| `commitStateQueueReg` | RegInit Vec | FtqSize × Vec(PW, 2bit) | Commit status by command (empty/toCommit/committed/flushed) |
| `entry_fetch_status` | RegInit Vec | FtqSize × 1bit | f_to_send / f_sent |
| `entry_hit_status` | RegInit Vec | FtqSize × 2bit | h_not_hit / h_false_hit / h_hit |

### 4.3 Replication structure (reduce fanout)

copied_ifu_ptr[5], copied_bpu_ptr[5]: 5 copies of ifuPtr/bpuPtr

copied_bpu_in_bypass_buf[5], copied_bpu_in_bypass_ptr[5]: bypass data replication

Connect independent signals to each of the five ICache read ports with copyNum=5

### 4.4 BPU enqueue 1-cycle delay

// Critical path mitigation: Register update after 1 cycle of BPU enqueue

last_cycle_bpu_in       = RegNext(bpu_in_fire)

last_cycle_bpu_in_ptr   = RegEnable(bpu_in_resp_ptr, bpu_in_fire)

last_cycle_bpu_target   = RegEnable(bpu_in_resp.getTarget(3), bpu_in_fire)

last_cycle_cfiIndex     = RegEnable(bpu_in_resp.cfiIndex(3), bpu_in_fire)

// Update entry_fetch_status, cfiIndex_vec, and update_target in the next cycle

### 4.5 bypass route (BPU enqueue → IFU transmission 0-cycle)

// SRAM bypass when the entry just enqueued by BPU matches ifuPtr

when(last_cycle_bpu_in && bpu_in_bypass_ptr === ifuPtr):

toIfuPcBundle    := bpu_in_bypass_buf_for_ifu

entry_is_to_send := true.B

entry_next_addr  := last_cycle_bpu_target

## 5. Functionality

### 5.1 BPU enqueue (fromBpu → FTQ)

io.fromBpu.resp.fire && allowBpuIn → enq_fire → bpuPtr += 1

bpu_in_resp = selectedResp (S3 redirect > S2 redirect > S1)

SRAM write (after 1 cycle): ftq_pc_mem, ftq_redirect_mem (lastStage), ftq_meta_mem (lastStage)

Rewind bpuPtr when S2/S3 redirect: bpuPtr := bpu_s2_resp.ftq_idx + 1

### 5.2 IFU/ICache transfer (FTQ → IFU/ICache)

entry_is_to_send = (entry_fetch_status(ifuPtr) === f_to_send) || bypass

io.toIfu.req.valid = entry_is_to_send && ifuPtr ≠ bpuPtr

io.toIfu.req.fire && allowToIfu → ifuPtr += 1, entry_fetch_status := f_sent

Simultaneous transmission of 5 copies to ICache (toICachePcBundle/toICacheEntryToSend)

allowToIfu = !ifuFlush && !backendRedirect && !backendRedirectReg

### 5.3 IFU predecode writeback (fromIfu → FTQ)

ifu_wb_valid = pdWb.valid → ftq_pd_mem write

commitStateQueueNext: inRange && valid slot c_empty → c_toCommit

ifuWbPtr += 1

FTB hit, mispred detected → has_false_hit → false_hit processed, hit_pd_mispred recorded

When IFU redirect occurs → toBpu.redirect.valid (Send IFU modification information)

### 5.4 Commit processing (fromBackend → FTQ)

canCommit: When all c_toCommit slots in the commPtr entry are committed or empty.

Update mispredict_vec with the actual result of ROB when committing

FTBEntryGen: Create a new FTB entry with predecode + commit information

toBpu.update.valid → BPU training data transfer (pc, ftb_entry, br_taken_mask, mispred_mask, etc.)

commPtr += 1

### 5.5 Backend redirect (fromBackend → FTQ)

backendRedirect.valid → stage2Flush = true

backendFlush = stage2Flush || RegNext(stage2Flush) (2 cycles)

allowBpuIn = allowToIfu = false (flush period)

bpuPtr, ifuPtr, pfPtr → redirect target-based restoration

toBpu.redirect.valid → BPU history restoration

icacheFlush → Invalidate ICache pipe

toIfu.redirect.valid → IFU flush

### 5.6 BPU S2/S3 override

// S2 redirect

when(bpu_s2_redirect):

bpuPtr := bpu_s2_resp.ftq_idx + 1

if ifuPtr > bpu_s2_resp.ftq_idx:

ifuPtr := bpu_s2_resp.ftq_idx

toIfu.flushFromBpu.s2 := {valid=true, bits=ftq_idx}

// S3 redirect

when(bpu_s3_redirect):

bpuPtr := bpu_s3_resp.ftq_idx + 1

if ifuPtr > bpu_s3_resp.ftq_idx:

ifuPtr := bpu_s3_resp.ftq_idx

toIfu.flushFromBpu.s3 := {valid=true, bits=ftq_idx}

### 5.7 FTQ full detection

validEntries = distanceBetween(bpuPtr, commPtr) ≥ FtqSize → new_entry_ready = false

io.fromBpu.resp.ready := new_entry_ready → BPU back-pressure

## 6. Flow / Backpressure Control

| Conditions | Action |
| --- | --- |
| FTQ full | `io.fromBpu.resp.ready = false` → BPU stall |
| backend redirect | `allowBpuIn/ToIfu = false` (2 cycles), restore bpuPtr/ifuPtr |
| BPU s2/s3 redirect | Rewind bpuPtr, rewind ifuPtr (if needed), pass flushFromBpu |
| IFU not ready | `toIfu.req.ready = f1_ready && icacheReady` → do not proceed with ifuPtr |
| entry not f_to_send | `entry_is_to_send = false` → toIfu.req.valid = false |
| ifuFlush | `allowToIfu = false` → Block IFU request |
| MMIO commit wait | IFU control based on `mmioCommitRead.valid` + `mmioLastCommit` |

## 7. Error / Exception Handling

| Situation | processing |
| --- | --- |
| Backend IPF/IGF/IAF | Write register `backendException`, `backendPcFaultPtr = ifuWbPtr` |
| throwing ICache backendException | `toICache.req.bits.backendException = backendPcFaultPtr === ifuPtr` |
| IFU predecode mismatch (false hit) | `entry_hit_status := h_false_hit`, pass false_hit flag to BPU |
| fallThroughError | `entry_hit_status := h_false_hit` (FTB ghost entry detection) |
| FTBEntryGen mispred | Create `mispred_mask` → `toBpu.update.bits.mispred_mask` |
| `XSError` invariant check | commPtr ≤ ifuWbPtr ≤ ifuPtr ≤ bpuPtr, IFU wb order, etc. |

## 8. Timing Hints

| Critical Path | Description |
| --- | --- |
| `entry_is_to_send` | `entry_fetch_status(ifuPtr.value) === f_to_send` — SRAM/register index with ifuPtr |
| `validEntries` | `distanceBetween(bpuPtr, commPtr)` — CircularQueuePtr distance calculation, directly connected to `io.fromBpu.resp.ready` |
| `toIfuPcBundle` mux | bypass / last_cycle_to_ifu_fire / otherwise 3-way mux — with SRAM rdata latency |
| `commitStateQueueReg` | FtqSize × PredictWidth 2-bit register — Reduce fanout by splitting copyNum |
| backend redirect ahead | `ftqIdxAhead / ftqIdxSelOH`: Advance the redirect cycle by 1 cycle `realAhdValid` |
| `FTBEntryGen` | combinational: br slot insertion/move logic, pftAddr calculation |

## 9. Pseudocode

// BPU enqueue

on io.fromBpu.resp.fire && allowBpuIn:

bpuPtr += 1

ftq_pc_mem.write(bpu_in_resp_ptr, fromBranchPrediction(bpu_in_resp))

[+1 cycle] entry_fetch_status(idx) := f_to_send

cfiIndex_vec(idx) := last_cycle_cfiIndex

update_target(idx) := last_cycle_bpu_target

// to IFU

each cycle:

if entry_is_to_send && ifuPtr ≠ bpuPtr:

toIfu.req.valid = true

if toIfu.req.fire && allowToIfu:

ifuPtr += 1

entry_fetch_status(ifuPtr_old) := f_sent

// IFU writeback

on ifu_wb_valid:

ftq_pd_mem.write(idx, pd)

commitStateQueue(idx)[valid && inRange] := c_toCommit

ifuWbPtr += 1

if hit && misOffset.valid:

has_false_hit = true → BPU update (false hit processing)

// Commit

when canCommit:

FTBEntryGen(pd, cfiIndex, target, hit) → new_entry

toBpu.update = {pc, ftb_entry=new_entry, br_taken_mask, mispred_mask, ...}

commPtr += 1

// Backend redirect

on fromBackend.redirect.valid:

stage2Flush = true  // → allowBpuIn/ToIfu = false

Restore bpuPtr, ifuPtr, pfPtr

toBpu.redirect.valid = true // Restore BPU history

toIfu.redirect.valid = true  // IFU flush

icacheFlush = true

[+1 cycle] backendFlush=true (lasts 2 cycles)

// BPU S2/S3 override

on bpu_s2_redirect:

bpuPtr := s2_ftq_idx + 1

if ifuPtr > s2_ftq_idx: ifuPtr := s2_ftq_idx

flushFromBpu.s2 = {valid, s2_ftq_idx}

## 10. Notes / Assumptions

copyNum=5: Register replication for ICache 5-port and BPU pointer fanout distribution

last_cycle_bpu_in 1-cycle delay: BPU enqueue critical path blocked — entry_fetch_status update occurs in the next enqueue cycle, so bypass logic is required

bpu_in_bypass_buf: PC enqueued by BPU is supplied directly to IFU without going through SRAM (1-entry bypass)

validEntries < FtqSize || canCommit: Allow enqueue if commit occurs even under FTQ full condition (prevent deadlock)

FtqRedirectAheadNum: Backend announces redirect 1 cycle earlier (ftqIdxAhead) → Redirect latency is reduced to realAhdValid

backendException Register: Maintains IPF/IAF information received from Backend until IFU writeback is completed — Exception is confirmed only when IFU writes back the entry

# frontend_NewIFU_analysis.md

Block: Frontend

Module: NewIFU

Source: IFU.scala, PreDecode.scala, FrontendBundle.scala

Protocols: Decoupled (toIbuffer, toUncache), Valid (pdWb, iTLBInter), ValidIO (icacheInter.resp)

Key Params: PredictWidth, HasCExtension, CommitWidth, fetchQueueSize=2

Last updated: 2026-02-18

→ See [frontend_block_diagram_overview.md](./frontend_block_diagram_overview.md) → See [frontend_in_out_seq_diagram.md](./frontend_in_out_seq_diagram.md#group-a)

## 1. Module Summary

Role: Retrieve instructions from ICache using the fetch address provided by FTQ, predecode them, and then transfer them to IBuffer. MMIO instructions are processed separately through InstrUncache.

Position: Frontend.scala → Module(new NewIFU) (inside FrontendInlinedImp)

Pipeline stage number: 3 register stages (F1/F2/F3) + F0 launch phase

## 2. Key Parameters

| Parameter | Source | Default | Impact |
| --- | --- | --- | --- |
| PredictWidth | `HasXSParameter.PredictWidth` | 16 | Maximum number of half-words per fetch block |
| HasCExtension | `HasXSParameter.HasCExtension` | true | Whether RVC commands are supported (16-bit processing) |
| CommitWidth | `p(XSCoreParamsKey).CommitWidth` | 6 | rob_commits port count |
| fetchQueueSize | `HasIFUConst.fetchQueueSize` | 2 | MMIO fetch internal queue size |
| blockOffBits | `HasICacheParameters` | 6 | Cache line offset bits (64B → 6bits) |
| mmioBusWidth | `HasInstrMMIOConst` | 64 | MMIO fetch bus width (bits) |
| VAddrBits | `HasXSParameter` | 39+ | Number of virtual address bits |

## 3. Interfaces

| Port | Dir | Bitwidth | Protocol | Description |
| --- | --- | --- | --- | --- |
| `io.ftqInter.fromFtq.req` | in | FetchRequestBundle | Decoupled | FTQ → IFU fetch request |
| `io.ftqInter.fromFtq.redirect` | in | BranchPredictionRedirect | Valid | backend redirect |
| `io.ftqInter.fromFtq.flushFromBpu` | in | BpuFlushInfo | — | BPU S2/S3 flush information |
| `io.ftqInter.toFtq.pdWb` | out | PredecodeWritebackBundle | Valid | predecode result → FTQ |
| `io.icacheInter.resp` | in | ICacheMainPipeResp | ValidIO | ICache → IFU response |
| `io.icacheInter.icacheReady` | in | Bool | — | ICache ready (F0 req.ready condition) |
| `io.icacheStop` | out | Bool | — | ICache stop request (`!f3_ready`) |
| `io.toIbuffer` | out | FetchToIBuffer | Decoupled | IFU → IBuffer command delivery |
| `io.toBackend` | out | IfuToBackendIO | — | IFU → Backend gpaddr record |
| `io.uncacheInter.toUncache` | out | InsUncacheReq | Decoupled | MMIO fetch request |
| `io.uncacheInter.fromUncache` | in | InsUncacheResp | Decoupled | MMIO fetch response |
| `io.iTLBInter` | in/out | TlbRequestIO | — | iTLB for MMIO reconversion (block possible) |
| `io.pmp` | in/out | ICachePMPBundle | — | PMP check (last port for IFU) |
| `io.mmioCommitRead` | out | mmioCommitRead | — | MMIO FTQ pointer/lastCommit check |
| `io.rob_commits` | in | Vec(CommitWidth, Valid) | — | ROB commit information (MMIO control) |
| `io.frontendTrigger` | in | FrontendTdataDistributeIO | — | Debug trigger settings |
| `io.csr_fsIsOff` | in | Bool | — | Whether to disable FS (RVC illegal decision) |

## 4. Internal Pipeline / State

### 4.1 Stage details

#### F0 — Launch Phase (combinational)

fromFtq.req.valid → f0_valid

fromFtq.req.bits  → f0_ftq_req (startAddr, nextlineStart, ftqIdx, ftqOffset)

f0_doubleLine = startAddr(blockOffBits-1) === 1 // Whether cross-cacheline or not

f0_vSetIdx = Vec(get_idx(startAddr), get_idx(nextlineStart))

f0_fire = fromFtq.req.fire = f0_valid && f1_ready && icacheReady

f0_flush_from_bpu = shouldFlushByStage2(f0_ftq_req.ftqIdx) || shouldFlushByStage3(...)

#### F1 — PC calculation (1 register stage)

f1_valid = RegInit(false.B)

f1_ftq_req = RegEnable(f0_ftq_req, f0_fire)

f1_doubleLine = RegEnable(f0_doubleLine, f0_fire)

f1_vSetIdx = RegEnable(f0_vSetIdx, f0_fire)

f1_fire = f1_valid && f2_ready

// PC adder optimization (PcCutPoint = VAddrBits/4 - 1)

f1_pc_high = f1_ftq_req.startAddr(VAddrBits-1, PcCutPoint)

f1_pc_lower_result(i) = Cat(0.U(1.W), startAddr(PcCutPoint-1, 0)) + (i*2).U // Includes overflow bit

f1_pc = CatPC(f1_pc_lower_result, f1_pc_high, f1_pc_high_plus1)

f1_cut_ptr(i) = startAddr(blockOffBits-1, 1) + i // ICache data cut pointer

#### F2 — ICache response processing (1 register stage)

f2_valid = RegInit(false.B)

f2_ftq_req = RegEnable(f1_ftq_req, f1_fire)

f2_doubleLine = RegEnable(f1_doubleLine, f1_fire)

f2_pc = CatPC(RegEnable(f1_pc_lower_result), ...)

f2_fire = f2_valid && f3_ready && icacheRespAllValid

// Check ICache response validity (timing critical)

f2_icache_all_resp_wire =

fromICache.valid &&

fromICache.bits.vaddr(0) === f2_ftq_req.startAddr &&

(!f2_doubleLine || fromICache.bits.vaddr(1) === f2_ftq_req.nextlineStart)

// data processing: 5 bank × 8B = 40B duplicate the response twice and extract the intermediate response

f2_data_2_cacheline = Cat(fromICache.bits.data, fromICache.bits.data)

f2_cut_data = cut(f2_data_2_cacheline, f2_cut_ptr) // PredictWidth+1 16-bit pieces

// create exception

f2_exception = ExceptionType.merge(f2_exception_in, f2_mmio_mismatch_exception)

f2_exception_vec(i) = Exception in the cacheline to which each pc(i) belongs

// Predecode (combinational, driven by f2_valid)

preDecoder.in = {f2_cut_data, f2_pc, frontendTrigger}

f2_pd = preDecoder.out.pd      // PreDecodeInfo per instruction

f2_instr = preDecoder.out.instr

f2_jump_offset = preDecoder.out.jumpOffset

//command range

f2_jump_range = Fill(PW, !ftqOffset.valid) | Fill(PW,1)>>~ftqOffset.bits

f2_ftr_range  = Fill(PW, ftqOffset.valid) | Fill(PW,1)>>~getBasicBlockIdx(nextStartAddr,startAddr)

f2_instr_range = f2_jump_range & f2_ftr_range

#### F3 — IBuffer transfer (1 register stage)

f3_valid = RegInit(false.B)

f3_ftq_req = RegEnable(f2_ftq_req, f2_fire)

f3_fire = io.toIbuffer.fire

// RVC extension

expanders(i): RVCExpander

f3_expd_instr(i) = Mux(expander.io.ill, original, expander.io.out.bits)

f3_ill(i) = expander.io.ill // illegal RVC command

// F3Predecoder: Determine brType/isCall/isRet (complement incomplete information in F2 predecode)

f3Predecoder.in.instr = f3_instr

f3_pd(i).brType = f3Predecoder.out.pd(i).brType

// PredChecker: BPU prediction vs actual decode result comparison

checkerIn = {ftqOffset, jumpOffset, target, instrRange, instrValid, pds, pc, fire_in}

checkerOutStage1 = immediate results (jal target mismatch, etc.)

checkerOutStage2 = 1-cycle delay result (more complex check)

// wb_redirect: IFU redirects itself to FTQ

wb_redirect = checkerOutStage1.valid (when mispred is detected)

// MMIO processing

if f3_pmp_mmio:

Enter MMIO mode → toUncache.valid = true

mmio_redirect = true → f2/f1 flush

// Send IBuffer

io.toIbuffer.bits = {instrs, valid, enqEnable, pd, pc, exceptionType, ...}

io.toIbuffer.valid = f3_valid && !mmio_state

// FTQ writeback

toFtq.pdWb.valid = ...

toFtq.pdWb.bits = {pc, pd, ftqIdx, misOffset, cfiOffset, target, jalTarget, instrRange}

### 4.2 flush propagation

backend_redirect = fromFtq.redirect.valid

f3_flush = backend_redirect || (wb_redirect && !f3_wb_not_flush)

f2_flush = backend_redirect || mmio_redirect || wb_redirect

f1_flush = f2_flush

f0_flush = f1_flush || f0_flush_from_bpu // Includes BPU S2/S3 override

### 4.3 Internal submodules

| Submodule | role |
| --- | --- |
| `PreDecode` | Finding instruction boundaries in 16-bit half-word streams, RVC status, and branch type primary determination |
| `F3Predecoder` | brType/isCall/isRet final decision in F3 |
| `PredChecker` | BPU prediction vs actual predecode comparison, IFU redirect generation |
| `FrontendTrigger` | HW trigger (debug) |
| `RVCExpander` × PredictWidth | 16-bit RVC → 32-bit expansion |

## 5. Functionality

### 5.1 Normal fetch flow

FTQ provides FetchRequestBundle(startAddr, ftqIdx, ftqOffset)

F0: Pass address to ICache (FTQ → ICache direct connection), IFU consumes req

F1: PC calculation (low-power adder isolation)

F2: Receive ICache response → Extract command data → predecode → Create exception

F3: RVC extension → PredChecker → IBuffer transfer + FTQ writeback

### 5.2 Cross-cacheline processing

f0_doubleLine = startAddr(blockOffBits-1) === 1 → Request the second cacheline as well

ICache responds with two cacheline data in 5 bank × 8B format

Duplicated twice with f2_data_2_cacheline = Cat(data, data) and extracted with cut() function

### 5.3 MMIO command processing

f2_pmp_mmio detection in F2 → MMIO FSM operation in F3

64-bit unit request to InstrUncache → Extract 1 command from response

After confirming ROB commit (mmioLastCommit), next MMIO fetch

Backend redirect possible during MMIO → Immediate escape

### 5.4 PredChecker operation

jalFault: It is a JAL command, but the target predicted by the BPU is different.

jalrFault: JALR but target mismatch

retFault: RET but target mismatch

targetFault: Taken branch but target mismatch

notCFIFault: Non-CFI instructions predicted to be taken

invalidTakenFault: Instructions outside the fetch range are predicted to be taken

When abnormality is detected → wb_redirect = true → IFU requests correction to FTQ

## 6. Flow / Backpressure Control

| Conditions | Action |
| --- | --- |
| IBuffer full (`!toIbuffer.ready`) | f3_fire=0, maintain f3_valid, `icacheStop=!f3_ready` → ICache stall |
| ICache not ready (`!icacheReady`) | `fromFtq.req.ready=0` → FTQ stall |
| ICache miss (`!icacheRespAllValid`) | f2_fire=0, maintain f2_valid → f1_ready=0 → FTQ stall |
| f2_icache_all_resp_reg | An ICache response was received, but F3 stall → stored in reg, preventing re-request |
| Backend redirect | f0~f3 all flush, f1/f2/f3_valid=0 |
| BPU S2/S3 flush | f0_flush_from_bpu → f0 flush (maintained after F1) |
| wb_redirect | f2/f3 flush (Keep F1), pass correction information to FTQ |
| MMIO redirect | f1/f2 flush, F3 occupied until MMIO completion |

## 7. Error / Exception Handling

| exception type | Detection point | Processing method |
| --- | --- | --- |
| Page Fault (PF) | F2: `fromICache.bits.exception` (iTLB result) | `ExceptionType.pf` → FetchToIBuffer.exceptionType |
| Guest Page Fault (GPF) | F2: iTLB response | `ExceptionType.gpf` → Same path |
| Access Fault (AF) | F2: PMP/ECC/TileLink corrupt | `ExceptionType.af` → Same path |
| MMIO mismatch | F2: pmp_mmio / itlb_pbmt mismatch in double-line | AF occurs (shown in second line) |
| Cross-page exception | F2: Next page exception for last-in-line RVI instruction | `crossPage_exception_vec` → `IBufferExceptionType.CrossPF/GPF/AF` |
| Illegal RVC | F3: `expander.io.ill` | `IBufferExceptionType.rvcII` |

ExceptionType.merge(iTLB_exception, mismatch_exception) — Priority: iTLB > mismatch

## 8. Timing Hints

| Critical Path | Description |
| --- | --- |
| F1: PC adder | `f1_pc_lower_result(i) = Cat(0,addr) + i*2` — PredictWidth parallel adder. Separate overflow processing with `CatPC` optimization |
| F2: ICache addr match | `fromICache.bits.vaddr(0) === f2_ftq_req.startAddr` — timing critical annotation (IFU.scala:365) |
| F2: cut() function | Vec(blockBytes, 16-bit) indexing × (PredictWidth+1) — big mux |
| F2: instr_range | `Fill(PW, ...) >> ~offset` — priority encoder equivalent |
| F3: PredChecker stage2 | Separate timing with 1-cycle delay. Complex comparison to checkerOutStage2 |
| F3: toIbuffer | DecodeWidth valid masking + exception type encoding parallel processing |

## 9. Pseudocode

F0 (combinational):

if !f0_flush_from_bpu && fromFtq.req.valid && icacheReady:

f0_fire = true

(ICache already receives req from FTQ directly)

F1:

if f0_fire && !f0_flush:

f1_valid := true

f1_ftq_req := f0_ftq_req

f1_pc := CatPC(f1_pc_lower_result, high, high+1)

if f1_flush:

f1_valid := false

f1_fire = f1_valid && f2_ready

F2:

if f1_fire && !f1_flush:

f2_valid := true

f2_ftq_req := f1_ftq_req

f2_pc := CatPC(f2_pc_lower_result, ...)

wait icacheRespAllValid:

f2_exception = ExceptionType.merge(itlb_exc, mmio_mismatch_exc)

f2_cut_data = cut(Cat(icache_data, icache_data), f2_cut_ptr)

preDecoder → f2_pd, f2_instr, f2_jump_offset

f2_instr_range = jump_range & ftr_range

if f2_flush:

f2_valid := false; f2_icache_all_resp_reg := false

f2_fire = f2_valid && f3_ready && icacheRespAllValid

F3:

if f2_fire && !f2_flush:

f3_valid := true

// RVC expand, F3Predecoder, PredChecker

if normal:

io.toIbuffer.valid = true

toFtq.pdWb.valid = true

if wb_redirect:

// flush f2/f3, send correction to FTQ

if mmio:

toUncache.valid = true

wait fromUncache.valid

wait mmioLastCommit

mmio_redirect → f1/f2 flush

if f3_flush:

f3_valid := false

## 10. Notes / Assumptions

f3_wb_not_flush: PredChecker detected redirect, but the result of F3 was delivered to IBuffer (non-flush condition)

The IFU sends a stop signal (icacheStop) to ICache to stop the ICache main pipe when F3 is stalled and F2 is blocked.

Determine TopDown stage trace array size with constant numOfStage = 3

FTQ-ICache direct connection: icache.io.fetch.req <> ftq.io.toICache.req (no IFU in between), IFU only receives response

fetchQueueSize=2: MMIO fetch internal buffer size (maximum 2 pending simultaneously)

# frontend_IBuffer_analysis.md

Block: Frontend

Module: IBuffer

Source: IBuffer.scala, FrontendBundle.scala

Protocols: Decoupled (in, out × DecodeWidth)

Key Params: IBufSize=48, IBufNBank=6, PredictWidth=16, DecodeWidth=6

Last updated: 2026-02-18

→ See [frontend_block_diagram_overview.md](./frontend_block_diagram_overview.md) → See [frontend_in_out_seq_diagram.md](./frontend_in_out_seq_diagram.md#group-a)

## 1. Module Summary

Role: Buffers fetch packets (up to PredictWidth commands) received from IFU and supplies up to DecodeWidth number of CtrlFlows to the Decode stage every cycle. When IBuffer is full, fetch back-pressures, and when flush, it is fully initialized.

Position: Frontend.scala → Module(new IBuffer) (inside FrontendInlinedImp)

Pipeline stage number: 1 register stage (Output Reg) + bypass path

## 2. Key Parameters

| Parameter | Source | Default | Impact |
| --- | --- | --- | --- |
| IBufSize | `p(XSCoreParamsKey).IBufSize` | 48 | Total number of buffer entries (IBufNBank × bankSize) |
| IBufNBank | `p(XSCoreParamsKey).IBufNBank` | 6 | Number of banks (requires ≥ DecodeWidth) |
| PredictWidth | `HasXSParameter.PredictWidth` | 16 | Maximum number of enqueue instructions in one cycle |
| DecodeWidth | `p(XSCoreParamsKey).DecodeWidth` | 6 | Maximum number of dequeue instructions in one cycle |
| bankSize | `IBufSize / IBufNBank` | 8 | Number of entries per bank |

Constraints: IBufSize % IBufNBank == 0, IBufNBank >= DecodeWidth

## 3. Interfaces

| Port | Dir | Bitwidth | Protocol | Description |
| --- | --- | --- | --- | --- |
| `io.in` | in | FetchToIBuffer | Decoupled | IFU → IBuffer (PredictWidth commands) |
| `io.out(i)` | out | CtrlFlow | Decoupled | IBuffer → Decode (DecodeWidth) |
| `io.flush` | in | Bool | — | Backend redirect → full flush |
| `io.decodeCanAccept` | in | Bool | — | Decode acceptability |
| `io.full` | out | Bool | — | IBuffer full status (`!allowEnq`) |
| `io.ControlRedirect` | in | Bool | — | flush Cause: Control redirect |
| `io.ControlBTBMissBubble` | in | Bool | — | flush Cause: BTB Miss bubble |
| `io.TAGEMissBubble` | in | Bool | — | flush Cause: TAGE Miss bubble |
| `io.SCMissBubble` | in | Bool | — | flush Cause: SC Miss bubble |
| `io.ITTAGEMissBubble` | in | Bool | — | flush Cause: ITTAGE Miss bubble |
| `io.RASMissBubble` | in | Bool | — | flush Cause: RAS Miss bubble |
| `io.MemVioRedirect` | in | Bool | — | flush Cause: Memory violation redirect |
| `io.stallReason` | out | StallReasonIO | — | Stall cause vector for TopDown analysis |

## 4. Internal Pipeline / State

### 4.1 Buffer structure

ibuf: Vec[IBufEntry] = RegInit(VecInit.fill(IBufSize)(0))

// IBufSize registers (not SRAM — requires precise R/W control)

bankedIBufView: Vec[Vec[IBufEntry]] =

VecInit.tabulate(IBufNBank)(bankID =>

VecInit.tabulate(bankSize)(inBankOffset =>

ibuf(bankID + inBankOffset * IBufNBank)

)

)

// bankID=0: ibuf[0, 6, 12, 18, ...]

// bankID=1: ibuf[1, 7, 13, 19, ...]

// ... interleaving batch

### 4.2 Pointer structure

| pointer | Type | role |
| --- | --- | --- |
| `enqPtrVec(i)` | Vec(PredictWidth, IBufPtr) | Each location pointer when enqueued |
| `enqPtr` | IBufPtr | = `enqPtrVec(0)` |
| `deqPtr` | IBufPtr | dequeue position (absolute) |
| `deqBankPtrVec(i)` | Vec(DecodeWidth, IBufBankPtr) | dequeue city bank pointer |
| `deqBankPtr` | IBufBankPtr | = `deqBankPtrVec(0)` |
| `deqInBankPtr(b)` | Vec(IBufNBank, IBufInBankPtr) | Internal pointer for each bank |

Invariant: deqPtr.value === deqBankPtr.value + deqInBankPtr(deqBankPtr.value).value × IBufNBank

### 4.3 Output Register (1 stage)

val outputEntries = RegInit(VecInit.fill(DecodeWidth)(0.U.asTypeOf(Valid(new IBufEntry))))

val outputEntriesValidNum = PriorityMuxDefault(...)

Output register directly connected to Decode

Complete replacement at decodeCanAccept, partial filling at outputEntriesIsNotFull

### 4.4 Bypass route

val useBypass = enqPtr === deqPtr && decodeCanAccept

// Empty state + Decode can be accepted → enqueue immediately passed to output (without going through IBuffer)

bypassEntries: Pass data from IFU directly to OutputEntries

When bypassing, numTryEnq = max(0, numFromFetch - DecodeWidth) — enqueue only the rest

## 5. Functionality

### 5.1 Enqueue (IFU → IBuffer)

numFromFetch = PopCount(io.in.bits.enqEnable) // Actual number of valid instructions

io.in.ready  = allowEnq

// Each PredictWidth command → Calculate the corresponding ibuf index

enqOffset(i) = PopCount(io.in.bits.valid.take(i))

// write at enqPtrVec(enqOffset(i)).value location

// bypass: bypass the first DecodeWidth, enqueue the rest

when(useBypass):

numBypass  = min(numFromFetch, DecodeWidth)

numTryEnq  = max(0, numFromFetch - DecodeWidth)

else:

numBypass  = 0

numTryEnq  = numFromFetch

when(io.in.fire && !io.flush):

ibuf[enqPtrVec(enqOffset(i) - DecodeWidth + k)].write(enqData(i)) // Excluding bypass

enqPtrVec += numTryEnq

### 5.2 Dequeue (IBuffer → Decode)

// 2nd stage read (area optimization)

// Stage 1: Select 1 entry from each bank (bankSize → 1 Mux)

readStage1(bankID) = Mux1H(UIntToOH(deqInBankPtr(bankID).value), bankedIBufView(bankID))

// Stage 2: Select DecodeWidth output (IBufNBank → 1 Mux)

deqEntries(i).bits = Mux1H(UIntToOH(deqBankPtrVec(i).value), readStage1)

// Determine number of outputs

when(decodeCanAccept):

numOut = min(numValid, DecodeWidth)

when(outputEntriesIsNotFull): // outputEntries last is empty

numOut = min(numValid, DecodeWidth - outputEntriesValidNum)

else:

numOut = 0

// update pointer

deqPtr        += numDeq

deqBankPtrVec += numDeq // loop through each bank pointer

deqInBankPtr(b)+= 1  (bankAdvance = numOut > validIdx)

### 5.3 Output register update

// io.out(i) = outputEntries(i) (Reg)

when(decodeCanAccept):

if useBypass && io.in.valid:

outputEntries := bypassEntries // bypass bypass direct output

else:

outputEntries := deqEntries // general dequeue

when(outputEntriesIsNotFull):

// Partial filling: Preserve existing valid entries + add new entries

outputEntries(i).bits = Mux(i < outputEntriesValidNum,

old_out, deqEntries(i - outputEntriesValidNum))

### 5.4 Full control

numValidNext = numValid + numEnq - numDeq

allowEnq     = (IBufSize - PredictWidth).U >= numValidNext

// Block in advance when almost full (Securing PredictWidth margin)

io.full = !allowEnq → io.frontendInfo.ibufFull (Backend TopDown signal)

### 5.5 Flush

on io.flush:

allowEnq      := true.B

enqPtrVec := 0, 1, 2, ... (restore initial value)

deqBankPtrVec := 0, 1, 2, ... (restore initial value)

deqInBankPtr := all 0

deqPtr        := 0

outputEntries.foreach(_.valid := false.B)

// maintain ibuf register contents (no valid bit, range managed by pointer)

## 6. Flow / Backpressure Control

| Conditions | Action |
| --- | --- |
| IBuffer full (`!allowEnq`) | `io.in.ready = false` → IFU toIbuffer stall → F3 stall → ICache stall |
| Decode stall (`!decodeCanAccept`) | `numOut = 0` → keep outputEntries → no move deqPtr |
| `outputEntriesIsNotFull` | Decode accepts only part → keeps the rest in output reg |
| Use Bypass | IBuffer empty + decodeCanAccept → enqueue-dequeue simultaneous processing (1-cycle saving) |
| Flush | Immediate full reset, restore `allowEnq := true` |

back-pressure propagation path:

Backend not accept → decodeCanAccept=false

→ numOut=0 → outputEntries fixed

→ increase numValidNext → allowEnq=false

→ io.in.ready=false → IFU stall → F3 stall

→ icacheStop=true → ICache stall

→ FTQ toIfu.req.ready=false → Block additional IFU requests

## 7. Error / Exception Handling

IBuffer itself does not generate exceptions, but preserves the exception information received from IFU and passes it to Decode.

| exception information | Save type | Delivery method |
| --- | --- | --- |
| Page Fault (PF) | `IBufferExceptionType.NonCrossPF / CrossPF` | `cf.exceptionVec(instrPageFault) := isPF(...)` |
| Guest Page Fault (GPF) | `NonCrossGPF / CrossGPF` | `cf.exceptionVec(instrGuestPageFault)` |
| Access Fault (AF) | `NonCrossAF / CrossAF` | `cf.exceptionVec(instrAccessFault)` |
| Illegal RVC | `rvcII` | `cf.exceptionVec(EX_II)` |
| Cross-page IPF fix | `CrossPF` | `cf.crossPageIPFFix := isCrossPage(...)` |
| Backend exception | `backendException: Bool` | `cf.backendException` delivered as is |
| Trigger | `TriggerAction()` | `cf.trigger` delivered as is |

IBufferExceptionType (3-bit):

000 None    001 NonCrossPF   010 NonCrossGPF   011 NonCrossAF

100 rvcII   101 CrossPF      110 CrossGPF      111 CrossAF

bit[2]: isCrossPage, bit[1:0]: exception type

### TopDown classification during flush

when(io.flush):

topdown_stage.reasons := set based on causes

BTBMissBubble  ← ControlBTBMissBubble

TAGEMissBubble ← TAGEMissBubble

SCMissBubble   ← SCMissBubble

ITTAGEMissBubble ← ITTAGEMissBubble

RASMissBubble  ← RASMissBubble

MemVioRedirectBubble ← MemVioRedirect

OtherRedirectBubble  ← otherwise

io.stallReason.reason(i) := matchBubble  // for wasted decode slots

## 8. Timing Hints

| Critical Path | Description |
| --- | --- |
| Enqueue write mux | For each ibuf entry, select 1 of PredictWidth (`Mux1H(validOH, enqData)`) — IBufSize × PredictWidth Mux |
| Read Dequeue Step 2 | Stage1: bankSize→1 Mux × IBufNBank, Stage2: IBufNBank→1 Mux × DecodeWidth |
| `enqOffset` PopCount | `PopCount(io.in.bits.valid.take(i))` × PredictWidth — parallel prefix sum |
| `outputEntriesValidNum` | `PriorityMuxDefault(outputEntries.map(_.valid)...)` — DecodeWidth priority encoder |
| `numValidNext` → `allowEnq` | Comparison after addition — `io.in.ready` critical path |
| deqBankPtr update | `deqBankPtrVec(i) + numDeq` × DecodeWidth — parallel circular ptr addition |

## 9. Pseudocode

// Enqueue

S0 (combinational):

numFromFetch = PopCount(in.bits.enqEnable)

useBypass    = (enqPtr === deqPtr) && decodeCanAccept

if useBypass:

numBypass  = min(numFromFetch, DecodeWidth)

numTryEnq  = max(0, numFromFetch - DecodeWidth)

else:

numBypass  = 0

numTryEnq  = numFromFetch

enqOffset(i) = PopCount(in.bits.valid.take(i))

enqData(i)   = IBufEntry.fromFetch(in.bits, i)

allowEnq = (IBufSize - PredictWidth) >= numValidNext

on in.fire && !flush:

for each i in PredictWidth:

if in.bits.valid(i) && in.bits.enqEnable(i) && !bypassed:

ibuf[enqPtr + enqOffset(i) - numBypass].write(enqData(i))

enqPtrVec += numTryEnq

// Dequeue

each cycle:

readStage1(b) = Mux1H(deqInBankPtr(b).value, bankedIBufView(b))

deqEntries(i) = Mux1H(deqBankPtrVec(i).value, readStage1)

numOut = ... // decodeCanAccept / outputEntriesIsNotFull condition

if decodeCanAccept:

if useBypass && in.valid:

outputEntries := bypassEntries

else:

outputEntries := deqEntries

elif outputEntriesIsNotFull:

outputEntries(i > outputEntriesValidNum) := deqEntries(i - outputEntriesValidNum)

deqPtr        += numDeq

deqBankPtrVec += numDeq

for b in IBufNBank: if bankAdvance(b): deqInBankPtr(b) += 1

// Output to Decode

io.out(i).valid = outputEntries(i).valid

io.out(i).bits  = outputEntries(i).bits.toCtrlFlow

// Flush

on flush:

enqPtrVec, deqBankPtrVec, deqInBankPtr, deqPtr := reset

outputEntries.foreach(_.valid := false)

allowEnq := true

## 10. Notes / Assumptions

ibuf is RegInit register array (not SRAM) — registers are used for precise write control and bypass

Bank interleaving (bankID + inBankOffset × IBufNBank): When dequeuing, only read a maximum of 1 entry from each bank to save area (IBufNBank:1 Mux + bankSize:1 Mux two steps)

allowEnq Hysteresis: (IBufSize - PredictWidth).U >= numValidNext — Worst case (PredictWidth simultaneous enqueue) overflow prevention

Reason for having 1 output register: Decode timing optimization — Direct reading from outputEntries register without combinational

IBufNBank >= DecodeWidth Prerequisite: Each dequeue slot (i) must read a different bank to avoid conflict.

TopDown stallReason: Record the cause in topdown_stage for 1 cycle immediately after flush → Tagging the stall cause in an empty slot in Decode

headBubble / instrHungry: Distinguish between the state in which the IBuffer is empty after the previous flush (normal empty state vs. delayed fetch)